CRYPTOCard© User Authentication

Token

Operation & Systems Guide

(Revision 4.1)

14 September, 1994

CRYPTOCard Corporation

160 Industrial Avenue

Carleton Place, Ontario, CANADA

K7C 3T2

Tel: (613) 253-8476    Fax: (613) 253-8493

TABLE OF CONTENTS

1.	OVERVIEW	  1

1.1	APPLICABILITY	  1

2.	SPECIFICATIONS	  2

3.	USER INTERFACE	  4

3.1.	DISPLAY	  4

3.2	KEYPAD	  4

3.2.1	ALPHANUMERIC KEYS	  4

3.2.2	FUNCTION KEYS	  5

4.	INITIALIZATION MODE	  6

4.1	MANUAL INITIALIZATION	  6

4.1.1	LOCKED	  6

4.1.2    OPTIONS	  6

4.1.2.1    OPTION FIELD 1	  7

4.1.2.2    OPTION FIELD 2	  8

4.1.2.3    OPTION FIELD 3	 10

4.1.3   OPTION EXAMPLES	 11

4.1.4	OPTION ERRORS	 11

4.1.5	KEY 1	 12

4.1.6	KEYn	 12

4.1.7	USER ID	 13

4.1.8	INITIAL PIN	 13

4.2	AUTOMATIC INITIALIZATION	 13

5.	END USER MODE	 14

5.1	PIN ENTRY	 14

5.2	FORCED PIN CHANGE	 14

5.3	USER ID	 14

5.4	VOLUNTARY PIN CHANGE	 15

5.5	CHALLENGE/MAC	 15

5.5.1	CHALLENGE	 15

5.5.2	MAC	 15

5.6	HANDLING PRECAUTIONS	 16

5.6.1	USER PRECAUTIONS	 16

5.6.2	SHIPPING PRECAUTIONS	 16

5.7	BATTERY REPLACEMENT	 17

6.	QUALITY ASSURANCE MODE	 18

6.1	QUALITY ASSURANCE MODE, OPTION 1	 18

6.2	QUALITY ASSURANCE MODE, OPTION 2	 18

6.3	QUALITY ASSURANCE MODE, OPTION 3	 18

6.4	QUALITY ASSURANCE MODE, OPTION 4	 18

TABLE OF CONTENTS - continued

7.	MESSAGES AND LANGUAGE OPTIONS	 20

8.	OCTAL CHARACTER SET	 21

9.	REDUCED INPUT MODE - GENERAL INFORMATION	 22

9.1	PURPOSE	 22

9.2	SECURITY	 22

9.3	INITIALIZATION	 22

9.4	OPERATION	 22

9.5	HOST	 23

9.6	MULTIPLE KEYS	 23

9.7	ALGORITHM	 23

10.	CRYPTOCard© TOKENS AND THE DES ALGORITHM	 24

10.1	INPUT	 24

10.2	OUTPUT	 24

10.3	MAC OPERATION	 24

10.4	DES EXAMPLE	 25

CRYPTOCard© - Operations/Systems Guide

# 1.0	OVERVIEW

CRYPTOCard© Corporation has developed a DES Token (hereafter referred to simply as the CRYPTOCard©) which can be employed by institutions seeking to enhance the security of their facilities.:

The CRYPTOCard© contains a powerful microcomputer, keypad, liquid crystal display and batteries.

Functionally, the CRYPTOCard© is intended to be utilized by remote users requiring access to host resources via PCs and communication links.  The CRYPTOCard© will provide a cryptographic response to a host-generated challenge.  Additionally, it will generate a Message Authentication Code for any variable length message.

## 1.1	APPLICABILITY

This document reflects the operation of a particular hardware/firmware configuration.  Specifically, it applies to Version 37 of the CRYPTOCard© firmware.

This document describes the two primary modes (Initialization Mode and End User Mode) plus the Quality Assurance Mode of operation of the CRYPTOCard©.

# 2.0	SPECIFICATIONS

Dimensions:				3.365 in. x 2.125 in. x .150 in

85.4 mm. x 53.9 mm. x 3.80 mm.

Weight:				1 oz.

30 gm.

Display:				8, 5 x 7 dot matrix characters

Keypad:				16 keys

Operating Temperature Range:	32 degrees F to 110 degrees F

0 degrees C to 43 degrees C

Storage Temperature Range:		-40 degrees F to 140 degrees F

-40 degrees C to 60 degrees C

Batteries:				2 x CR2016, Lithium/Manganese Dioxide coin cells

Cryptographic Keys:			up to 3, 64-bit DES keys

DES Calculation Speed:		200 milliseconds

Options:

There are a number of operating options that may be selected by the issuer at the time the cryptographic keys are loaded.  These are:

PIN Entry Feedback:		a)	Select PIN Entry Feedback with User allowed

to change PIN at will

b)	Select PIN Entry Feedback with User not allowed to

change PIN

c)	Omit PIN Entry Feedback

Output Format:		a)	hexadecimal, 8-digit

b)	decimal, 8-digit

c)	hexadecimal, telephone style

d)	decimal, telephone style

Reduced Input Mode:		a)	Select reduced input mode where challenge string is

derived from the stored response to the previous

challenge

b)	Force full challenge entry each time

User ID			a)	Display 8 character User ID after PIN entry

b)	Display 'Ready' message after PIN Entry

Number of Tries			Set number of unsuccessful tries to enter PIN from 1

to 7 or unlimited number.  If maximum number of

unsuccessful tries is exceeded, cryptographic keys

and other data is erased.

Minimum PIN Length			Set minimum PIN length from 3 to 8 digits.

Maximum is always 8.

Time Out Period			a)	Card shuts itself off after 30 seconds of

inactivity.

b)	Shut off after 60 seconds

Language of Prompts			a)	English

b)	French

c)	German

d)	Italian

e)	Portuguese

f)	Swedish

g)	Spanish

Number of Keys			Card may be initialized with 1, 2 or 3 keys.  Also the

card may be initialized with a split single key.

# 3.	USER INTERFACE

The user interacts with the CRYPTOCard© via a 16 key keypad and an 8 digit liquid crystal display.

## 3.1	DISPLAY

The display has the ability to show eight alphanumeric characters at a time.  Alphanumeric characters consist of the alphabetic characters 'A' through 'Z', the numeric characters '0' through '9', and some special characters.  Quotation marks can also be displayed; additionally, they are used throughout this document to highlight the messages that are displayed.

The display will clear automatically after 30 or 60 seconds of keyboard inactivity.  In this document, the words screen and display are interchangeable.

## 3.2	KEYPAD

The keypad consists of two classes of keys:  The function keys which are not displayed on the screen, and the alphanumeric keys which result in characters being displayed on the screen.  If a key is not relevant to the operation at hand, it is ignored by the CRYPTOCard©.

### 3.2.1	ALPHANUMERIC KEYS

The ten alphanumeric keys are used principally for the entry of decimal digits and secondarily, for the entry of the octal digits '0' through '7' or hexadecimal digits '0' through 'F'.  The alphanumeric keypad itself is arranged like a telephone keypad where each numeral may also represent a series of letters.  This feature becomes significant when the token is being used in hexadecimal mode as the hexadecimal digits 'A' through 'F' are selected with the multipurpose keys '2' and '3'.

In general, information is entered one byte at a time.  The default format is octal but hexadecimal may also be used.  If a keypad operation requires the entry of information represented in octal, the user should push the three keys in succession that correspond to the octal character, followed by the scroll(->) function key.  Please refer to Section 8.0 for further details on the octal character set.  The first digit of an octal triplet must be in the range '0' through '3'; the remaining two digits can be in the range '0' through '7'.  The three octal digits are scrolled onto the display.  If more than three octal digits are entered, only the last three are displayed and they remain there until the scroll function key is pushed.  Errors are corrected by rekeying the three octal digits, not with the aid of the CLR function key.

The octal digits are displayed on the left hand side of the screen.  The right hand side displays a number ranging from 1 to 8 which indicates the sequence number of the octal triplet currently being entered.

The entry of non-alphanumeric non-displayable characters (i.e. outside the octal range of 040 through 177) is only allowed when entering cryptographic keys.

### 3.2.2	FUNCTION KEYS

There are six function keys:

1.	ON/OFF

If the CRYPTOCard© is off, pressing the ON/OFF function key turns on the CRYPTOCard©.  Conversely, if the CRYPTOCard© is on, pressing this key turns off the CRYPTOCard©.  The CRYPTOCard© turns itself off automatically after 30 or 60 seconds of user inactivity.

2.	CLR

The primary purpose of this function key is to allow the user to make corrections before the ENTer key is pressed.  Pressing this key completely CLeaRs the current entry.  The user must then reenter the correct data.

When entry of longer strings is allowed, the CLR function key clears the current display plus all previously entered portions of that string.  The user must then reenter the entire data starting at the very first character.

The secondary purpose of the CLR key is to allow an official to clear or reject the last initialization step.

3.	->

The scroll (->) key is used to signal completion of an octal character comprised of three octal digits.  It is generally only used for manual initialization.

4.	ENT

The user indicates completion of a related string of characters by pressing the ENTer key.

5.	CH/MAC

After the end user has successfully logged onto the CRYPTOCard©, he can elect to enter either a CHallenge string, to be encrypted using the Data Encryption Standard (DES) or, a message, for which the Message Authentication Code (MAC) is desired.  Depression of this key indicates both of these choices.

6.	CPIN (Alternatively may appear as "New PSC")

The end user can elect to Change his or her Personal Identification Number (PIN) by pressing this key.  The function of this key can be altered by certain option parameters which may be specified by the issuer.  See section 4.2.1, Option Character 1.

# 4.0	INITIALIZATION MODE

Upon delivery of the CRYPTOCard© from the manufacturer, the card must undergo initialization before being issued to the end user.  The initialization phase is used to specify such things as:

-	whether or not an end user's identification code is to be supported.

-	whether or not feedback is required on wrong PIN entry.

-	the number of DES cryptographic keys to be supported.  The CRYPTOCard© can

handle three keys for use with different host systems.

-	choice of language for message display

(Note: Throughout this document, English 1 (see Section 7.0) has been assumed

as the language choice.)

-	display format for DES encryption results. Four formats are available.

-	minimum PIN length.  Minimum PIN length can range from 3 to 8 characters.

-	the maximum number of consecutive invalid attempts at entering the PIN, if

applicable.

-	length of time-out

-	the DES cryptographic key(s)

-	the end user's identification code, if applicable

-	the initial PIN

The CRYPTOCard© will not function at the end user level until this phase is successfully completed.  Initialization can be accomplished either by manual entry of the information required in response to prompts from the CRYPTOCard©, or automatically using our Initializer Fixture and related software.

## 4.1	MANUAL INITIALIZATION

### 4.1.1	LOCKED

When the CRYPTOCard© is first turned on, it displays the message "Locked".  The official responsible for this phase of initialization should push the ENTer key in order to proceed with the next step.  Any other key entry is ignored.

### 4.1.2	OPTIONS

The CRYPTOCard© goes on to display "Options?".  There are three option fields to enter, each consisting of 3 octal digits.  The official should enter the 3 sequences of three octal digits plus the scroll key representing the configuration desired for this CRYPTOCard©, followed by the ENTer key.

#### 4.1.2.1	OPTION FIELD 1

Digit 1 (leftmost digit)	- PIN Entry Feedback Options

Digit 2 (middle digit)	- Decimal Display Option

Digit 3 (rightmost digit)	- Telephone Style Display Option

PIN ENTRY FEEDBACK

PIN entry feedback provides a positive indication to the user that his PIN has been entered correctly.  This option digit may have four values corresponding to the options listed below:

Value		Option

0		No PIN Entry Feedback, PIN is fixed at time of issue

1		PIN Entry Feedback, PIN may be changed

2		Not Valid.  There is no provision for a user changeable PIN without

feedback as this may lead to confusion and operational difficulties.

3		PIN Entry Feedback, PIN is fixed at time of issue

If the end user is to be informed of improper attempts at entering the PIN, PIN entry feedback should be selected.  Each time the end user improperly enters his PIN, he will be re-prompted to enter the correct PIN.  This sequence will continue until the correct PIN is entered or the end user exceeds the maximum allowable attempts at PIN entry (another configuration option).

If PIN entry feedback is not selected the end user will not be informed of improper PIN entry by a re-prompt.  Instead, the CRYPTOCard© will use a key, formed by the "exclusive OR" of the correct key and the invalid PIN, for DES encryption.  The end user will always be prevented from changing the PIN and hence the CPIN key will be inoperative.

DECIMAL DISPLAY

This option has no effect on the DES encryption algorithm, just on the method used to display the results.  If the results of encryption are to be displayed in hexadecimal, set this option to zero.  If they are to be displayed in decimal, set this option to one.  With decimal display format, hexadecimal characters in the range A through F are converted to 0 through 5 respectively.

TELEPHONE DISPLAY

Similarly, this option also has no effect on the DES encryption algorithm, just on the method used to display the results.  If the first eight encryption characters are to be displayed as is, set this option to zero.  If they are to be displayed in a telephone number format with the 4th character replaced by a hyphen i.e. XXX-XXXX. set this option to one.

#### 4.1.2.2	OPTION FIELD 2

Digit 1 (leftmost digit)	- User ID and Reduced Input Options

Digit 2 (middle digit)	- Number of Unsuccessful Tries Option

Digit 3 (rightmost digit)	- Minimum PIN Length

USER ID AND REDUCED INPUT OPTIONS

The following are the permissible values of this digit:

Value		Option

0		Full challenge entry, no User ID required

1		Full challenge entry, User ID required

2		Reduced input entry, no User ID required

3		Reduced input entry, USER ID required

If User ID's are required,  a subsequent step in the Initialization Mode prompts for the entry of the User ID.  During the End User Mode, the User ID is displayed upon successful PIN entry and completion of DES encryption.  If User ID is not used, the message 'Ready' or its foreign language equivalent is displayed under the same circumstances.

When the reduced input option is selected, the user does not generally have to enter the new challenge string.  The card will predict the next challenge string based on the previous challenge.  When the user presses ENTer after the User ID or Ready message appears, this challenge string will appear on the display.  The host will use the same predictive algorithm and display the challenge.  The user simply compares the CRYPTOCard©'s predicted challenge with the one displayed by the host.  If they are the same, the user presses ENTer to accept it.  The card then displays the response string.  If for some reason the challenge strings do not compare, the user must press the CHallenge/MAC key and enter the challenge string in the usual way.  This could happen if the CRYPTOCard© and host became 'out-of-sync'.  Section 9 has a more detailed explanation of the reduced input mode.

TRIES

This option specifies the number of consecutive attempts at unsuccessfully entering a PIN that are allowed before the card reverts to the 'Locked' condition.  It is specified as follows:

0  Unlimited

1  One

2  Two

3  Three

4  Four

5  Five

6  Six

7  Seven

Note: If Pin Entry Feedback is disabled in option character 1, it is suggested that this field should be set to 0.  Otherwise, the card will go to the locked state upon violation of the maximum number of erroneous PIN entry attempts.

MINIMUM PIN LENGTH

Specify the minimum acceptable PIN length as follows:

3  Three

4  Four

5  Five

6  Six

7  Seven

0  Eight

#### OPTION FIELD 3

Digit 1 (leftmost digit)	- Time Out Period

Digit 2 (middle digit)	- Language Option

Digit 3 (rightmost digit)	- Number of Keys

TIME-OUT

This option controls the length of time the CRYPTOCard© will stay on with no keypad activity.  Set this option to one for sixty seconds; set it to 0 for thirty seconds.  The thirty second option will contribute to longer battery life.  Users should be encouraged to explicity turn off the device using the ON/OFF key rather than allowing it to time out on its own.  This will further enhance battery life.

LANGUAGE CHOICE

Specify the code of the language chosen for message displays as follows:

0  English 1

1  English 2

2  French

3  German

4  Italian

5  Portuguese

6  Swedish

7  Spanish

The messages described in this document assume English 1 was chosen.  Please refer to section 7.0 for the foreign language equivalents.  This option has no effect on the User ID display which assumes use of the English language character set.

NUMBER OF KEYS

Set this field equal to the number of keys to be supported as follows:

0	split single key

1	one key

2	two keys

3	three keys

NOTE: Split single key entry

Split single key entry provides extra security at the point of issue by accepting two keys entered by different people.  These keys are then exclusive ORed within the CRYPTOCard© to obtain the final key.

### 4.1.3	OPTION EXAMPLES

Example 1:

Suppose a customer wants his CRYPTOCard©'s configured as follows:

-PIN entry feedback, user may change PIN

-decimal display but not telephone display

-User ID's

-full challenge entry

-three tries at PIN entry allowed

-minimum PIN length of 4 characters

-30 second time-out

-English 1 language messages

-one DES key

The three option characters are:		110 134 001

and the 13 key entry is:			1 1 0 -> 1 3 4-> 0 0 1 -> ENT

Example 2:

The requirements are as follows:

-PIN entry feedback, fixed PIN

-hexadecimal display in telephone format

-User ID's

-reduced input mode

-four tries at PIN entry allowed

-minimum PIN length of 6 characters

-60 second time-out

-German language messages

-split entry DES key

The three option characters are:		301 346 130

and the 13 key entry is:			3 0 1 -> 3 4 6-> 1 3 0 -> ENT

### 4.1.4	OPTION ERRORS

If an invalid configuration option is selected, the CRYPTOCard© re-displays the "Options?" message prompt.  Otherwise, the CRYPTOCard© proceeds to the next step.

### 4.1.5	KEY 1

The CRYPTOCard© goes on to display "Key1?".  The official should enter the twenty-four (24) octal digits (i.e., 8 sequences of 3-digits) representing the end user's eight byte DES cryptographic key, followed by the ENTer key.

Once the key had been entered, the CRYPTOCard© performs the DES encryption algorithm on plain text of binary zeroes using the entered key.  The first half of the resulting cipher text is displayed on the screen in the format specified.  If the result is correct, the official should press the ENTer key; otherwise the CLR key, which in turn forces the re-display of the message "Key1?" and this step is repeated.

As an example, to enter the key:

212 374 254 336 000 103 127 052

the security officer would enter:

212 -> where -> is the scroll key)

374 ->

254 ->

336 ->

000 ->

103 ->

127 ->

052 ->

ENT (i.e. the ENTer key)

### 4.1.6	KEYn

If more than one key has been selected in option character 3, the CRYPTOCard© displays "Keyn?" where n ranges from 2 to 3.  The official should enter the twenty-four octal digits representing the nth DES cryptographic key, followed by the ENTer key.  The CRYPTOCard© performs the DES encryption algorithm on plain text of binary zeroes using the second key.  The first half of the resulting cipher text is displayed on the screen.  If the result is correct, the official presses the ENTer key; otherwise, the CLR key, which in turn, forces the re-display of the message "KEYn?" and this step is repeated.  This step is repeated with n incremented until n equals the value specified in Option Character 3.

If split single key entry has been selected in option character 3, the CRYPTOCard© may be turned off at this point to await further processing.  In a highly secure environment, the card would then be passed to a second security officer who has no knowledge of the first part of the key as entered in paragraph 4.3.  When turned on, the card displays "Key2?" on the screen.  The security officer then enters the second part of the key which is known only to him.  This is done the same way as the first key.  When ENTer is pressed, the card encrypts binary zeroes using this key and displays the result.  If the result is correct, the official should press the ENTer key; otherwise the CLR key, which in turn forces the re-display of the message "Key2?"and this step is repeated.  After verifying the correct entry of the second key, the ENTer button should be pressed again. The card Exclusive OR's the two keys together to produce one key which is not known to any single individual.  The result of encrypting binary zeroes with this final key is then displayed.

### 4.1.7	USER ID

If User ID's are part of the configuration, the CRYPTOCard© displays the message "User ID?"; otherwise this step is skipped.  The official should key in the 7 or 8 alphanumeric character end user identification using the equivalent octal representation, followed by the ENTer key.  If an octal triplet represents a non-displayable character, the CRYPTOCard© displays the message "Invalid" and awaits the reentry of a new octal triplet.  The user identification is displayed in its alphanumeric form once the ENTer key is pressed.  If the user ID is correct, the official presses the ENTer key; otherwise the CLR key, which forces the re-display of the message "User ID?" and this step must then be repeated.  A complete chart of valid octal equivalents of ASCII alphanumeric characters and symbols is shown in Section 8.

### 4.1.8	INITIAL PIN

The final step of initialization occurs when the CRYPTOCard© seeks the entry of the end user's initial PIN by displaying "New PIN?".  A three to eight decimal digit entry must be made depending on the Minimum Pin Length.  For security reasons, asterisks ("*") are displayed in place of the actual decimal digit keyed.  The CRYPTOCard© then displays "Verify"  and the official must repeat the entry of the end user's initial PIN.  If the second PIN entry does not match the first, the CRYPTOCard© re-displays the message "New PIN?" and repeats this step.  If the two PIN's do match, the CRYPTOCard© signals completion of the initialization mode by displaying the message "Card OK".  The card automatically goes into End User Mode the next time the card is turned on, or if the ENTer key is pressed.

If the CRYPTOCard© is inadvertently powered down during the initialization mode it will, after being turned on again, re-display the most recent prompt or message.

## 4.2	AUTOMATIC INITIALIZATION

Refer to the most recent issue of the CRYPTOCard© Token Initializer User's Guide for complete information on this topic.

# 5.	END USER MODE

## 5.1	PIN ENTRY

When the end user presses the function key labelled ON/OFF, the CRYPTOCard© responds by displaying "PIN?".  In order to gain further access to the card's functions, the end user must enter the correct PIN.  If the PIN that is entered does not match the one stored in the CRYPTOCard©, two possibilities exist:

-if PIN Entry Feedback is supported, the message "PIN?" is re-displayed and this step is repeated.  The number of incorrect PIN's that can be entered in succession is controlled by an initialization parameter.  The CRYPTOCard© keeps track of the number of unsuccessful attempts, even when it "powers down".  If the number of attempts exceeds the maximum before a correct PIN entry is made, the CRYPTOCard© reverts to its pre-initialization state, displays the message "Locked" and awaits official initialization.

-if Pin Entry Feedback is not supported, the incorrect PIN is accepted but an invalid key is used for DES encryption during the rest of this session.

If the PIN entry is correct, the CRYPTOCard© displays the end user's identification or "Ready" depending on whether User ID's are supported or not.

## 5.2	FORCED PIN CHANGE

This step occurs if PIN Entry Feedback and User Changeable PIN have been selected.   Following successful entry of the initial PIN, the CRYPTOCard© forces the end user to replace the initial PIN with a PIN of his or her own choosing.  It does so by displaying "New PIN?".  A multiple digit entry ranging from the minimum specified in the configuration option to a maximum of eight decimal digits must be made.  For security reasons, asterisks ("*") are displayed in place of the actual decimal digit keyed.  If the new PIN is the same as the old PIN, the CRYPTOCard© rejects it and re-displays "New PIN?".  If the new PIN is acceptable, the CRYPTOCard© then displays "Verify" and the end user must repeat the entry of the new PIN.  If the second PIN entry does not match the first, the CRYPTOCard© re-displays the message "New PIN?" and repeats this step.  If the CRYPTOCard© powers down the display before a new PIN is successfully entered, the old PIN remains in effect.  If the two PIN's do match, the CRYPTOCard© signals completion of this step by displaying the end user's identification or "Ready" message.

## 5.3	USER ID

The end user has only two valid responses to the display of the end user's identification or "Ready" message: depression of either the CH/MAC key or ENTer key to begin the challenge process or the CPIN key to change the PIN (if allowed).

## 5.4	VOLUNTARY PIN CHANGE

If the end user wants to change his or her PIN yet again and User Changeable PIN is allowed, then the CPIN function key should be pushed.  The CRYPTOCard© responds by displaying "New PIN?".  A multiple key entry ranging from the minimum specified in the configuration option to a maximum of eight decimal digits must be made.  For security reasons, asterisks ("*") are displayed in place of the actual decimal digit keyed.  If the new PIN is the same as the old PIN, the CRYPTOCard© rejects it and re-displays "New PIN?".  If the new PIN is acceptable, the CRYPTOCard© then displays "Verify" and the end user must repeat the entry of the new PIN.  If the second PIN entry does not match the first, the CRYPTOCard© re-displays the message "New PIN?" and repeats this step.  If the two new PIN"s do match, the CRYPTOCard© signals completion of this step by once again displaying the end user's identification or "Ready" message.

## 5.5	CHALLENGE/MAC

If the end user elects to enter either a challenge string, to be encrypted using the Data Encryption Standard (DES), or a message, for which the Message Authentication Code (MAC) is required, he must press either the CH/MAC function key or ENTer key.

### 5.5.1	CHALLENGE

In the case of a challenge, the end user enters five to eight decimal digits representing the plain text challenge string to be encrypted, followed by the ENTer key.  A five to seven digit entry is internally padded in the least significant positions with binary zeroes.  The entered digits are interpreted as ASCII.  e.g. the digit '4' is interpreted as hexadecimal 34.  The plain text is encrypted according to the DES algorithm and the most significant 32 bits of the 64-bit cipher text are displayed according to the format specified in the configuration option.  The depression of any key will cause a re-display of the end user's identification or the "Ready" message.  If the end user wishes to encrypt another challenge string, he simply pushes the CH/MAC function key or ENTer key again and this step is repeated.  Section 10. gives more details on the DES encryption.

### 5.5.2	MAC

A MAC operation on a message of five to eight characters in the same as a challenge operation.  A MAC operation on more than eight numeric characters (maximum of 64) is more elaborate.  It is characterized by the absence of the ENTer key until the last numeric character in the message is entered, and incorporation of DES Cipher Block Chaining (CBC) mode.  Once the ENTer key is pressed, the input string is padded on the right with sufficient binary zeroes to round out the total bit count to an even multiple of 64.  The 32 most significant bits of the calculated MAC are displayed.  The initialization vector is reset to binary zeroes at the beginning to each MAC sequence.  The depression of any key will cause a re-display of the end user's identification or "Ready" message.  If the end user wishes to calculate another MAC, he simply pushes the CH/MAC function key or ENTer key again and this step is repeated.

## 5.6	HANDLING PRECAUTIONS

### 5.6.1	USER PRECAUTIONS

CRYPTOCard©s are equipped with liquid crystal displays which are made of glass.  With improper handling this glass will break.  A broken display is characterized by large black areas on the screen that look like ink blots.  Please include the following precautionary note when preparing user instructional materials.

CAUTION

The CRYPTOCard© is equipped with a glass display device.  To prevent breakage, observe the following:

1)	Do not drop the card onto hard surfaces.

2)	Do not attempt to bend or twist the card.

3)	Do not carry the card in a hip pocket wallet.

4)	Do not place heavy objects on top of the card.

1

### 5.6.2	SHIPPING PRECAUTIONS

If it is necessary to ship CRYPTOCard©'s to a different location, please ensure that they are adequately padded and packaged within a hard walled container such as a cardboard box.  Do not ship them in unprotected envelopes.

## 5.7	BATTERY REPLACEMENT

The CRYPTOCard© RB model is equipped with user replaceable batteries.  Under normal usage, these should last for up to 3 years before replacement is required.

The batteries are held in place within the unit by plastic clips, one on the top edge of the card and one on the bottom edge.  Each clip has a groove in it to facilitate removal. To remove a battery, use a fingernail or some flat narrow object to engage the groove and pull straight out.  Replace the batteries one at a time, putting them first into the battery clips and then sliding them back into place in the card.  Be sure to observe correct polarity.  The battery clips should slide in smoothly and click into position.  If any resistance is encountered, check that the plus signs on the battery and the clip are both facing the back of the unit.

Users should be advised of the following precautions regarding battery replacement:

CAUTION

- Replace batteries with type CR2016 lithium/manganese dioxide coin cells only.

- Always replace both batteries with new cells from the same manufacturer.

- Remove and replace only one cell at a time to avoid loosing the encryption key(s) and other information stored within the card.  If this data is lost, the card must be returned to the issuer for re-initialization.

- Observe proper polarity when changing cells.  The '+' sign on the cell itself and on the battery clip must both face the back of the unit when inserting new batteries.

- Do not incinerate old cells or attempt to disassemble, cut, puncture or crush them.

- 2

# 6.	QUALITY ASSURANCE MODE

In addition to the two primary modes of operation, there is a Quality Assurance (secondary) mode.  This mode is available for one half second immediately after turning on the CRYPTOCard©.  The user should enter the sequence 2 2 5 3 7 n where n ranges from 1 to 4.  The user is allowed a maximum of two seconds between keypad entries, excluding the first which must be made within the half second window.  Failure to comply with this timing restriction causes the CRYPTOCard© to revert to the appropriate primary mode so as not to interfere with normal operation.  As the name implies, this mode is not really intended for end users.  Rather, it is intended to be of use to the CRYPTOCard© Corporation and its immediate customers.

## 6.1	QUALITY ASSURANCE MODE, OPTION 1

Quality Assurance 1 forces a complete reset of the CRYPTOCard©'s microprocessor, returning the card to the same state as it was received from the manufacturer.  All configuration parameters including encryption keys are erased.

## 6.2	QUALITY ASSURANCE MODE, OPTION 2

Quality Assurance 2 allows the entry of Keys and User ID's during the Initialization Mode to be done in hexadecimal rather than octal.  All hexadecimal nybbles are entered using the multipurpose alphanumeric keypad.  For example, hexadecimal nybbles '2', 'A', 'B' and 'C' are selected with the multipurpose '2' key; '3', 'D', 'E' and 'F' are entered using the multi-purpose '3' key.

## 6.3	QUALITY ASSURANCE MODE, OPTION 3

Quality Assurance 3 returns the entry mode to octal from hexadecimal.

## 6.4	QUALITY ASSURANCE MODE, OPTION 4

Quality Assurance 4 displays various statistics on the state of the CRYPTOCard©.  Pressing any key causes the CRYPTOCard© to display the next screen of statistical data.  All data is presented in hexadecimal.  Refer to the diagrams on the following page for details.

0

Pressing any key causes the CRYPTOCard© to revert to primary mode.

# 7.	MESSAGES AND LANGUAGE OPTIONS

The following is a list of messages corresponding to the various supported languages.

English-1	English-2	French	German	Italian

Card OK	Card OK	Correct		Karte OK	Carta OK

Invalid		Invalid		Faux		Ungültig	Invalido

Key?		Key?		Clef?		Chiffre?	Chiave?

Locked		Locked		Bloqué		Gesperrt	Chiuso

New PIN?	New PSC?	Nouveau?	Neue PIN	Nuovo?

Options?	Options?	Options?	Auswahl?	Scelta?

PIN?		PSC?		PIN?		PIN?		PIN?

Ready		Ready		Prêt		Fertig		Pronto

Spaces!	Spaces!	Espaces!	Zw. Raum	Spazi!

User ID?	User ID?	Usagé		Ident. #		Usare?

Verify		New PSC?	Verifier	Beweise	Verifica

English-1	Portuguese	Swedish	Spanish

Card OK	Correcto	Kort OK	Correcto

Invalid		Errado		Ogiltig		Nulo

Key?		Codigó		Tangent?	Clave?

Locked		Nulo		Spärrad	Atascado

New PIN?	Nôvo PIN	Ny PIN?	Nuevo?

Options?	Opçôes?	Val?		Opciones

PIN?		PIN?		PIN?		PIN?

Ready		Avance		Klar		Listo

Spaces!	Espaços!	Tomrum!	Espacios

User ID?	Nr.Conta	Kund-ID?	Ident. #

Verify		Repetir		Bekräfta	Asegurar

# 8.	OCTAL CHARACTER SET

Alphanume ric (ASCII) characters are represented in octal as follows:

ASCII	          Octal	           ASCII               Octal        	       ASCII       Octal

space		040		G		107		n	156

!		041		H		110		o	157

"		042		I		111		p	160

#		043		J		112		q	161

$		044		K		113		r	162

%		045		L		114		s	163

&		046		M		115		t	164

'  (apostrophe)	047		N		116		u	165

(		050		O		117		v	166

)		051		P		120		w	167

*		052		Q		121		x	170

+		053		R		122		y	171

,  (comma)	054		S		123		z	172

-		055		T		124		{	173

.  (period)	056		U		125		|	174

/		057		V		126		}	175

0		060		W		127		~	176

1		061		X		130		rubout	177

2		062		Y		131

3		063		Z		132

4		064		[		133

5		065		\		134

6		066		]		135

7		067		^		136

8		070		_ (underscore)	137

9		071		`		140

:  (colon)	072		a		141

;  (semi-colon)	073		b		142

<		074		c		143

=		075		d		144

>		076		e		145

?		077		f		146

@		100		g		147

A		101		h		150

B		102		i		151

C		103		j		152

D		104		k		153

E		105		l		154

F		106		m		155

# 9.	REDUCED INPUT MODE - GENERAL INFORMATION

## 9.1	PURPOSE

To implement an operating mode for the CRYPTOCard© in which usage is simplified by reducing the number of card keystrokes required.  This is done by using the results from one challenge as the default input to the next one.

## 9.2	SECURITY

This mode of operation may be perceived as not being as secure as random challenges.  i.e. challenge strings appear to become predictable based on previous results.  However, the challenge is actually not predictable unless the encryption key is known.  This is because, only the most significant half of the response is ever revealed on the display.  The least significant half remains secret.  (See notes on 'algorithm' below).

## 9.3	INITIALIZATION

The most significant bit of the digit 1 of option field 2 will be set to indicate use of this mode.  The possible octal values for digit 1 then become:

0	- standard challenge entry, no user ID required

1	- standard challenge entry, user ID required

2	- reduced keystroke mode, no user ID required

3	- reduced keystroke mode, user ID required

## 9.4	OPERATION

Standard Operation:  With standard challenge entry, when the user ID or READY message appears, the user can press either ENTer or CHallenge/MAC and a blank screen appears.  He then proceeds to enter the challenge string.  If standard mode is selected in a card with software which provides reduced mode, the card will operate identically to previous versions that did not have this feature.  Full backward compatibility is maintained.

Reduced Operation:  In reduced keystroke mode, when ENTer is pressed after the display of user ID or READY, the result of encrypting the modified version of the previously entered challenge string is displayed.  This is referred to as the default challenge.  Pressing ENTer again, displays the result of encrypting the displayed string.  (Note: At this point, ENTer cannot be used to enter a new challenge string.  This must be done by pressing either CHallenge/MAC or CLR keys, entering the string, then pressing ENTer.)  If ENTer is subsequently pressed, USER ID appears and the cycle begins again.

The host software displays a challenge string for the user at logon time.  When the user presses ENTer after user ID or READY appears, he can compare the token's default challenge string with the one displayed by the host.  If they are the same he simply presses ENTer again to generate the response.  If they differ, he must press CHallenge/MAC or CLR and enter the explicit challenge string as displayed by the host.

It should be noted here that the host system can decide whether or not to use the default challenge in any given circumstance.

The first time a user operates the CRYPTOCard© using reduced keystroke mode, the default challenge appears as all 0's on his display.  Upon the first logon attempt, the host could also use this value, but it might be best to force a full challenge for security reasons.  Once either option is exercised, the host and card thereafter become synchronized.  However, if synchronization is ever lost, the user simply enters the challenge string again.

After each challenge operation, the modified challenge is always stored, ready for the next possible use.  If a challenge is explicitly entered, it then becomes the new string to be modified and stored.

## 9.5	HOST

The host software will be required to store the modified version of the previous challenge string in its secure database.  It then modifies this string each time, the same way that the CRYPTOCard© modifies it.

## 9.6	MULTIPLE KEYS

In a token with more than one key selected at initialization time, the reduced keystroke feature, if enabled, will retain only the last challenge.  It will not do this for each key.  Therefore, this feature will be useful to someone who uses one service frequently and others occasionally, and not so useful to someone who accesses a variety of services frequently.

## 9.7	ALGORITHM

A simple algorithm must be used to recursively generate new challenge strings from an initial string.  The algorithm implemented works as follows:

- All 8 bytes of the response to the challenge string are used, not just the 4 displayed bytes.

- The value of most significant nybble of each byte is changed to 3H.

- Any occurrences of the hexadecimal characters AH through FH in the least significant nybble of each byte are changed to 0H through 5H respectively.  The resulting 8 bytes can then be interpreted as a string of 8 ASCII characters representing an 8 digit decimal number.  This becomes the default challenge and appears on the display as the decimal digits represented by the ASCII characters.  The 8 ASCII characters are used as the 8 byte challenge, just as in other modes.

Note again that only half of the response to a challenge is displayed, yet the whole response is used to generate a new default challenge.  Therefore, an intruder can't predict a new challenge string from the response to a previous challenge unless he knows the DES key.

# 10.	CRYPTOCard© TOKENS AND THE DES ALGORITHM

The Cryptocard challenge-response token employs a implementation of the standard DES algorithm.

## 10.1	INPUT

The user enters a string of decimal digits via the keypad.  The minimum string length is 5 digits with no maximum.  For DES, the nominal plain text block length is 56 bits plus 8 bits of parity.  We ignore the fact that the last byte of each block may or may not be parity.  We process 64 bits regardless; parity is not checked ( or generated).

On input, each decimal digit forms a full byte of the input block.  The value of the byte is the ASCII equivalent of the digit; i.e. '0' decimal becomes 30 hex and '9' decimal becomes 39 hex.  If there are fewer than 8 decimal digits in an input block, padding in the form of binary zeroes is added at the least significant end.

If more than 8 digits are input, then the first 8 digits are processed as the input block as stated above and cipher block chaining is used thereafter.  If the last block is incomplete, then it is padded with zeroes at the least significant end.

## 10.2	OUTPUT

There are 3 output display modes; one of them is selected at initialization time:

1)	Hexadecimal Display

The result of encrypting the 64 bit input block is a 64 bit cipher.  Since the token display has 8 characters, only the most significant half of this result can be displayed.  Each hexadecimal digit displayed represents the equivalent of 4 binary bits from the cipher.

2)	Decimal Display

The decimal display is derived from the hexadecimal result by subtracting 10 (hexadecimal A) from any hexadecimal digit between A and F.

3)	Telephone Number Display

This option works in both hexadecimal and decimal modes.  If this option is selected at initialization time, the fourth most significant digit on the display is simply replaced with a hyphen character '-'.

## 10.3	MAC OPERATION

Since a string of 5 digits or more can be entered, a MAC can be generated for a numeric string of any length greater than 4 digits.

## 10.4	DES EXAMPLE

Here is an example of a DES Key and the results to be expected from the CRYPTOCard© when this key is loaded.

Hex		B9	1A	0B	D8	E6	7A	DE	19

Octal		271	032	013	330	346	172	336	031

Input String		Hex Result		Decimal Result		Telephone Format

(Decimal)

1526731		BDC9B0CC		13291022		132-1022

5193012		012B1B00		01211100		012-1100

040989100000030	A3C103FE		03210354		032-0354

051089			27F06982		27506982		275-6982