Using CRYPTOCard© Authentication Tokens

in Client-Server Environments

A “White Paper” Describing

CRYPTOCard Support for RADIUS and TACACS+

(Revision 1.0)

23 January, 1996

Research and Development

CRYPTOCard Corporation

160 Industrial Avenue

Carleton Place, Ontario, CANADA

K7C 3T2

Tel: (613) 253-8476    Fax: (613) 253-8493

Revision History

Applicability

Overview

Terminology

The Purpose of an Authentication Server

A Brief Description of TACACS+

A Brief Description of RADIUS

The CRYPTOCard RB-1 Token in Client-Server Environments

Using the RB-1 Token

Adding RB-1 Authentication Features to TACACS+

Modification of the Original Code

Configuration

Adding RB-1 Authentication Features to RADIUS

Modification of the Original Code

Configuration

CRYPTOCard Token Database File

References

Appendix

Flow of Data between NAS and TACACS+ Daemon

Flow of Data between NAS and RADIUS Server

# Revision History

23-Jan-96 - v1.0: Stephen Seal: Combine TACACS+ and RADIUS documents; Format for publishing

23-Jan-96 - v0.9: Joseph Lee: Initial Draft

# Applicability

The information contained in this document applies to CRYPTOCard’s implementation of Cisco’s TACACS+ model authentication server (version 2.1) and to Livingston’s model of a RADIUS authentication server (base version 1.16.5.x).

The software for both of these implementations is available in source code form from CRYPTOCard’s WWW site at http://www.cryptocard.com or directly through CRYPTOCard’s ftp site ftp://ftp.cryptocard.com/pub/RADIUS and ftp://ftp.cryptocard.com/pub/TACPLUS.  There is no charge for the use of this software, however no warranty either expressed or implied is provided with this software.  User’s of this software do so at their own risk and CRYPTOCard will not be held responsible for any loss resulting from the use or misuse of this software.  Support for this software will be provided on an “as available” basis.

Please feel free to contact us with any comments.  You may contact us 	by phone (613) 253-8476, fax (613) 253-8493 or e-mail support@cryptocard.com.

TACACS+ is (c) 1995 by Cisco Systems

RADIUS is in Internet Draft form.

CRYPTOCard and CRYPTOCard Initializer are (c) 1991-1996 by CRYPTOCard Corporation

# Overview

The need for network access security has become increasingly important with the rapid growth of remote PC telecommunications technology.  In general, a network access security system consists of access servers, routers and a centralized database that stores the necessary information required for authentication, authorization and accounting purposes.  In addition, a protocol specification is required to define the rules for transferring information between an access server and the database. CISCO's TACACS+ protocol and Livingston’s RADIUS protocol are designed with these objectives in mind.

In the case of authentication, it is no longer sufficient to rely on static passwords (eg via PAP) as dial-up and single sign-on access control mechanisms. Even CHAP does not provide the highest level of authentication security since this methodology does not authenticate the end-user of the system.  Unauthorized access to computers and networks through the use of easily guessed, or shared passwords has led to increasingly critical information system breaches and associated economic losses. The corrupted password of even one user can readily lead to the security collapse of an organization's entire information system.

CRYPTOCard's challenge-response technology allows a secured host computer to "see through" a complex network and authenticate a prospective user regardless of the physical location of that user.

By equipping authorized users with something unique (the CRYPTOCard Token), something known only to the users (Personal Identification Numbers [PlNs]), and an encryption process that employs completely random passwords and secure algorithms, legitimate users can quickly and securely log-on to computers and networks without the risk of ever revealing a password to a potential attacker.

As a result, CRYPTOCard has incorporated its RB-1challenge-response authentication technology into both the TACACS+ and RADIUS environments. Users can now implement our RB-1 tokens in their network access systems and use third party network access servers which provide either TACACS+ or RADIUS protocol implementations to manage user authentication requests.

# Terminology

The following terms are used throughout this document:

Challenge - A random string of digits that an authentication server issues to a user attempting to gain access to a computer network.

Challenge-Response - A methodology to confirm the identity of a person or computer program that uses random challenges and unguessable and unpredictable responses.  A challenge-response system provides enchanced security by equipping all users who will be gaining access to a system with a special calculator device (a token) that must be used to calculate a response for any given challenge.

Client - A computer program that makes requests for services from a Server.

CRYPTOCard Initializer - a hardware device that is used to program CRYPTOCard’s RB-1 calculator devices.

CRYPTOCard RB-1 - a challenge-response calculator or token.

Daemon - A single process on a UNIX operating system.  In the context of this paper it refers to the TACACS+ or RADIUS program running on a UNIX platform that is providing the authentication services.

NAS - Network Access Server.  A communications gateway that provides remote users with access to a local area network.  A NAS normally provides, but is not restricted to providing, public telephone access to a network.

Response - The result of encrypting a challenge with a secret key using a predefined one-way hashing algorithm.  In this context the Data Encryption Standard (DES) is used [1].

Server - A computer program or system that provides some service to other programs or systems on a network.  In the context of this paper, it refers to the system providing the authentication services.

TCP - Transmission Control Protocol.  A transport level protocol that provides reliable, full duplex, stream service from one process to another [4].

UDP - User Datagram Protocol.  A standard protocol that allows an application program on one machine to send data to another machine [4].

# The Purpose of an Authentication Server

The main purpose of an authentication server is to provide three basic functions of  network access: authentication, authorization and accounting.

Authentication is the process of determining the identity of an end user who tries to get access to a protected network.

Authorization is the process of  determining what an end user can do within the perimeter of the protected network.

Accounting is the process of recording what an end user is doing or has done in the protected network.

Since we are mainly concerned with the authentication process, we will not describe authorization and accounting in this paper. Interested readers can contact most third party NAS vendors for more information on these topics.

The relationship between each component in a remote access system serviced by an authentication server is shown below in Figure 1 - System Components.

Figure 1 - System Components

## A Brief Description of TACACS+

TACACS+ uses TCP for reliable transport. There are three authentication packet types in TACACS+: START, CONTINUE and REPLY.  A START packet signifies the beginning of an authentication process. It contains information such as the type of authentication to be performed and somtimes the username.  A REPLY packet is used by the TACACS+ daemon to signify whether an authentication has succeeded or whether further information is required to carry on the authentication process.  In the latter case, a REPLY packet will also contain instructions on what kind of new information is required.  To supply the new information to the daemon, the server will ask the user to enter the required information and embed it in a CONTINUE packet sent back to the daemon.

Note that the REPLY packet is the only packet that is generated by the daemon and it is always sent in response to a START and a CONTINUE message (except when the client NAS indicates an abort in the CONTINUE packet). On the other hand, the NAS can only send START and CONTINUE messages to the server. START messages are sent only to initiate a authentication session or to initiate a restart. All other messages from the NAS are contained in CONTINUE packets.

For a detailed review of the TACACS+ protocol see the appendix and consult [3].

## A Brief Description of RADIUS

RADIUS uses UDP for transporting authentication packets back and forth between the NAS and the RADIUS daemon.  There are four types of packets: Access-Request, Access-Accept, Access-Reject and Access-Challenge that are used by RADIUS for authentication purposes.

Access-Request packets are sent only by the NAS to the RADIUS daemon.  These contain information used by the RADIUS daemon to determine whether a particular user should be granted access to the internal network connected to the NAS.  Access-Accept, Access-Reject and Access-Challenge packets are all generated by the RADIUS daemon.  Access-Accept packets are sent to the NAS when an authentication attempt is successful.  On the other hand,  Access-Reject packets are sent to the NAS when an authentication attempt is unsuccessful.  Access-Challenge packets are sent to the NAS when further information is required for authentication.

For a detailed review of the RADIUS protocol see the appendix and consult [5].

# The CRYPTOCard RB-1 Token in Client-Server Environments

## Using the RB-1 Token

There are two modes of authentication in CRYPTOCard's RB-1 token: full challenge-response mode and event-synchronous (reduced input) mode. In the following example, we assume that an end user is trying to prove her identity to an authentication server through a NAS by showing that she has the possession of a shared secret (the encryption key) which is contained in the RB-1 token.  The RB-1 is assumed to be pre-loaded with a unique secret key that is known only by the server and the RB-1 token (even the user does not know the secret).

In the full challenge-response mode, when the end user attempts to get access to the network (e.g. , by dialing up, by telnet, etc. ), the NAS will prompt the user for a unique user name and contact the server.   If the username provided is valid and known to the server daemon, the server daemon will generate a random string consisting of 8 characters (from 0 to 9) and send it to the user through the NAS.  In addition, the server daemon will keep a record of this challenge for later use.

Upon receiving this random string (the challenge), the user has to encrypt it by typing it into the RB-1 and using the RB-1 to calculate the response.  This response is then given to the NAS which sends it back to the server daemon.  By looking up an internal database, the server daemon can retrieve a secret key that is assigned to the end user.  The server daemon will then try to check the validity of the response by encrypting the challenge using the end user's secret key.  If the response generated by the server matches that entered by the end user, access to the server informs the NAS that access may be granted.  Since the odds of guessing a correct response by a remote user who does not know the secret key is extremely small, a correct response entered by the end user is taken as a proof that she has possession of the shared secret.

In the event-synchronous mode, a default challenge is generated by manipulating the previously used response string.  This default challenge is stored in both the server and in the end user's RB-1 token device.  When the user attempts to access the network she can compare the token's default challenge string with the one given to the NAS by the server.  If they are the same, the end user simply presses ENTER in the token device to get the response, without having to enter the challenge again in the token device, thus saving 8 keystrokes.  If for some reason, the default challenge strings do not match, the user simply has to enter the challenge string as determined by the server into the RB-1 and press ENTER to get the response and at the same time restore the synchronization process.  The first challenge, is set arbitrarily by the server, and the end user has to enter it explicitly in the token device the first time she logs on to get the token synchronized with the server.   Note that the RB-1 must be configured to provide the event-synchronous mode of operation.

The encryption algorithm used in CRYPTOCard's RB-1 authentication token and the server daemon is the Data Encryption Standard [1]. The algorithm for generating a new challenge from a previous challenge can be found in [2].

## Adding RB-1 Authentication Features to TACACS+

### Modification of the Original Code

For the files that come with the original TACACS+ 2.1 server code, any extra code that is added to them starts with the statement "#ifdef CRYPTOCARD" and ends with the statement "#endif".  Hence, in order to compile the modified server code to support CRYPTOCard's RB-1 authentication token, simply define the constant "CRYPTOCARD" in the file "tac_plus.h".  To use the generic TACACS+ login authentication function, simply UNdefine the constant CRYPTOCARD in the header file "tac_plus.h".

### Configuration

Once the souce code to the TACACS+ daemon has been compiled to support the CRYPTOCard RB-1 authentication token, the system administrator can configure a RB-1 token user's name and key in the configuration file (see sample file "cfgfile").  The implementation follows Cisco’s original resursive configuration file structure.  The username and key may be configured in either of two ways [6]:

Figure 2 - TACACS+ Configuration File

(1) user = <username> {

token = cleartext <key>

<nextchallenge = xxxxxxxx>

.

.

}

or

(2) user = <username> {

token = file <filename>

.

.

}

or

(3) user = <username> {

member = token_group

.

.

}

group = token_group {

token = file <filename>

.

.

}

In (1), <key> is user <username>'s token DES key and it is specified in a cleartext format.  It could be in either octal (3 octal digits for 1 byte) or hexadecimal format (with the prefix 0x).

To indicate event-synchronous mode, simply add the statement <nextchallenge = xxxxxxxx> where xxxxxxxx indicates 8 hexadecimal digits that will be used as the next challenge. The absence of this statement implies that full challenge-response mode is used instead of reduced input mode.

In (2), user <username>'s token key is stored in a CRYPTOCard Token Database file.  This file stores the username, key, display format (hexadecimal, decimal, telephone or non-telephone mode) and the default challenge (if applicable) of each token user.  A sample cryptocard database file is provided in Figure 1 - System Components

## Adding RB-1 Authentication Features to RADIUS

### Modification of the Original Code

For the files that come with the original RADIUS 1.16 server code, any extra code that is added to them will start with the statement "#ifdef CRYPTOCARD" and end with the statement "#endif".  Hence, in order to compile the modified server code to support CRYPTOCard's RB-1 authentication token, simply define the constant "CRYPTOCARD" in "cryptocard.h".  To use the generic RADIUS login authentication

function, simply UNdefine the constant CRYPTOCARD in the header file "cryptocard.h".

### Configuration

Once the RADIUS source code has been compiled to support CRYPTOCard's

RB-1 authentication token, the system administrator can configure a RB-1 token user's

name and what he/she is allowed to do once authentication has succeeded in the file

"users".  A sample configuration for a RB-1 token user is as follows [7]:

Figure 3 - RADIUS Users File

<username>	Token = CRYPTOCARD

User-Service-Type = Framed-User,

Framed-Protocol = PPP,

Framed-Address = 160.0.1.10,

Framed-Netmask = 255.255.255.0,

Framed-Routing = Broadcast-Listen,

Framed-Compression = Van-Jacobsen-TCP-IP,

Framed-MTU = 1006

Framed-IPX-Network = 255.255.255.254

To indicate full challenge-response mode, simply replace the statement "Token  =

CRYPTOCARD"  by "Token  =  RANDOM_CRYPTOCARD".

<username> 's token is stored in a cryptocard database file.  The database file stores the username, key, display format (hexadecimal, decimal, telephone or non-telephone mode) and the default challenge (if applicable) of each token user.  The cryptocard database file is described below.

## CRYPTOCard Token Database File

The Token Database file is a flat ASCII text file that contains information about all RB-1 tokens issued to users.  A sample of this file is shown below:

Figure 4 - Token Database File

# This is the CRYPTOCard definition database file.  It contains

# all the information needed to identify CRYPTOCard users to the server daemon.

#

# Comments start with a # sign.

# Under the display column:

#	0 implies hexadecimal with no phone display,

#	1 implies hexadecimal with phone display,

#	2 implies decimal with no phone display, and

#	3 implies decimal with phone display.

#

#User    Key(Encrypted)                                  Display Challenge

#------  ------------------------------------------------ ----- -----------

steve    1a76e42354f70e0318d45d256efc205d3ce30d322430b08c    0    86029492

joe      c583874d77da7b2dd7e8aa45d00093afd511dfd516c5bfab    0    95822142

joseph   4d628a0c5dbf03b5c55611161bfd3e14a6de75ff479442d4    0

Each user's key is encrypted by Xoring it with the MD5 hashed value over a string formed

by concatenating the username to a secret value S.  Please refer to the README file included with each server  for further details on the encryption algorithm.

Since the username is in plaintext format in both the configuration file and the database file, the security of this algorithm lies totally on the obscurity of the secret value S, which is stored in MASTERKEY_FILE as defined in "cryptocard.h".  For this purpose, THE FILE  "MASTERKEY_FILE" SHOULD BE PROTECTED AND ACCESSIBLE ONLY BY THE SYSTEM ADMINISTRATOR.

Although one could argue that since both the configuration file and the database file should be accessible only by the system administrator, the mere fact that a cryptanalyst has got access to these files imply that the MASTERKEY_FILE is also within the grasp of the attacker, and hence the encryption of the user's key serves no purpose at all.  Nonetheless, the encryption of each user's key in the database file still provides a better degree of obscurity to the token keys over an unencrypted database format.

To indicate event-synchronous mode, add the nextchallenge (8 hexadecimal digits) at the end of each user's record.  The absence of this piece of data implies that full challenge-response mode is used instead of reduced input mode.

The database file can be generated  by running CRYPTOCard's Initializer Interface program.  This  program provides a handy way to program RB-1 tokens as well as maintain the database file.  It is available from CRYPTOCard’s WWW site at http://www.cryptocard.com or directly through our ftp site at ftp://ftp.cryptocard.com/pub/initializer.

# References

[1] NBS FIPS PUB 46, "Data Encryption Standard", National Bureau of Standards, U.S. Department of Commerce, January 1988.

[2] "CRYPTOCard User Authentication Token Operation & Systems Guide", CRYPTOCard Corporation, Sept. 1994.

[3] "TACACS+ Protocol Specification", Cisco Systems.

[4] Taken from: Comer, Douglas E., “Internetworking with TCP/IP - Volume 1”, 2nd edition, 1991, Prentice-Hall Inc.

[5] Rigney, C, et al “- RADIUS - Remote Authentication Dial-in User Service”, IETF Working Group, Internet Draft

[6] see also TACACS+ Server users guide, provided with Server code.

[7] see also the Livingston Portmaster Users Guide and the RADIUS Server instructions provided with the Server code.

# Appendix

## Flow of Data between NAS and TACACS+ Daemon

Once the user gets into contact with the NAS, authentication will start as follows:

- An authentication START packet is sent by the NAS to the TACACS+ daemon. In the packet body, the "authen_type" field is set to TAC_PLUS_AUTHEN_TYPE_ASCII, the "service" field is set to TAC_PLUS_AUTHEN_SVC_LOGIN (see [3] for the representation of these constants) and the "user" field is set to contain the end user's name.

- Upon receiving a packet from the NAS, the daemon will "fork" a new process and this new process will handle the request of this particular user who wants to be authenticated.

- An authentication REPLY packet is sent to the NAS by the forked process. Regardless of whether the username that came with the START packet is valid or not, a challenge will be put in the "server_msg" field in the REPLY packet body.  In cases where the username is invalid or where a valid username is received and the user is configured in full challenge-response mode, a random challenge will be generated. Otherwise, the default challenge (from configuration file or from a cryptocard database file, depending on how things are configured) is retrieved and put in the "server_msg" field. At the same time, the challenge will be stored in internal memory within the forked process.

- An authentication CONTINUE packet is sent by the NAS to the TACACS+ daemon. The response entered by the user is stored in the "user_msg" field in this packet.

- The forked process in the daemon, upon receiving the CONTINUE packet from the NAS for this particular user, will try to see if the entered response is equal to the expected response (which it computes by encrypting the stored challenge using the user's secret key).  The forked process in the daemon will then send an authentication REPLY packet back to the NAS, and the "status" field of this packet will indicate whether the authentication has succeeded or not.  If authentication has succeeded, user would begranted access to the LAN, and authorization parameters in the configuration file will dictate what the users could do thereafter.  At the same time, if the user is configured in event-synchronous mode, the default challenge stored in configuration file or in cryptocard database file will be updated.

- The forked process will end at this point of time.

Note that all packets sent between the NAS and Server are encrypted using MD5 and a shared secret.

## Flow of Data between NAS and RADIUS Server

Once the user gets into contact with the NAS, authentication will start as follows:

- An Access-Request packet is sent by the NAS to the RADIUS server.  This packet will contain information such as username that is required to initiate an authentication process.

- Upon receiving an Access-Request packet from the NAS, the RADIUS server will "fork" a new process and this new process will handle the request of this particular user.

- An Access-Challenge packet is sent back to the NAS by the forked process. If the username that came with the Access-Request packet is valid, a challenge will be embedded in an Access-Challenge packet and sent back to the NAS. The default challenge (from a cryptocard database file) will be used if applicable otherwise a random challenge will be calculated by the server daemon and sent back to the NAS.  If the username is invalid an Access-Reject packet will be sent back to the NAS.  The issued challenge will be stored in the cryptocard database file before the forked process dies out.

- An Access-Request packet is sent by the NAS to the RADIUS server.  This packet will contain the response entered by the user.

- The RADIUS server will "fork" a new process and this new process will try tosee if the entered response is equal to the expected response (which it computes by encrypting the stored challenge using the user's secret key).  The forked process in the RADIUS server will then send an Access-Accept or an Access-Reject packet back to the NAS.  If authentication has succeeded, user would be granted access to the LAN, and authorization parameters in the configuration file will dictate what the users could do thereafter.  At the same time, if the user is configured in event-synchronous mode, the default challenge stored in the cryptocard database file will be updated.

- The forked process will end at this point of time.

Note that all packets sent between the NAS and Server are authenticated by using a MD5 hash on a 16 byte authenticator and a shared secret known by the NAS and Server.