BandWagon

Configuration

Guide

Draft Copy

Rev. 1.0

Willie Hernandez

February 6, 1996

Table of contents

Notes Cautions and Warnings

About This Guide

Introduction

Chapter 1.  Type of Commands

Command Interface Structure

User Friendly Features

Help Facility

Commands Buffer

Command line editing

BandWagon Command Definition

Chapter 2.  \statistics path

\statistics\ras

\statistics\ras\kmesg

\statistics\ras\uname

\statistics\ras\ps

\statistics\ras\stack

\statistics\ras\sas

\statistics\ras\date

\statistics\network

\statistics\network\ping

\statistics\network\ipxping

\statistics\network\hosts

\statistics\network\ruptime

\statistics\network\ip

\statistics\network\ipx

\statistics\network\netbeui

\statistics\network\netstat

\statistics\network\arp

\statistics\network\route

\statistics\network\acs

Chapter 3.  \administration path

\administration\activity

\administration\wall

\administration\write

\administration\reboot

\administration\detach

\administration\date

\administration\ifconfig

\administration\reset

\administration\route\ip

\administration\route\ipx

\administration\route\netbeui

\administration\path\start

\administration\path\stop

\administration\path\list

\administration\path\delport

\administration\who

Chapter 4.  \Configuration directory

\configuration\runtime

BandWagon:\\configuration\runtime\maintenance

\configuration\runtime\ports

\configuration\runtime\network\ip

\configuration\runtime\network\ipx

\configuration\runtime\network\snmp

\configuration\runtime\network\acs>

\configuration\runtime\ras

\configuration\permanent

\configuration\permanent\maintenance

\configuration\permanent\ports

\configuration\permanent\network\ip

\configuration\permanent\network\ipx

\configuration\permanent\network\snmp

\configuration\permanent\network\acs>

\configuration\permanent\ras

\configuration\permanent\service

\configuration\permanent\winras

\configuration\path

\configuration\security

\configuration\security\users

\configuration\security\sas (security access server)

\configuration\security\audittrial

\configuration\modemchats

\configuration\logchats

Chapter 5.  Configuration Guidelines

Adding a user

Adding a PATH

IP connection

IPX

NetBEUI

Setup the RAS

IPX Configuration

NetBEUI Configuration

ISDN Configuration

ACS Configuration

SNMP Configuration

IP tunnelling Configuration

Modem Chat Configuration

Log Chat Configuration

SAS Configuration

TCP/IP Client

User Configuration (for ip)

IPX Client

NetBEUI Client

NASI Client

Chapter 6. BandWagon RAS Main Board Diagnostics & Power On Self Test Document

User Interface

Power on Sequence

DIAGNOSTICS

Main Menu of the BandWagon RAS Diagnostics

Main Board Logic Tests Menu

ASIO Adapter Tests Menu

Network TESTS

Sync Card Tests

System Debug Aid

Description Of Tests

Memory Test

Cache Tests

RTC Test

Flash Test

NV RAM Test

Continuous READ/WRITE Memory

Continuous READ/WRITE - I/O

Dump Memory

DUMP I/O

Examine Memory

Examine I/O

DPRAM Test

Mail Box Test

HOST-ASIO Communications Test

Reset ASIO

Continues TEST

79C970Internal Loop Back Test

79C970 MENDEC Loop Back Test

External Loop Back Test

Single/Continuous Mode

Display Station Address

Table Of Figures

Figure 1. BandWagon Rear View

1 - Figure 2 - Main Menu

Figure 3 Command Line Tree

Figure 4 - \statistics\ras\kmesg

Figure 5 - BandWagon example:  \statistics\ras\kmsg

Figure 6 - BandWagon definition - \statistics\ras\uname

Figure 7 - BandWagon example: \statistics\ras\uname

Figure 8 - BandWagon Definition - \statistics\ras\ps

Figure 9 - BandWagon example: \statistics\ras\ps

Figure 10 - BandWagon's Definition - \statistics\ras\stack

Figure 11 - BandWagon's example: \statistics\ras\stack

Figure 12 - BandWagon's definition - \statistics\ras\sar

Figure 13 - BandWagon example: \statistics\ras\sar -u

Figure 14 - BandWagon example: \statistics\ras\sar -i

Figure 15 - BandWagon example: \statistics\ras\sar -m

Figure 16 - BandWagon example: \statistics\ras\sar -t <port>

Figure 17 - BandWagon example: \statistics\ras\sar -I

Figure 18 - BandWagon's definition - \statistics\ras\date

Figure 19 - BandWagon example: \statistics\ras\date

Figure 20 - BandWagon's definition: \statistics\network\ping

Figure 21 - BandWagon example: \statistics\network\ping

Figure 22 - BandWagon definition: \statistics\network\ipxping

Figure 23 - BandWagon example: \statistics\network\ipxping

Figure 24 - BandWagon definition: \statistics\network\hosts

Figure 25 - BandWagon example: \statistics\network\hosts

Figure 26 - BandWagon definition - \statistics\network\ruptime

Figure 27 - BandWagon example: \statistics\network\ruptime

Figure 28 - BandWagon's definition: \statistics\network\ip

Figure 29 - BandWagon example: \statistics\network\ip i, -a, -n

Figure 30 - BandWagon Example: \statistics\network\ip -s

Figure 31 - BandWagon example: \statistics\network\ip -s tcp

Figure 32 - BandWagon example: \statistics\network\ip -c 1

Figure 33 - BandWagon example: \statistics\network\ip -s route

Figure 34 - BandWagon example: \statistics\network\ip -r

Figure 35 - BandWagon example: \statistics\network\ip -s icmp

Figure 36 - BandWagon's definition: \statistics\network\ipx

Figure 37 - BandWagon example: \statistics\network\ipx -a

Figure 38 - BandWagon example: \statistics\network\ipx -s rip

Figure 39 - BandWagon example: \statistics\network\ipx -s sap

Figure 40 - BandWagon example: \statistics\network\ipx -r

Figure 41 - BandWagon example: \statistics\network\ipx -i

Figure 42 - BandWagon example: \statistics\network\ipx -u

Figure 43 - BandWagon  example: \statistics\network\ipx -n

Figure 44 - BandWagon's definition: \statistics\network\netbeui

Figure 45 - BandWagon example: \statistics\network\netbeui -g

Figure 46 - BandWagon example: \statistics\network\netbeui -I

Figure 47 -BandWagon example: \statistics\network\netbeui -I <interface>

Figure 48 - BandWagon example: \statistics\network\netbeui -s

Figure 49 - BandWagon's definition: \statistics\network\netstat

Figure 50 - BandWagon example: \statistics\network\netstat -s

Figure 51 - BandWagon example: \statistics\network\netstat -m

Figure 52 - BandWagon example: \statistics\network\netstat -h

Figure 53 - BandWagon example: \statistics\network\netstat -I <interface>

Figure 54 - BandWagon's definition: \statistics\network\arp

Figure 55 - BandWagon's definition: .. \statistics\network\route ip

Figure 56 - BandWagon example: .. \statistics\network\route ip

Figure 57 - BandWagon's definition: \statistics\network\network ipx

Figure 58 - BandWagon example: .. \statistics\network\route ipx

Figure 59 - BandWagon's definition: .. \statistics\network\route netbeui

Figure 60 - BandWagon example: .. \statistics\network\route netbeui

Figure 61 - BandWagon's definition: \statistics\network\acs

Figure 62 - BandWagon example: acs -p

Figure 63 - BandWagon example: \statistics\network\acs -u

Figure 64 - BandWagon example: \statistics\network\acs -r

Figure 65 - BandWagon example: \statistics\network\acs -d

Figure 66 - BandWagon's definition: \administration\activity

Figure 67 - BandWagon example: \administration\ activity

Figure 68 - BandWagon's definition \administration\wall

Figure 69 - BandWagon example: \administration\wall

Figure 70 - BandWagon's definition: \administration\write

Figure 71 - BandWagon's definition: \activity\detach

Figure 72 - BandWagon example: \administration\detach

Figure 73 - BandWagon's definition: \administration\date

Figure 74 - BandWagon's definition: \administration\ifconfig

Figure 75 - BandWagon example: \administration\ifconfig

Figure 76 - BandWagon example: \administration\ifconfig -n <port>

Figure 77 - BandWagon's definition: \administration\reset

Figure 78 - BandWagon example:\administration\reset -n

Figure 79 - BandWagon's definition: \administration\route\ip\show

Figure 80 - BandWagon example: \administration\route\ip\show

Figure 81 - BandWagon's definition: \administrtion\route\ip\add

Figure 82 - BandWagon example: \administration\route\ip\add

Figure 83 - BandWagon's definition: \administration\route\ip\delete

Figure 84 - BandWagon example: \administration\route\ip\delete

Figure 85 - BandWagon's definition: \administration\route\ip\flush

Figure 86 - BandWagon's definition: \administration\route\ip\trigger

Figure 87 - BandWagon example: \administration\route\ipx\show

Figure 88 - BandWagon example: \administration\route\netbeui\show

Figure 89 - BandWagon's definition: \administration\route\netbeui\add

Figure 90 - BandWagon's definition: \administration\path start

Figure 91 - BandWagon example: \administration\path start

Figure 92 - BandWagon's definition: \administration\path stop

Figure 93 - BandWagon example: \administration\path stop

Figure 94 - BandWagon's definition: \administration\path  list

Figure 95 - BandWagon example: \administration\path list

Figure 96 - BandWagon's definition: \administration\path addport

Figure 97- BandWagon's definition: \administration\path delport

Figure 98 - BandWagon's definition: \administaion\who

Figure 99 - BandWagon example: \administration\who

Figure 100 - BandWagon's definition:  \configuration\runtime

Figure 101 - BandWagon's definition: \configuration\runtime\maintenance\upload

Figure 102 - BandWagon example:.-.\configuration\runtime\maintenance\upload

Figure 103 - BandWagon's definition:.-.\configuration\runtime\maintenance\gethost

Figure 104 - BandWagon example: - \configuration\runtime maintenance\gethost

Figure 105 - BandWagon's definition: \configuration\runtime\ports\show

Figure 106 - BandWagon example: \configuration\runtime\ports\show

Figure 107 - BandWagon's definition: \configuration\runtime\ports\modify

Figure 108 - BandWagon example: \configuration\runtime\ports\modify

Figure 109 - BandWagon's definition: \configuration\runtime\ports\groupmod

Figure 110 - BandWagon's definition: \configuration\runtime\network\ip\show

Figure 111 - BandWagon example: \configuration\runtime\network\ip\show

Figure 112 - BandWagon definition: \configuration\runtime\network\ip\modify

Figure 113 - BandWagon example: \configuration\runtime\network\ipx\show

Figure 114 - BandWagon example: \configuration\runtime\network\ipx\modify

Figure 115 - BandWagon example: \configuration\runtime\network\snmp\show

Figure 116 - BandWagon example: \configuration\runtime\network\isdn\digi\show

Figure 117 - BandWagon definition: \configuration\runtime\network\isdn\pri\show

Figure 118 - BandWagon's definition: \configuratin\runtime\network\netbeui

Figure 119 - BandWagon example: \configuration\runtime\network\acs\modify

Figure 120 - BandWagon example: \configuration\runtime\network\acs\show

Figure 121 - BandWagon's definitions: \configuration\runtime\ras\modify

Figure 122 - BandWagon's example:  \configuration\runtime\ras\show

Figure 123 - BandWagon's definition: \configuration\permanent\maintenance\upload

Figure 124 - BandWagon example:.-.\configuration\permanent\maintenance\upload

Figure 125 - BandWagon's definition: \configuration\permanent\maintenance\download

Figure 126 - BandWagon's example: \configuration\permanent\maintenance\download

Figure 127 - BandWagon's definition: \configuration\permanent\maintenance\factory

Figure 128 - BandWagon example: \configuration\permanent\maintenance\factory

Figure 129 - BandWagon's definition: \configuration\permanent\ports\show

Figure 130 - BandWagon example: \configuration\permanent\ports\show

Figure 131 - BandWagon's definition: \configuration\permanent\ports\modify

Figure 132 - BandWagon example: \configuration\permanent\ports\modify

Figure 133 - BandWagon's definition: \configuration\permanent\ports\groupmod

Figure 134 - BandWagon's definition: \configuration\permanent\network\ip\show

Figure 135 - BandWagon example: \configuration\permanent\network\ip\show

Figure 136 - BandWagon definition: \configuration\permanent\network\ip\modify

Figure 137 - BandWagon example: \configuration\permanent\network\ipx\show

Figure 138 - BandWagon example: \configuration\permanent\network\ipx\modify

Figure 139 - BandWagon example: \configuration\permanent\network\snmp\show

Figure 140 - BandWagon example: \configuration\permanent\network\iptunnel\show

Figure 141 - BandWagon example: \configuration\permanent\network\iptunnel\add

Figure 142 - BandWagon example:  \configuration\permanent\network\iptunnel\modify

Figure 143 - BandWagon example:  \configuration\permanent\network\iptunnel\delete

Figure 144 - BandWagon example:  \configuration\permanent\network\iptunnel\clear

Figure 145 - BandWagon example: \configuration\permanent\network\isdn\digi\show

Figure 146 - BandWagon definition: \configuration\permanent\network\isdn\pri\show

Figure 147 - BandWagon's definition: \configuratin\permanent\network\netbeui

Figure 148 - BandWagon example: \configuration\permanent\network\acs\modify

Figure 149 - BandWagon example: \configuration\permanent\network\acs\show

Figure 150 - BandWagon's definitions: \configuration\permanent\ras\modify

Figure 151 - BandWagon's example:  \configuration\permanent\ras\show

Figure 152 - BandWagon example: \configuration\permanent\service

Figure 153 - BandWagon example: \configuration\permanent\winras

Figure 154 - BandWagon example: \configuration\path

Figure 155 - BandWagon's example: \configuration\security\users\show

Figure 156 - BandWagon's user's database: adding a user

Figure 157 - BandWagon's definition: \configuration\security\users\modify

Figure 158 - BandWagon example: \configuration\security\users\modify

Figure 159 - BandWagon's definition: \configuration\security\users\delete

Figure 160 - BandWagon example: \configuration\security\users\delete

Figure 161 - BandWagon's definition: \configuration\security\user\reset

Figure 162 - BandWagon's definition: \configuration\security\user\clear

Figure 163 - BandWagon example: \configuration\security\user\clear

Figure 164 - BandWagon's definition: \configuration\security\user\upload

Figure 165 - BandWagon example: \configuration\security\user\upload

Figure 166 - BandWagon's definition: \configuration\security\user\download

Figure 167 - BandWagon example: \configuration\security\user\download

Figure 168 - BandWagon's definition:  \configuration\security\sas\show

Figure 169 - BandWagon example:  \configuration\security\sas\add

Figure 170 - BandWagon example:  \configuration\security\sas\modify

Figure 171 - BandWagon example:  \configuration\security\sas\delete

Figure 172 - BandWagon example:  \configuration\security\audittrial\show

Figure 173 - BandWagon's definition:  \configuration\security\audittrial\flush

Figure 174 - BandWagon's definition:  \configuration\security\audittrail\configure\loadpc

Figure 175 - BandWagon's definition:  \configuration\security\audittrail\configure\print

Figure 176 - BandWagon's definition:  \configuration\security\audittrail\configure\upload

Figure 177 - BandWagon example:  \configuration\security\audittrail\configure\upload

Figure 178 - BandWagon example:  \configuration\modemchats\show

Figure 179 - BandWagon's definition: \configuration\modemchats\add

Figure 180 - BandWagon example:  \configuration\modemchats\add

Figure 181 - BandWagon's definition:  \configuration\modemchats\delete

Figure 182 - BandWagon example:  \configuration\modemchats\delete

Figure 183 - BandWagon's definition:  \configuration\modemchats\upload

Figure 184 - BandWagon example:  \configuration\modemchats\upload

Figure 185 - BandWagon's definition:  \configuration\modemchats\clear

Figure 186 - BandWagon example:  \configuration\modemchats\clear

Figure 187 - BandWagon's definition:  \configuration\modemchats\download

Figure 188 - BandWagon example:  \configuration\modemchats\download

Figure 189 - BandWagon's definition:  \configuration\logchats\show

Figure 190 - BandWagon example:  \configuration\logchats\show

Figure 191 - BandWagon's definition:  \configuration\logchats\add

Figure 192 - BandWagon example:  \configuration\logchats\add

Figure 193 - BandWagon example:  \configuration\logchats\delete

Figure 194 - BandWagon example:  \configuration\logchats\clear

Figure 195 - BandWagon example:  \configuration\logchats\download

Figure 196 - BandWagon example:  \configuration\logchats\upload

# Notes Cautions and Warnings

Special notes, cautions and warnings may appear in this guide.   Each one has a special meaning and format.   They are as follows:

F Note: 	A note is supplementary or background information.   It may also be an alternative way to perform a given task or just a hint or a remainder to make things easier.

I Caution: 	This is more important than a note.   It provides crucial information relevant to the task or topic at hand.   Failure to follow the recommendation may cause system failure or damage system components or the equipment.   Always follow the recommendations indicated in the caution.

¥ Warning: 	A warning gives critical and vital information.   Failure to follow the warnings could lead to bodily harm to humans, or create serious legal liability.

For Proper Printing of Document Set Page Setup as Follows:

- MARGINS:

- Top:	.8"

- Botoom	1"

- Left	1"

- Right	1"

- Gutter	0

- Header	.4"

- Footer	.7"

- Apply to Whole Document.

# About This Guide

The BandWagon configuration guide is designed to give a system administration all the information needed to properly configure the BandWagon Remote Access Server, perform administration tasks, do routine maintenance and use the diagnostics embedded in the BandWagon's operating system.

This guide assumes the user is familiar with routing protocols such as IP, IPX and NetBui and their terminology.

This manual is provided to you as a Microsoft Word™ 6.0 document.  It has been formatted for standard 8.5x11 letter-size paper.

To preserve compatibility with a wide variety of systems, this manual uses only Arial, Time New Roman and windings TrueType fonts.  These fonts are distributed with Windows™ .  If you have removed them from your system, you can reload them from your windows diskettes.  Please refer to the windows documentation for instructions on this procedure.

You may feel free to print this document for your own personal use as many times as you like.  However, you cannot edit it or distribute it without LeeMah’s consent.

# Introduction

The objective of this document is to provide the network administrator with a detailed description of the user interface that is provided in the BandWagon Remote Access Server (RAS).  The user (craft) interface is provided in the rear of the BandWagon as shown in Figure 1. Below.

Figure 1. BandWagon Rear View

Console port

For information on connecting a terminal or PC to the BandWagon please refer to the BandWagon installation guide’s section “Connecting a Console Terminal” for more information.  In addition it is assumed that you have completed the section “Connecting to a TFTP Host Server” on the BandWagon Installation Guide.

Once you complete the connection to the TFTP server, your screen will look as shown in figure 1 below.

- Figure 2 - Main Menu

Welcome to the Remote Administration Facility

ras  :\>

Chose One  ->

statistics, administration, configuration, quit

->>

The main configuration screen shown above consists of:

- node name : current path > in first line

- list of commands/directories in this path in the second line

- a configurable prompt in the third line

The default node name is BandWagon

The default prompt is  ->>

# Chapter 1.  Type of Commands

The commands provided on the Remote Access Server craft interface will allow the system administrator to perform the following functions:

- Define user-ids, passwords, security mode, call back numbers, privileges, etc.

- Define port characteristics, such as port speed, parity, number of bits, flow control, etc.

- Define route table entries, ipx routing, IP routing for remote LAN access

- SNMP agent configuration

- Configuring ports for SLIP, or PPP operations

- Upload and download configuration file

- Perform Remote Access Server firmware upgrades

- Provide System wide diagnostics.

## Command Interface Structure

The command interface is organized in a tree like directory structure.  The commands are group according to their administrative activity.  The commands will be available under a functional group or directory.  For example, commands used to configure network parameters to be effective only at runtime (not saved in the configuration database) will be accessible in the subdirectory "configuration\runtime\network."

For a summary of the commands available on the Remote Access Server refer to Appendix A.

Under the root of the main command input line are the following sudirectories:

Statistics directory

This directory consists of commands that are used to display statistics about the RAS and the local network.

Administration directory

This directory consists of commands that are used to perform runtime administration activities.

configuration\runtime directory

This directory consists of commands that are used to configure the ports, ras, network protocol parameters and commands for maintenance.  It should be noted that any changes in the configurations are valid for the current runtime session only and not carried across reboots.

configuration\permanent directory

This directory consists of commands that are similar to those in configuration\runtime directory.  However, it should be noted that any changes in the configurations are permanent and available across reboots, but they take effect only from next reboot.

configuration\path directory

This directory consists of commands that are used to add, modify, delete and show the destination information in the path database.  It should be noted that any changes in the configurations are permanent and available across reboots.

configuration\users directory

This directory consists of commands that are used to add.  modify, delete, and show the user entries in the user database.  Commands, upload and download, are available for uploading or downloading the user database from a host in the network.  It should be noted that any changes in the configurations are permanent and available across reboots.

configuration\modemchats directory

This directory consists of commands that are used to add, delete, show and clear the modemchat scripts database.  Commands, upload and download, are available for uploading or downloading the modemchat database from a host in the network.  It should be noted that any changes in the configurations are permanent and available across reboots.

configuration\logchats directory

This directory consists of commands that are used to add, delete, show and clear the logchat scripts database.  Commands, upload and download, are available for uploading or downloading the logchats database from a host in the network.  It should be noted that any changes in the configurations are permanent and available across reboots.

Directory Navigation And Command Invocation

The root directory of the RAS command structure is "\".  The user can move from one directory to another using the "cd" command provided in the RAS.

For example: cd \configuration\permanent

The pathname specified in the cd command may be an absolute pathname or a pathname relative to the current pathname.  The parent directory of any present directory is specified by "..".

For example:

cd ..\..\runtime\ports

The keyword cd of the directory navigation command is optional.  Specifying just the pathname of the directory without the keyword cd, also puts the user in the new directory.  It is also possible to specify a relative pathname using "..".

The commands can be invoked by typing

- the command in the current directory

- the complete pathname of the command (i.e.  \statistics\ network\ping) from any directory

- the command with its pathname relative to the current directory (i.e.  ..\..\runtime\maintenance\gethost)

The command names and directory names may be specified partially typing only the part with which it can be unambiguously identified.

For instance:

\administration\wall

\a\wa

both result in the execution of the command wall in the administration directory.

## User Friendly Features

### Help Facility

Help messages can be obtained by typing help at the command prompt.  Typing help followed by a command or directory name present in the current directory gives more details about tile command or directory.  In place of the keyword help, "?" can also be used.

### Commands Buffer

A history of commands is maintained in a command buffer by RAS's craft interface to avoid typing of the same commands repeatedly.  The just invoked command can be recalled by typing Esc k.  Thereafter stack of prior commands can be recalled by typing - and +.   Typing a RETURN after recalling the command, results in execution of the command shown at the prompt.

### Command line editing

This facility enables the user to just print a previous command and edit the command line t so as to bring about a new command.

Typing Esc key enters the command editing mode.  The following table gives the list of commands available in the command editing mode.

| Command | Function |
| --- | --- |
| ESC | Enter into command editing mode |
| k | recall the previous command |
| - | go to previous command |
| + | goto next command |
| j | goto next command |
| h | move cursor one character to the right |
| l | move cursor one character to the left |
| i | insert text at the current cursor position |
| a | append text next to the current cursor position |
| A | append text at the end of the line |
| x | delete a character at the current cursor position |
| X | delete a character at the previous cursor position |
| r | replace a character at the current cursor position |
| R | Enter replace mode |
| O | Position cursor at beginning of command line |
| $ | Position cursor to end of command line |

## BandWagon Command Definition

The BandWagon RAS is a full fledge routing system.  Extreme care has been taken to keep to a minimum the number of steps required to configure the BandWagon.  Only essential LAN routing parameters need to be set.  The BandWagon operating system defines the rest of the routing parameters dynamically.  The BandWagon provides a set of administrative tools designed to allow the system administrator total control on how, who, when, and whether or not specific users can access the remote access LAN attached to the BandWagon.

The BandWagon also provides different levels of security, that varies from a total open  to a highly secured access server.  It is necessary for the system administrator to become familiar with the different security options provided by Leemah Datacom that are supported by the BandWagon as well as security options provided by the IP standards that are also supported in the BandWagon.

F Note: Leemah	Need to know name of documents that describe Leemah's security options supported by the BandWagon.  We can reference the literature here, or if text is provided I can include it in an appendix.

#### Command Tree

Figure 2.0 Illustrates the command tree structure of the BandWagon.  The following section will define the different commands available in the BandWagon command interface.  It is necessary for the user to have a good understanding on LAN protocols as they relate to routers and bridges.

The command definitions will provide a brief description of the command and its associated parameters, it will show the description provided by the "Help" facility provided in the BandWagon command line interface, and will show an example of the command when applicable.

The main command line, or the root path provides the option to select from three main functions, namely: statistics, administration, and configuration.

Some of the commands in the command tree are for engineering used only.  These commands will be indicated as "Technical Support Only".  Their  Help" definition will be shown and no attempt will be made to explain the parameters as shown in the example window.  These commands are tools to be used by Leemah's technical support personnel.  You may be instructed to use some of these commands by Leemah's customer support center in the effort to help you resolve your problem.  These commands will be shown in this guide as a reference only.

Figure 3 Command Line Tree

# Chapter 2.  \statistics path

The statistics section provides system statistics and tools needed to perform network analysis and troubleshooting functions.  The statistics path is divided into the \ras and the \network paths.

## \statistics\ras

This path provides RAS operating system related statistics.  It contains the following additional options:

- kmsg

- uname

- ps

- stack

- sar

- date

### \statistics\ras\kmesg

This command displays session messages captured by the BandWagon RAS.  The system administrator can use this command to review important messages that were displayed during a working session.  The display example below shows also the display with the -p option to display any panic messages in the buffer.

Figure 4 - \statistics\ras\kmesg

->> ? kmesg

KMESG                                                     KMESG

SYNOPSIS

kmesg        [-p]

DESCRIPTION

Show system error diagnostic messages collected

OPTIONS

-p     Show any previous panic message stored in the NVRAM

BandWagon:\statistics\ras>

kmesg, uname, ps, stack, sar, date

->> kmesg -p

There is no PANIC message stored in the NVRAM

Figure 5 - BandWagon example:  \statistics\ras\kmsg

->> kmesg

####################################

####################################

####### ##########  ####### ########

###### # ########  # #####   #######

##### #  #######  #  #### #  #######

#### ##  ######  ##  ### ##  #######

###  #####  ###  ## ###  ##

#######  ####  ####  # ####  # #####

#######        ####   #####   ######

####################################

####################################

------------------------------------

LeeMah DataCom  Security Corporation

------------------------------------

HCL Hewlett-Packard Limited, India

SecureRAS 3200 Boot Loader B.0.90

FLASH Memory checksum(00006657) is bad

AMD  LAN 970  Unit 0 V1.1  IRQ =  9     IOADDR = 0x340    amd0

Ethernet Address is 0.64.246.184.32.100

/etc/ras/ras121b.mem .~.,.,.,.,.,.,.,.,.,.,.,.,.,.,.,.,.,.,.,.,.,.,.,.,.,.,.,.,.

/etc/ras/asio.rom .,.,.,.,.,.,.,.,.,.,.,.,.,.,*

/etc/ras/pri.bin .,.,.,.,.,.,.,.,.,.,.,.,.,*

Copyright HCL Hewlett-Packard Limited, India

under license to LeeMah DataCom Security Corporation

LeeMah 3200 BandWagon D.1.21.B

NVRAM: size = 768 K  Maximum number of users configurable = 4522

COMM Serial   Unit 0 V1.0  IRQ =  3     IOADDR = 0x3f8    CO

ASIO Serial   Unit 0 V1.0  IRQ = 10   MEM = 0x80000000    A1 -A8

SYNC          Unit 0 V1.0  IRQ = 11     IOADDR = 0x200    S1

AMD  LAN 970  Unit 0 V1.1  IRQ =  9     IOADDR = 0x340    amd0

Ethernet Address is 00:40:f6:b8:20:64

16:30:22 WARNING: Audit Trail Database is corrupted. Clearing all the entries

16:30:22 WARNING: Clearing Audit Trail configuration

Jan 15 16:30:29 1996 BandWagon 3200 booted

16:30:29 WARNING: WinRAS not configured - WinRAS daemon not started

F Note: Leemah	Need to know total number of messages that the system holds or any other related quantitative information related to # messages.  Also how does the system treats a buffer full condition?.  Also what is the significance of panic messages?, how many are in the system?, can HCLA provide a list and meaning of all panic messages?  This can be very important and all system messages should be defined in an appendix for reference.

### \statistics\ras\uname

This is the place for the administrator to find information about the RAS operating system software and hardware model versions.

Figure 6 - BandWagon definition - \statistics\ras\uname

->> ? uname

UNAME                                                                      UNAME

SYNOPSIS

uname

DESCRIPTION

Show details of this BandWagon installation

System Name    always BandWagon

Node Name      name of this RAS as set in the configuration

Version        Version of the RAS Software running

Model Number   Hardware Model Number of this system, currently 3200

OPTIONS

none

Figure 7 - BandWagon example: \statistics\ras\uname

->> uname

BandWagon  BandWagon  D.1.21.B  3200

### \statistics\ras\ps

The \statistics\ras\ps directory displays the operating system process status table.  It shows all the processes running in the BandWagon and relevant information associated to their current operating state.  This is an engineering support command and you may familiarize yourself with the feel and look of the screen since you may be asked to display this screen by LeeMah's technical support personnel.  The figure below shows a typical display of the ps command.

Figure 8 - BandWagon Definition - \statistics\ras\ps

BandWagon:\statistics\ras>

kmesg, uname, ps, stack, sar, date

->> ? ps

PS                                                                            PS

SYNOPSIS

ps

DESCRIPTION

Show the list of running processes, with the following details:

PID        The process-id of a process

USER       Access code of the process owner

STATE      The state of the process

PRIO       Process priority (Lower number means higher priority)

WCHAN      Event for which the process is sleeping

TTY        Controlling terminal for the process ("?" if none)

TIME       The cumulative execution time for the process

SIGNAL     The pending signal(s) for the process

FLAGS      The flags associated with the process

COMMAND    The command name

OPTIONS

none

Additional explanation:

- PID - Process Identification number

- State - Process current state - Possible values: current, free, ready, sleep, suspend, semawait, delay

- PRIO - Priority - Possible Values:  ???????, the lower the number the higher the priority

- WCHAN - Indicates the address at which the process is sleeping, or ignoring signals, awaiting a condition to become true (:Leemah: notice the definition on the help is refers to it as "event" instead of address, should the help description be changed?)

- SIGNAL - Any signal pending to be received by this process

- Flags - is 0 if the process is sleeping ignoring signals, and 1 if it is sleeping receiving signals.

Figure 9 - BandWagon example: \statistics\ras\ps

BandWagon:\statistics\ras>

kmesg, uname, ps, stack, sar, date

->> ps

PID  USER    STATE   PRIO   WCHAN   TTY   TIME SIGNAL FLAGS COMMAND

0    -      sleep      0   1413bc   ?    0:03      0   0   init

1    -      sleep     20   3dc4f0   ?    0:00      0   1   STREAMS

2    -      sleep     20   141ac8   ?    0:00      0   0   watchdog

4    -      delay     20        0   ?    0:00      0   0   persd

5    -      delay     20        0   ?    0:00      0   0   rwhod

6    -      sleep     20   59d898   ?    0:00      0   1   eventProcessor

7    -      delay     20        0   ?    0:00       0   0   fragmentReassembl

8    -      sleep     20   59d8c8   ?    0:00       0   1   retransmitter

9    -      delay     20        0   ?    0:00       0   0   spoofWatchdog

10    -      sleep     20   17bbdc   ?    0:00       0   1   iptd

11    -      delay     20        0   ?    0:00       0   0   iptmond

12    -      sleep     20   17bbdc   ?    0:00       0   1   radmd

13    -      sleep     20   17bbdc   ?    0:00       0   1   snmpd

16    -      sleep     20   17bbdc   ?    0:00       0   1   radiusd

17    -      sleep     20   17bbdc   ?    0:00       0   1   cpassd

18    -      sleep     20   1626dc   ?    0:00       0   1   printd

20  root     current   20        0   CO   0:00       0   0   admin

21    -      sleep     20   3dc624   A1   0:00       0   1   dialin: A1

22    -      sleep     20   3dc63c   A2   0:00       0   1   dialin: A2

23    -      sleep     20   3dc654   A3   0:00       0   1   dialin: A3

24    -      sleep     20   3dc66c   A4   0:00       0   1   dialin: A4

25    -      sleep     20   3dc684   A5   0:00       0   1   dialin: A5

26    -      sleep     20   3dc69c   A6   0:00       0   1   dialin: A6

27    -      sleep     20   3dc6b4   A7   0:00       0   1   dialin: A7

28    -      sleep     20   514ab4   ?    0:00       0   1   dialin: A8

29    -      sleep     20   3dc6e4   S1   0:00       0   1   dialin: S1

### \statistics\ras\stack

The information provided in this tab is intended for engineering technical support only.  It indicates the call stack associated to the process identification (pid), and relevant arguments such as return address, local variables and stack frame address, specified in the command.  The pid is the number obtained form the ps tab.

Figure 10 - BandWagon's Definition - \statistics\ras\stack

->> ? stack

STACK                                                                      STACK

SYNOPSIS

stack        <pid>

DESCRIPTION

Show the stack of the indicated process

OPTIONS

none

Figure 11 - BandWagon's example: \statistics\ras\stack

->> stack 20

db_stacktrace (0 17bd64 1fbe58) ret: do_stack+8b, l: 41cc64 1b87fc, bp: 41cc64

do_stack+8b (1fbe58 4 0) ret: linemgr+d9, l: 14 1fbe58, bp: 41cc7c

linemgr+d9 (1fbe58 0 41cf0b) ret: run_admin+8e, l: 2d4 1f7fb0, bp: 41cc9c

run_admin+8e (1f7fb0 0 1f7fb0) ret: run_login+496, l: 2d42e7 41cf0b, bp: 41ccb4

run_login+496 (0 1f7fb0 0) ret: login_main+70, l: 313233 34353637, bp: 41cf74

login_main+70 (1 41cfec 1f7fb0) ret: mgr_main+e1, l: 0 0, bp: 41cfbc

mgr_main+e1 (1 41cfec 1f7fb0) ret: userret, l: 0 0, bp: 41cfdc

### \statistics\ras\sas

This command displays system related information.  The information presented by this command is mostly for engineering technical support.  For a system administrator the "CPU" utilization and the "port"  activity/traffic reports may provide information as to how the BandWagon resources are being utilized and if system loading is affecting the performance of the BandWagon.

At any time press "Alt+c" to terminate report display.

Figure 12 - BandWagon's definition - \statistics\ras\sar

->> ? sar

SAR                                                               SAR

SYNOPSIS

sar  [interval [count]] [-<u|i|m|t<ports>|-I<ifp> [-p] [-a]>]

DESCRIPTION

Show the system activity report.  The report can be repeated and

controlled  by  the  options  described below.  The report can be

terminated by typing INTR (normally ^C).

OPTIONS

interval Repeat the report every <interval> seconds.  The default

value is to report every 1 second.

count    Repeat the report for <count> times for every <interval>.

The default value is to report forever.

-u         Report CPU utilization.  This is the default

-i         Report interrupt activity

-m         Report STREAMS activity

-t<ports>  Report serial port activity on <ports>

<ports> can be specified as A4 or A1,A5-A8

-I<ifp>    Report interface traffic (bytes) on interface <ifp>

-p         Display interface traffic in packets instead of bytes

-a         Display traffic in all the active ports in the interface

Figure 13 - BandWagon example: \statistics\ras\sar -u

->> sar -u

19:08:30  %usr  %sys  %idle   clock     serial       amd

19:08:31     0     0    100      0    0    0    0     0

19:08:32     0     0    100      0    0    0    0     0

19:08:33     0     0    100      0    0    0    0     0

19:08:34     0     0    100      0    0    0    0     0

19:08:35     0     0    100      0    0    0    0     0

19:08:36     0     0    100      0    0    0    0     0

Average      0     0    100      0    0    0    0     0

Figure 14 - BandWagon example: \statistics\ras\sar -i

->> sar -i

Interrupt  0   1   2   3   4   5   6   7   8   9  10  11  12  13  14  15

19:11:35   0   0   0   0   0   0   0   0   0   0   0   0   0   0   0   0

19:11:36   0   0   0   0   0   0   0   0   0   0   0   0   0   0   0   0

19:11:37   0   0   0   0   0   0   0   0   0   0   0   0   0   0   0   0

19:11:38   0   0   0   0   0   0   0   0   0   0   0   0   0   0   0   0

19:11:39   0   0   0   0   0   0   0   0   0   0   0   0   0   0   0   0

19:11:40   0   0   0   0   0   0   0   0   0   0   0   0   0   0   0   0

19:11:41   0   0   0   0   0   0   0   0   0   0   0   0   0   0   0   0

Average    0   0   0   0   0   0   0   0   0   0   0   0   0   0   0   0

Figure 15 - BandWagon example: \statistics\ras\sar -m

->> sar -m

Modules   STR TCPIP ASYNC SYNC ISDN PTP  LAN  IPX  NB   LLC OTHERS

19:15:27    0    0    0    0    0    0    0    0    0    0    0

19:15:28    0    0    0    0    0    0    0    0    0    0    0

19:15:29    0    0    0    0    0    0    0    0    0    0    0

19:15:30    0    0    0    0    0    0    0    0    0    0    0

19:15:31    0    0    0    0    0    0    0    0    0    0    0

19:15:32    0    0    0    0    0    0    0    0    0    0    0

19:15:33    0    0    0    0    0    0    0    0    0    0    0

19:15:34    0    0    0    0    0    0    0    0    0    0    0

Average     0    0    0    0    0    0    0    0    0    0    0

Figure 16 - BandWagon example: \statistics\ras\sar -t <port>

->> sar -t A1

Traffic        Activity A1

xmt    rcv   over  error  lost

19:17:50        0      0     0      0     0

19:17:51        0      0     0      0     0

19:17:52        0      0     0      0     0

19:17:53        0      0     0      0     0

19:17:54        0      0     0      0     0

19:17:55        0      0     0      0     0

19:17:56        0      0     0      0     0

19:17:57        0      0     0      0     0

19:17:58        0      0     0      0     0

19:17:59        0      0     0      0     0

Average         0      0     0      0     0

Figure 17 - BandWagon example: \statistics\ras\sar -I

BandWagon:\statistics\ras>

kmesg, uname, ps, stack, sar, date

->> sar 5 7 -I ppp0 p

ppp0 (bytes)        IP                      IPX                   NetBeui

XMT         RCV         XMT         RCV         XMT         RCV

22:27:07                                                        12          15

22:27:12                                                         1           0

22:27:17                                                         0           0

22:27:22                                                         0           0

22:27:27                                                         0          37

22:27:32                                                         0           0

22:27:37                                                         1           1

Average                                                          2           7

### \statistics\ras\date

This commands displays the full date set in the  database.

Figure 18 - BandWagon's definition - \statistics\ras\date

->> ? date

DATE                                       DATE

SYNOPSIS

date

DESCRIPTION

Show date and time

OPTIONS

none

Figure 19 - BandWagon example: \statistics\ras\date

->> date

Jan 20 20:11:34 1996

## \statistics\network

In this section the BandWagon keeps all the network related statistics.  These include routing, protocol, hosts, and in addition some troubleshooting tools such as "ping" and "ipxping".

### \statistics\network\ping

The ping command is a very useful troubleshooting tool.  It is used to determine if a given host can be reached across the elements of an internet.  If the BandWagon is part of a network consisting of multiple gateways and routers, the ping command can be used to determine if specific hosts or workstations can be reached.  The ping command uses the hostname or the IP address of the destination to forward tests packets with an echo request.  If the echo packet reaches the intended host, the host will respond with the echo packet.  If the packet does not reach the destination, the administrator can ping intermediate gateways and routers to determine in which specific path of the internet the packet is being dropped.  To see the list of all the hosts and IP addresses contained in the boot server's host file, use the command \configuration\runtime\maintenance\gethost

Figure 20 - BandWagon's definition: \statistics\network\ping

->> ? ping

PING                                                                      PING

SYNOPSIS

ping         [-s] <hostname> [packetsize [count]]

DESCRIPTION

ping utilizes the ICMP protocol's ECHO_REQUEST datagram to elicit an

ICMP ECHO_RESPONSE from the specified host or network gateway.  If host

responds, ping will print "host is alive" on the standard output and

exit.  Otherwise after 20 seconds, it will write "no answer from host"

OPTIONS

-s  ping sends one datagram per second, and prints one line of output for

every RESPONSE that it receives.  It computes round trip times and

packet loss statistics; it displays a summary upon termination by

typing INTR (normally ^C)

packetsize   The default packet size is 64 bytes, or a size can be given

with the packetsize option.  The packet size should be between 56 and

4096.

count  valid only with the '-s' option. ping normally sends REQUESTs

until terminated by INTR.  If count is specified, ping will

send only that number of REQUESTs.  The packetsize must be given

before the count is specified

In the example below the ping command is used to test the network path to the boot host 192.10.1.1.  The command is set to send 5 packets of 1000 bytes each.  The translation of the RAS hostname is done by the RAS automatically using a copy of /etc/hosts file downloaded from the boot server during boot time.

Figure 21 - BandWagon example: \statistics\network\ping

ping, ipxping, hosts, ruptime, ip, ipx, netbeui, netstat, arp, route, acs

->> ping -s 192.10.1.1 1000 5

PING 192.10.1.1: 1000 data bytes

1000 bytes from 192.10.1.1: icmp_seq=0. time=0. ms

1000 bytes from 192.10.1.1: icmp_seq=1. time=0. ms

1000 bytes from 192.10.1.1: icmp_seq=2. time=0. ms

1000 bytes from 192.10.1.1: icmp_seq=3. time=50. ms

----192.10.1.1 PING Statistics----

10 packets transmitted, 10 packets received, 0% packet loss

round-trip (ms)  min/avg/max = 0/10/50

### \statistics\network\ipxping

This command performs the same function as the ping command.  It ipxping uses a node address of the form net.mac to forward packet to the intended host.  Note that NetWare file servers and client do not support this function.  It is, therefore, useful to test the internet between two BandWagon servers.

Figure 22 - BandWagon definition: \statistics\network\ipxping

->> ? ipxping

IPXPING                                                               IPXPING

SYNOPSIS

ipxping      [-s] <hostaddr> [packetsize [count]]

DESCRIPTION

ipxping utilizes the XNS ECHO protocol's ECHO_REQUEST datagram to

elicit an XNS ECHO_RESPONSE from the specified host or network

gateway supporting the XNS ECHO protocol.  If host responds, ipxping

will print "host is alive" on the standard output and exit. Otherwise

after 20 seconds, it will write "no answer from host"

OPTIONS

-s       ipxping sends one datagram per second, and prints one line of

output for every RESPONSE that it receives.  It computes

round trip times and packet loss statistics; it displays a

summary upon termination by typing INTR (normally ^C).

packetsize The default packet size is 64 bytes, or a size can be given

with the packetsize option.  The packet size should be

between 12 and 546.

count    valid only with the '-s' option. ipxping normally sends

REQUESTs until terminated by INTR.  If count is specified,

ipxping will send only that number of REQUESTs. The

packetsize must be given before the count is specified

Figure 23 - BandWagon example: \statistics\network\ipxping

F Note: Leemah	need to get a display sample of an IPXPING command.

### \statistics\network\hosts

This commands displays the hosts database.

Figure 24 - BandWagon definition: \statistics\network\hosts

->> ? hosts

HOSTS                                                                   HOSTS

SYNOPSIS

hosts        [ <hostname(s)> | <internet address(es)> ]

DESCRIPTION

Show details about hosts

OPTIONS

<host>    perform this command for <host> only. <host> may be an

official host name or an alias or an internet address.

If no <host> is given, this command is performed on all

known hosts

Figure 25 - BandWagon example: \statistics\network\hosts

F Note: Leemah	need to get an actual display sample of the hosts command.

### \statistics\network\ruptime

The ruptime command displays a summary of all rwhod packets received from other hosts on the LAN.  If there are no servers transmitting rwhod packets the display will be empty.

Figure 26 - BandWagon definition - \statistics\network\ruptime

->> ? ruptime

RUPTIME                                                              RUPTIME

SYNOPSIS

ruptime

DESCRIPTION

Show status of machines on the local network.  It uses the rwhod

protocol to collect this information.

OPTIONS

none

Figure 27 - BandWagon example: \statistics\network\ruptime

F Note: Leemah	need to get an actual display sample of the ruptime command.  When I displayed the command it showed an empty screen.  Also need to know what "ruptime" stands for.

### \statistics\network\ip

This command provides statistics related to the ip protocol processing.  The information in this command is intended for engineering technical support.

Figure 28 - BandWagon's definition: \statistics\network\ip

->> ? ip

IP                                                                        IP

SYNOPSIS

ip           [-s<protocol>] [-<a|i|r>]* [-c<channel>] [-n]

[-t<interval>]

DESCRIPTION

Show the TCP/IP protocol suite statistics

OPTIONS

-a             Show channel statistics

-s <protocol>  Show per-protocol statistics

<protocol> can be ip, udp, tcp, route or icmp

-r             Show the routing tables

-i             Show the state of network interfaces for this suite

-c <channel>   Show detailed channel statistics for TCP channel

-n             Show network addresses as numbers

-t <interval>  Update information every <interval> seconds

Figure 29 - BandWagon example: \statistics\network\ip i, -a, -n

->> ip -i

Name  Mtu   Network         Address        Ipkts Ierrs    Opkts Oerrs  Coll

amd0 1500   192.10.1.0      192.10.1.2        22     0       41     1     0

lo0  1500   127.0.0.1       127.0.0.1          0     0        0     0     0

Figure 30 - BandWagon Example: \statistics\network\ip -s

->> ip -s

Active Internet connections

Proto Recv-Q Send-Q Local Address       Foreign Address    (state)       channel

tcp        0      0 *.login             *.*                 LISTEN(  5)  0

tcp        0      0 *.1657              *.*                 LISTEN(  5)  1

udp        0      0 *.echo              *.*                 IDLE

udp        0      0 *.discard           *.*                 IDLE

udp        0      0 *.who               *.*                 IDLE

udp        0      0 *.route             *.*                 IDLE

udp        0      0 *.1025              *.*                 IDLE

udp        0      0 *.snmp              *.*                 IDLE

udp        0      0 *.1645              *.*                 IDLE

Figure 31 - BandWagon example: \statistics\network\ip -s tcp

->> ip -s tcp

tcp:

connections initiated: 0

connections accepted: 0

connections established: 0

connections dropped: 0

embryonic connections dropped: 0

total packets sent: 0

data packets retransmitted: 0

control (RST) packets sent: 0

total packets received: 0

packets received in sequence: 0

bytes received in sequence: 0

packets received with cksum errs: 0

packets received with bad offset: 0

packets received too short: 0

duplicate-only packets received: 0

duplicate-only bytes received: 0

packets with some duplicate data: 0

dup. bytes in part-dup. packets: 0

packets received out of order: 0

bytes received out of order: 0

packets with data after window: 0

Figure 32 - BandWagon example: \statistics\network\ip -c 1

->> ip -c 1

tcp        0      0 *.1657              *.*                 LISTEN(  5)  1

ISS No      = 00000000  IRS No      = 00000000  Flags = 0  Event = 0

Send UNA    = 00000000  Send NXT    = 00000000  Send MAX    = 00000000

Send Urgent = 00000000  Send Window =        0  Rcv Win Adv =        0

Recv NXT    = 00000000  Recv Adv    = 00000000  Recv Space  =     1024

Wind Update seq No. = 00000000   Wind Update ack No. = 00000000

Figure 33 - BandWagon example: \statistics\network\ip -s route

->> ip -sroute

Route Statistics

bad routing redirects                    : 0

dynamically created routes               : 0

new gateways due to redirects            : 0

destinations found unreachable           : 3

uses of a wildcard route                 : 0

Figure 34 - BandWagon example: \statistics\network\ip -r

->> ip -r

Routing tables

Destination      Gateway            Flags     Refs     Use  Metric  Interface

127.0.0.1        127.0.0.1          UH          0         0     0   lo0

192.10.1.2       127.0.0.1          UH          0         0     0   lo0

192.10.1.0       192.10.1.2         U           1        30     0   amd0

Figure 35 - BandWagon example: \statistics\network\ip -s icmp

->> ip -sicmp

icmp:

0 calls to icmp_error

0 errors not generated 'cuz old message was icmp

0 errors detected when sending icmp msgs

Output histogram:

total messages: 26

echo: 26

0 messages with bad code fields

0 messages < minimum length

0 bad checksums

0 messages with bad length

Input histogram:

total messages: 16

echo reply: 16

0 message responses generated

Important parameters:

- ULP - Upper Layer Protocol UDP or TCP

- Flags - U means up, H means Host, G means gateway, D means dynamic

- MTU - Maximum transmission unit, indicates the maximum size in bytes that the interface can transmit in a single frame

- Ipkts - Input Packets

- Ierrs - Input errors

- Coll - Collisions encountered on Ethernet interface

- Canput - Indicates whether given module is willing to accept more packets.  Typically canput failures indicates overload of a slow link.

F Note: Leemah	it will be nice to have definitions for more of the parameters in the screens for ip and ipx.

### \statistics\network\ipx

Vital IPX protocol statistics can be obtained with this command.  This section is intended to be used for engineering technical support.  Good knowledge of the IPX protocol is required to fully understand the information provided by this command.

Figure 36 - BandWagon's definition: \statistics\network\ipx

->> ? ipx

IPX                                                                          IPX

SYNOPSIS

ipx          [-s<protocol>] [-<a|i|r|u>]* [-n] [-t<interval>]

DESCRIPTION

Show the SPX/IPX protocol suite statistics

OPTIONS

-a             Show channel statistics

-s <protocol>  Show per-protocol statistics

<protocol> can be ipx, rip or sap

-r             Show the routing tables

-i             Show the state of network interfaces for this suite

-u             Show the IP tunneling statistics

-n             Show network addresses as numbers

-t <interval>  Update information every <interval> seconds

Figure 37 - BandWagon example: \statistics\network\ipx -a

->> ipx

Active SPX/IPX connections

Proto Recv-Q Send-Q Local Address       Foreign Address    (state)

ipx        0      0 *.echo              *.*                 BOUND

ipx        0      0 *.rip               *.*                 BOUND

ipx        0      0 *.sap               *.*                 BOUND

There are two possible states BOUND and UNBOUD.  It is unlikely to see a channel in UNBOUD state, since it will take a very short time for the RAS process opens an IPX socket and binds it.

Figure 38 - BandWagon example: \statistics\network\ipx -s rip

->> ipx -s rip

Route Statistics

bad routing redirects                    : 0

dynamically created routes               : 0

new gateways due to redirects            : 0

destinations found unreachable           : 0

uses of a wildcard route                 : 0

Figure 39 - BandWagon example: \statistics\network\ipx -s sap

->> ipx -ssap

service  serverName             server Address        hops age       interface

619      LEEMAH                  *He+A@@@@@D...PJ

30043c14.01          1    60          amd0

30043c14.01          1    60          amd0

4         80,402                 *He+A@@@@@D...PJ

632                              30043c14.01          1    60          amd0

The servername is displayed by the RAS exactly as it is received from File Servers which are broadcasting SAP information.  Some file servers include some control characters in the  address field that have no meaning when displayed.  The number "619, 4, 632" indicates the type of service offered by the server.  The server Address indicates the IPX (net.mac) address of the File Server.  The hops indicate the distance of the Server (in hops) from the RAS, the age indicates the time (in seconds) since the SAP entry was re-validated by the server.  The interface indicates which of the RAS's various LAN/PPP interfaces this SAP entry was learned from.

Figure 40 - BandWagon example: \statistics\network\ipx -r

->> ipx -r

Routing tables

Destination      Gateway            Flags     Refs     Use  Metric  Interface

a.01             a.01               UH         0         0   0:01      lo0

a1.40f6b82064    a.01               UH         0         0   0:01      lo0

a1.*             a1.40f6b82064      U          2        39   0:01      amd0

The refs section indicates the number of active channels which are using this routing entry.  The 'use' indicates the total number of times this routing entry was used to route a packet.  Metric is in X:XX format, indicating the <number of hops>:<time ticks>.  IPX RIP attempts to measure the time of a hop also (IP RIP uses only count of number of hops).  But mostly,  all IPX routes send the <time ticks> as one more than the <number of hops>.  The information is obtained from other nodes and displayed by the RAS, which does not modify this information anyway.

Figure 41 - BandWagon example: \statistics\network\ipx -i

->> ipx -i

Name  Mtu   Network       Address            Ipkts Ierrs    Opkts Oerrs  Coll

amd0 1500   a1.*          a1.40f6b82064         37     0       39     1     0

lo0  1500   a.01          a.01                   0     0        0     0     0

Figure 42 - BandWagon example: \statistics\network\ipx -u

->> ipx -u

IP tunneling Statistics

Input UDP packets received               : 0

Output UDP packets sent                  : 0

Input IPX packets received               : 0

Output IPX packets sent                  : 0

Canput failures on the IPX side          : 0

unroutable packets                       : 0

Total number of packets dropped          : 0

Figure 43 - BandWagon  example: \statistics\network\ipx -n

->> ipx -n

Active SPX/IPX connections

Proto Recv-Q Send-Q Local Address       Foreign Address    (state)

ipx        0      0 *.echo              *.*                 BOUND

ipx        0      0 *.rip               *.*                 BOUND

ipx        0      0 *.sap               *.*                 BOUND

### \statistics\network\netbeui

NetBeui is the most commonly used protocol in LAN Manager and Windows implementations.  It supports the NetBIOS 1.0 interface standards and NetBIOS 3.0 interface standards for use on OS/2 workstaions and servers.  This command provides a means to obtain operational statistics specific to NetBeui protocol implementation.  This section is intended to be used by engineering technical support.

Figure 44 - BandWagon's definition: \statistics\network\netbeui

->> ? netbeui

NETBEUI                                                              NETBEUI

SYNOPSIS

netbeui      [-g] [-i] [-s] [-I <interface>] [-t <interval>]

DESCRIPTION

Show the NetBeui protocol stack statistics

OPTIONS

-g                Show global statistics

-i                Show the state of network interfaces for this suite

-I <interface>    Show interface specific statistics

<interface> can be any interface returned by option

[-i]

-s                Show the Active Sessions

-t <interval>     Update information every <interval> seconds

Figure 45 - BandWagon example: \statistics\network\netbeui -g

->> netbeui -g

NetBeui stack global statistics:

Total number of stacks cloned            : 3

Stack clones presently in use            : 1

Figure 46 - BandWagon example: \statistics\network\netbeui -I

->> netbeui -i

Name  Mtu   Destination                        Ipkts Ierrs    Opkts Oerrs  Coll

amd0 1500   Local LAN Interface                  253     0        5     1     0

Figure 47 -BandWagon example: \statistics\network\netbeui -I <interface>

->> netbeui -I amd0

NetBeui stack statistics for amd0 interface :

Number of name packets received          : 86

Number of llc2 packets dropped           : 0

Number of llc2 packets passed            : 0

Number of llc2 packets received          : 0

Number of name packets sent              : 0

Number of session packets received       : 0

Number of session packets sent           : 0

Number of NBF hangups                    : 0

Number of unknown name packet            : 0

Number of sessions routing               : 0

Number of packets disc nbf MUX           : 0

Number of packets disc unkwn msg type    : 0

Number of S_Q and N_Q packs rec          : 0

Number of A_N_Q and A_G_N_Q packs rec    : 54

Number of S_R and N_R packs rec          : 0

Number of A_N_R packs rec                : 0

Number of DGRM packs rec                 : 32

Number of D_BCAST packs rec              : 0

Number of unsupported packs rec          : 0

Number of UI packs rec                   : 86

Number of I packs rec                    : 0

Number of XID packs rec                  : 0

Number of TEST packs rec                 : 0

Number of unknown packs rec by llc       : 0

Number of query retries rec              : 0

Number of queries                        : 0

Number of FIND.NAME queries              : 0

Number of response retries               : 0

Number of FIND.NAME responses            : 0

Number of S_C                            : 0

Number of S_I                            : 0

Number of S_A                            : 0

Number of S_E                            : 0

Number of D_F_M                          : 0

Number of D_O_L                          : 0

Number of Other session packets          : 0

Number of D_F_M dropped                  : 0

Parameter definition:

Number of name packets received             :  self-explanatory

Number of llc2 packets dropped              :  self-explanatory

Number of llc2 packets passed               :  self-explanatory

Number of llc2 packets received             :  self-explanatory

Number of name packets sent                 :  self-explanatory

Number of session packets received          :  self-explanatory

Number of session packets sent        :  self-explanatory

Number of NBF hangups                 :  self-explanatory

Number of unknown name packet         :  self-explanatory

Number of sessions routing            :  self-explanatory

Number of packets disc nbf MUX        :  self-explanatory

Number of packets disc unkwn msg type :  self-explanatory

Number of S Q and N_Q packs rec          S_Q stands for Session Query

N_Q stands for Name Query

Number of A_N_Q and A_G_N_Q packs rec    A_N_Q stands for Add Name Query

A_G_N_Q stands for Add

GroupName Query

Number of S_R and N_R packs rec          S_R stands for Session Response

N_R stands for Name Response

Number of A_N_R packs rec                A_N_R stands for Add Name

Response

Number of DGRM packs rec                 DGRM stands for Datagrams

Number of D_BCAST packs rec              D_BCAST stands for Datagram

Broadcast

Number of unsupported packs rec          self-explanatory

Number of UI packs rec                   UI stands for Unnumbered

Information

Number of I packs rec                    I stands for Information

Number of XID packs rec                  XID stands for

Number of TEST packs rec                 LLC Test packets

Number of unknown packs rec by Ilc       self-explanatory

Number of query retries rec              Number of session query

requests which were retried

Number of queries                        Number of session query

requests received

Number of FIND.NAME queries              self-explanatory

Number of response retries               self-explanatory

Number of FIND.NAME responses            self-explanatory

Number of   S  C                         S  C  stands  for  Session

Confirm

Number of   S  I                         S  I  stands  for  Session

Initialize

Number  of  S  A                         S  A  stands  for  Session

Alive

Number  of  S  E                         S  E  stands  for  Session End

Number  of  D  F M                       D  F M stands for Data First

Middle

Number of D O_L                          D_O_L stands for Data Only Last

Number of Other session packets          self-explanatory

Number of D_F_M dropped                  self-explanatory

Figure 48 - BandWagon example: \statistics\network\netbeui -s

F Note: Leemah	I tried this command with a netbeui remote connection and still did not show any data, just the empty table with the headers.  Need to know why and also a display sample if possible.

### \statistics\network\netstat

The information presented with this command is strictly reserved for LeeMah's technical support personnel for diagnostics.  It will be presented for reference only and not attempt will be made to define the parameters any further.  If you have any questions about any parameter definition in particular please contact LeeMah's Datacomm technical support center.

Figure 49 - BandWagon's definition: \statistics\network\netstat

->> ? netstat

NETSTAT                                                                  NETSTAT

SYNOPSIS

netstat      [-<s|m|h>]* [-I<interface>] [-t<interval>]

DESCRIPTION

Show network and  memory details

OPTIONS

-s             Show spoofing related statistics

-m             Show memory buffer usage statistics

-h             Show malloc heap usage statistics

-I <interface> Show detailed statistics about <interface> -t interval

Figure 50 - BandWagon example: \statistics\network\netstat -s

->> netstat -s

SPOOF  Statistics                        :  IP_RIP    IPX_RIP     SAP

Input packets Received                   :      0        0        0

Output packets sent                      :      1        0        0

Number of Retransmits                    :      0        0        0

Requests received                        :      0        0        0

Triggered Requests received              :      0        0        0

Requests sent                            :      1        0        0

Triggered Requests sent                  :      0        0        0

Responses received                       :      0        0        0

Triggered Responses received             :      0        0        0

Triggered Acknowledgements sent          :      0        0        0

Responses sent                           :      0        0        0

Triggered Responses sent                 :      0        0        0

Triggered Acknowledgements received      :      0        0        0

Figure 51 - BandWagon example: \statistics\network\netstat -m

->> netstat -m

STREAMS usage statistics

class            Use       Total         Max        Fail      Config

stream  :          20          52          20           0         350

queue   :          56         208          60           0         700

mblock  :           5       15001          53           0        4131

dblock  :           5       13207          53           0

16 :           0        4483          12           0         768

48 :           0        1642           5           0         768

64 :           0        2212           6           0         512

128 :           5        1758           8           0         256

256 :           0        1000           3           0         128

512 :           0          11           1           0         120

1024 :           0          31           2           0          96

1536 :           0        2070          44           0         108

timeout :           2      194151           6           0         200

bufcall :           0           0           0           0          20

session :           1           8           1           0         150

udpchan :           5           6           5           0          16

tcpchan :           2           2           2           0         150

tcpcb   :           2           2           2           0         225

icmpchan:           0           0           0           0          32

ipxchan :           2           2           2           0          64

malloc  :       16860      168072       17744           0      131072

Figure 52 - BandWagon example: \statistics\network\netstat -h

->> netstat -h

Malloc Heap usage statistics

Use       Total         Max        Fail      Config

16860      168088       17744           0      131072

Malloc Heap Free List

Address       Size

0x0020c7cc    0x0000003c

0x002105c8    0x00000084

0x00210698    0x00000068

0x0021072c    0x00000058

Figure 53 - BandWagon example: \statistics\network\netstat -I <interface>

->> netstat -I ppp0

Interface Statistics                          IP        IPX      NetBeui

Packets transmitted             :          0          0        205

Total octets transmitted        :          0          0      16002

Unicast packets transmitted     :          0          0        205

Non-Unicast packets transmitted :          0          0          0

Output packets discarded        :          0          0          0

Packets received                :          0          0        376

Total octets received           :          0          0      36211

Unicast packets received        :          0          0        376

Non-Unicast packets received    :          0          0          0

Input packets discarded         :          0          0          0

Output Errors                   : 0

Collisions                      : 0

Input Errors                    : 0

Received for unknown protocol   : 0

Detailed PPP statistics.

Bad FCS                         : 0

Bad Addresses                   : 0

Bad Controls                    : 0

Detailed LCP Report:

Tx Fcs Size                     : 16

Rx Fcs Size                     : 16

Initial MRU                     : 1500

Local MRU                       : 1500

Remote MRU                      : 1500

Async Map                       : Off

PAP                             : Off

CHAP                            : Off

Magic Number                    : Off

Protocol Field Compression      : Off

Address/Control Field Compr     : Off

Current NBFCP State             : 6

Other Statistics:

total ports open                : 1

In Chars for Sesn 0 on A8       : 39368

Out Chars for Sesn 0 on A8      : 17802

### \statistics\network\arp

Figure 54 - BandWagon's definition: \statistics\network\arp

->> ? arp

ARP                                                                          ARP

SYNOPSIS

arp

DESCRIPTION

Displays the Internet-to-Ethernet address translation table maintained

by the Address Resolution Protocol (ARP). For each entry in the table,

the hostname, its internet address, the corresponding Ethernet address

along with its status information are shown

OPTIONS

none

### \statistics\network\route

This section displays routing information for IP, IPX and NetBeui protocols.

#### \statistics\network\route\ip

Figure 55 - BandWagon's definition: .. \statistics\network\route ip

BandWagon:\statistics\network\route>

->> ? ip

IP                                                                            IP

SYNOPSIS

ip

DESCRIPTION

Show the TCP/IP protocol suite routing table entries

OPTIONS

none

Figure 56 - BandWagon example: .. \statistics\network\route ip

->> ip

Routing tables

Destination      Gateway            Flags     Refs     Use  Metric  Interface

127.0.0.1        127.0.0.1          UH          0         0     0   lo0

192.10.1.2       127.0.0.1          UH          0         0     0   lo0

192.10.1.0       192.10.1.2         U           1         3     0   amd0

#### \statistics\network\route\ipx

Figure 57 - BandWagon's definition: \statistics\network\network ipx

BandWagon:\statistics\network\route>

ip, ipx, netbeui

->> ? ipx

IPX                                                                    IPX

SYNOPSIS

ipx

DESCRIPTION

Show the SPX/IPX protocol suite routing table entries

OPTIONS

none

Figure 58 - BandWagon example: .. \statistics\network\route ipx

\ statistics\ network\ route>

ip, ipx, netbeui

->> ipx

Routing tables

Destination      Gateway      Flags        Refs        Use   Metric  Interface

12340001.01     12340001.01    UH            O           0    0:01      loO

8013e001.40f6b88001

12340001.01    UH            O           0    0:01      loO

8013e001."   8013e001.40f6b88001U            1        ·2384   0:01      amd0

30043c14."   8 013e0 01 . cOa8 3 4 154 3UG   0           0    1:02      amd0

#### \statistics\network\route\netbeui

Figure 59 - BandWagon's definition: .. \statistics\network\route netbeui

BandWagon:\statistics\network\route>

ip, ipx, netbeui

->> ? netbeui

NETBEUI                                                               NETBEUI

SYNOPSIS

netbeui

DESCRIPTION

Print the NetBEUI routing table.

OPTIONS

None

Figure 60 - BandWagon example: .. \statistics\network\route netbeui

F Note: Leemah	I tried this command with a netbeui remote connection and still did not show any data, just the empty table with the headers.  Need to know why and also a display sample if possible.

### \statistics\network\acs

ACS means Asynchronous Communications Service.  This is the RAS equivalent of Novel’s NetWare NACS for NASI compatibility.  Any attempts made by LAN users to access ACS services across the LAN by means of the BandWagon will be validated by using the entries of ACS users.

Figure 61 - BandWagon's definition: \statistics\network\acs

->> ? acs

ACS                                                                          ACS

SYNOPSIS

acs          [ -p | -u | -r | -d <specific_portname> ]

DESCRIPTION

Command to show information about ACS port status and the users.

OPTIONS

-p                     Show all ACS ports and their status.

-u                     Show all users of ACS.

-r                     Show all retransmit list elements of ACS.

-d <specific_portname> Closes the connection established on the

specified ACS port.

Figure 62 - BandWagon example: acs -p

Figure 63 - BandWagon example: \statistics\network\acs -u

Figure 64 - BandWagon example: \statistics\network\acs -r

Figure 65 - BandWagon example: \statistics\network\acs -d

F Note: Leemah  	Need to get display examples of the "acs" -p, -u, -r, -d displays for inclusion into document.

# Chapter 3.  \administration path

This section contains the commands required to perform day-to-day network operations.  These functions include:

- monitoring port activity

- control port status

- broadcast system messages

- monitor system process

- verify path usage (a path can be considered a group user profile)

- verify dynamic routes

- add and delete static routes

- set system date

- detach active processes

- perform system reboot

## \administration\activity

Figure 66 - BandWagon's definition: \administration\activity

BandWagon:\administration>

activity, wall, write, reboot, detach, date, ifconfig, reset, route, path, who

->> ? activity

ACTIVITY                                                              ACTIVITY

SYNOPSIS

activity     [ <portname-portrange> ]

DESCRIPTION

Show the activity on the indicated port(s)

It shows the port type, and other relevant information. Additionally,

information about the sessions opened from the indicated port(s) will

be displayed

OPTIONS

<portname>             show activity of <portname>

<portrange>            show activity of these ports e.g. A1,A4-A7

Displays the activity on the selected port.  It also shows the port type and other important information such as opened sessions by the port.

Figure 67 - BandWagon example: \administration\ activity

->> activity

Port Type        Modem Signals  Traffic ->   Transmit    Receive Ovrun Errs Lost

CO  dialin      dtr dcd          dsr         9226        208     0    0    0

A1  dialin      dtr     rts                     0          0     0    0    0

A2  dialin      dtr     rts                     0          0     0    0    0

A3  dialin      dtr     rts                     0          0     0    0    0

A4  dialin      dtr     rts                     0          0     0    0    0

A5  dialin      dtr     rts                     0          0     0    0    0

A6  dialin      dtr     rts                     0          0     0    0    0

A7  dialin      dtr     rts                     0          0     0    0    0

A8  dialin      dtr dcd rts cts             34657      34181     0    0    0

Sessions from port A8  are:

0       (  ppp )

S1  dialin                                      0          0     0    0    0

The Transmit, receive, errors, and lost  are in number of bytes, overrun indicates the number of occurrences of this condition.  It indicates that information is being received faster that it can be transmitted to the LAN.

## \administration\wall

Sends messages to ports that are open, and do not have a link protocol running.  This basically means, ports that do not have any user login.

Figure 68 - BandWagon's definition \administration\wall

->> ? wall

WALL                                                                    WALL

SYNOPSIS

wall         <message>

DESCRIPTION

Send message to all ports

OPTIONS

none

Figure 69 - BandWagon example: \administration\wall

->> wall "Hello"

"Hello"

## \administration\write

Write sends a message to a specific port.  The port has to be in the idle state.  The RAS will send back to the console an error message if a message is sent to a port in a connected state.

Figure 70 - BandWagon's definition: \administration\write

->> ? write

WRITE                                                          WRITE

SYNOPSIS

write        <portname[-portname]>   <message>

DESCRIPTION

Write <message> on the indicated port(s)

OPTIONS

<portname>             write <message> on <portname>

<portname-portname>    write <message> on these ports e.g. A4-A7

## \administration\reboot

The reboot command performs a system reset.  The system prompts the operator to confirm the reboot command before rebooting.  The reboot will display the same sequences as it does when it powers up.  This command is used to make configurations changes done in the permanent database to take effect after the reboot sequence.  Any changes made in the  configuration section will be lost after the reboot sequence is completed.

## \administration\detach

Detach disconnects a port unconditionally.  The session number is obtained from the activity display, see figure 67 above.

Figure 71 - BandWagon's definition: \activity\detach

->> ? detach

DETACH                                                                  DETACH

SYNOPSIS

detach       [ session number ]

DESCRIPTION

Close the session indicated by <session_number> disconnecting it from

the host.

OPTIONS

none

Figure 72 - BandWagon example: \administration\detach

->> detach 0

Se

11:35:58 ppd: shut down ppp0 to nbssion 0 from port A8  is being closed

BandWagon:\administration>

activity, wall, write, reboot, detach, date, ifconfig, reset, route, path, who

## \administration\date

Displays and sets the date and time of day.  The same command is used to display and set the system date and time as shown in the figure below.

Figure 73 - BandWagon's definition: \administration\date

->>? date

DATE                                                                      DATE

SYNOPSIS

date         [ mmddhhmmss[yy] ]

DESCRIPTION

The current date and time are printed, if no argument is given.

Otherwise, the current date is set to the argument given.

OPTIONS

The first mm is the month number (01-12);

dd is the day number in the month (01-31);

hh is the hour number (24-hour system, 00-23);

the second mm is the minute number (00-59);

ss is the seconds (00-59);

yy is the last 2 digits of the year number and is optional.

The current year is the default if no year is mentioned.

A year between 93 and 99 is in the 20th century, i.e. 1993-1999

A year between 00 and 92 is in the 21st century, i.e. 2000-2092

For example:  date 1008004500

sets the date to Oct 8, 12:45:00 AM.

After changing the date and time, it will display the new date.

## \administration\ifconfig

Figure 74 - BandWagon's definition: \administration\ifconfig

->> ? ifconfig

IFCONFIG                                                              IFCONFIG

SYNOPSIS

ifconfig     [-n] [<interface> [<prot>:down]]

DESCRIPTION

Show / Modify the interface configuration

OPTIONS

-n             Show network addresses as numbers

<interface>    Show / Modify <interface> only

<prot>:down    Bring down the interface for protocol <prot>

<prot> can be ip, ipx or nb

This option is available only to administrator

Figure 75 - BandWagon example: \administration\ifconfig

->> ifconfig

ppp0  HCL Hewlett-Packard PPP  to nb

NetBeui: flags=451 <UP,RUNNING,NetBeui_MULTICAST,PTP>

amd0  AMD 970 LAN Controller

IP: flags=c3 <UP,RUNNING,BROADCAST,ARP>

inet 192.10.1.2 netmask 255.255.255.0 destination 192.10.1.0

NetBeui: flags=441 <UP,RUNNING,NetBeui_MULTICAST>

lo0  HCL Hewlett-Packard Software Loopback

IP: flags=49 <UP,RUNNING,LOOPBACK>

inet 127.0.0.1 netmask 255.0.0.0 destination 127.0.0.1

In the figure above ifconfig is used to display the logical port connection display.  The  logical port ppp0 is connected to path nb using protocol ppp point-to point (PTP), to transport the NetBeui protocol.  The NetBeui protocol is also being transported at the LAN interface port.  The flags value is immediately defined after the flag hex value.  The possible states are UP, DOWN, RUNNING, BROADCAST, NetBeui_MULTICAST, LOOPBACK, PTP ( or Point-to-Point), PTP-GATEWAY, ARP indicating that the device needs ARP resolution, QUIT indicating that RIP information is not to be sent,  SPOOFED indicating that the interface is spoofing RIP (and SAP, if IPX interface) traffic.

Figure 76 - BandWagon example: \administration\ifconfig -n <port>

->> ifconfig -n ppp0 nb:down

11:37:54 ppd: shut down ppp0 to nbppp0: nb shut down

In the example above ifconfig is used to bring down the logical port connection to path nb

## \administration\reset

Figure 77 - BandWagon's definition: \administration\reset

->> ? reset

RESET                                                          RESET

SYNOPSIS

reset     <portname>

DESCRIPTION

This command resets the given port and stops the ongoing activities

over the port.  The port is reinitialized and becomes ready for use.

OPTIONS

<portname>             reset the <portname>

Figure 78 - BandWagon example:\administration\reset -n

->> reset A8

11:39:17 ppd: shut down ppp0 to nbreset: port reset successfully

BandWagon:\administration>

activity, wall, write, reboot, detach, date, ifconfig, reset, route, path, who

The reset -n command is used to down or disconnect a physical port.  Useful if modem fails during  modemchat initialization.

## \administration\route\ip

The \administration\route\ip path provides the options to add and delete ip routes.  These commands can be used to define static route tables if the routing information protocol (RIP) is turned off.  This can be important in applications involving heavy traffic and to reduce the processing overhead impose by the constant updates of the routing tables imposed by the RIP protocol.  It is possible to add static route even when (RIP) is turned off.  However, this is not recommended.  Careful network design is required to ensure proper communications among routers and gateways across the WAN.  It is necessary to formulate a sound network communications plan in order to be able to properly use static routing in a network.  It is necessary to first gain complete familiarity with the functional environment before attempting to implement a static routed network.  Using first dynamic routing can provide a generate good data that can reveal network usage patterns not known to network administrators.  Once these are known it is save to implement a plan that uses the static route approach.

The show command displays the ip dynamic routing table entries

Figure 79 - BandWagon's definition: \administration\route\ip\show

->> ? show

SHOW                                                                      SHOW

SYNOPSIS

show         [-n]

DESCRIPTION

Show the routing table entries

OPTIONS

-n        Show network addresses as numbers

BandWagon:\administration\route\ip>

show, add, delete, flush, trigger

Figure 80 - BandWagon example: \administration\route\ip\show

->> show

Routing tables

Destination      Gateway            Flags     Refs     Use  Metric  Interface

127.0.0.1        127.0.0.1          UH          0         0     0   lo0

192.10.1.2       127.0.0.1          UH          0         0     0   lo0

192.10.1.0       192.10.1.2         U           1         3     0   amd0

In the example below the \administration\ip\add command is used to add a static route.  The \administration\ip\show command is used to verify the route path is correct.  The \administration\ip\delete command is used to delete the entry.

Figure 81 - BandWagon's definition: \administrtion\route\ip\add

->> ? add

ADD                                                                       ADD

SYNOPSIS

add          [-n] [net|host] <dstn> <gw> [metric]

DESCRIPTION

Add a new routing table entry.

OPTIONS

<dstn>    is the destination host or network to which the route leads;

<gw>      is the gateway to which packets should be addressed;

[metric]  indicates the number of hops to the destination. If no metric

is specified, route assumes a value of 0

[net|host] indicates whether the route is to a network or to a host. If

unspecified, routes to a host are  distinguished from those

to a network by checking the address associated with <dstn>

-n        Show network addresses as numbers

Figure 82 - BandWagon example: \administration\route\ip\add

->> add host 192.10.4.1 192.10.1.1 2

add: host 192.10.4.1: gateway 192.10.1.1, flags 7done

BandWagon:\administration\route\ip>

show, add, delete, flush, trigger

->> show

Routing tables

Destination      Gateway            Flags     Refs     Use  Metric  Interface

127.0.0.1        127.0.0.1          UH          0         0     0   lo0

192.10.1.2       127.0.0.1          UH          0         0     0   lo0

192.10.4.1       192.10.1.1         UGH         0         0     2   amd0

192.10.1.0       192.10.1.2         U           1         3     0   amd0

Figure 83 - BandWagon's definition: \administration\route\ip\delete

->> ? delete

DELETE                                                             DELETE

SYNOPSIS

delete       [-n] <dstn> <gw>

DESCRIPTION

Delete an existing routing table entry.

OPTIONS

<dstn>     is the destination host or network to which the route leads;

<gw>       is the gateway to which packets should be addressed;

-n        Show network addresses as numbers

Figure 84 - BandWagon example: \administration\route\ip\delete

BandWagon:\administration\route\ip>

show, add, delete, flush, trigger

->> delete 192.10.1.0 192.10.1.2

delete: net 192.10.1.0: gateway 192.10.1.2, flags 1done

BandWagon:\administration\route\ip>

show, add, delete, flush, trigger

->> show

Routing tables

Destination    Gateway      Flags     Refs     Use  Metric  Interface

127.0.0.1      127.0.0.1      UH          0         0     0   lo0

192.10.1.2     127.0.0.1      UH          0         0     0   lo0

Figure 85 - BandWagon's definition: \administration\route\ip\flush

->> ? flush

FLUSH                                                                      FLUSH

SYNOPSIS

flush        [-n]

DESCRIPTION

Flush the routing table entries

OPTIONS

-n        Show network addresses as numbers

BandWagon:\administration\route\ip>

show, add, delete, flush, trigger

->> flush

Flushing routing tables:

BandWagon:\administration\route\ip>

show, add, delete, flush, trigger

F Note: Leemah  	I do not know what this command is really for.

Figure 86 - BandWagon's definition: \administration\route\ip\trigger

->> ? trigger

TRIGGER                                                               TRIGGER

SYNOPSIS

trigger

DESCRIPTION

Trigger routing request packets on interfaces with triggered spoofing.

This command SHOULD be given after a flush, if routes are to be

relearned on interfaces with triggered spoofing.

OPTIONS

none

BandWagon:\administration\route\ip>

show, add, delete, flush, trigger

## \administration\route\ipx

The \administration\route\ipx commands provide the same functions as the \administration\route\ip section described before.  They include:

- ..\show - To display the ipx route tables

- ..\add - To add static routes to IPX

- ..\delete - To delete static routes from the IPX routing tables

Note: Leemah  	I really do not know what this flush command is for.

- ..\flush - To flush ...packets

- \trigger - To generate trigger IPX requests with spoofing

IPX uses a proprietary version of RIP with Novel’s implementation of Xerox Sequenced Packet Protocol (SPP), called sequenced packet exchange (SPX).  An important difference between XNS RIP and Novel RIP is that IPX incorporates an additional cost metric for estimating the delay to a particular destination.  IPX also uses the service advertising protocol (SAP) to locate resources of a specific type like NetBeui.  IPX is a connections protocol that allows each packet to be transmitted immediately as it seeks the route with the least amount of delay to its destination.  A higher layer protocol is required to keep track of packet sequencing and delivery.  IPX uses SPX for this purpose.

The figure below shows a typical IPX routing table.

Figure 87 - BandWagon example: \administration\route\ipx\show

BandWagon:\administration\route\ipx>

show, add, delete, flush, trigger

->> show

Routing tables

Destination      Gateway            Flags     Refs     Use   Metric   Interface

12340001.01     12340001.01          UH        0        0    0:01      lo0

8013e001.40f6b88001

12340001.01          UH        0        0    0:01      lo0

8013e001.*    8013e001.40f6b88001    U         1      2041   0:01      amd0

30043c14.*    8013e001.c0a8341543    UG        0        0    1:02      amd0

## \administration\route\netbeui

The \administration\route\netbeui commands provide the same functions as the \administration\route\ip section described before.  They include:

- ..\show - To display the ipx route tables

- ..\add - To add static routes to NetBeui

- ..\delete - To delete static routes from the NetBeui routing tables

Note: Leemah  	I really do not know what this flush command is for.

- ..\flush - To flush ...packets

Figure 88 - BandWagon example: \administration\route\netbeui\show

BandWagon:\administration\route\netbeui>

show, add, delete, flush

->> show

name                       available at       flags

[HOME...........^A]          ppp0               SRCH_BRD_ALLOWED

[HOME...........^^]          ppp0               SRCH_BRD_ALLOWED

[LAPTOP.........^A]          ppp0               SRCH_BRD_ALLOWED

[LAPTOP.........^C]          ppp0               SRCH_BRD_ALLOWED

[LAPTOP..........]          ppp0               SRCH_BRD_ALLOWED

BandWagon:\administration\route\netbeui

There is an important difference in the use of the commands in this section for NetBeui.  The figure below shows the BandWagon's definition of the ..\add command.

Figure 89 - BandWagon's definition: \administration\route\netbeui\add

->> ? add

ADD                                                                        ADD

SYNOPSIS

add             <name> <interface>

DESCRIPTION

Add routes to the NetBEUI routing table.

OPTIONS

<name>     is the destination name to which this route leads

The name can have maximum 16 characters and spaces should

be entered as '.' and other non printable characters should

entered in the CNTRL-Alpha format

<interface>

is the interface of the type 'ppp0' on which the above name

can be reached

The list of valid interfaces can be got from:

'\statistics\network\netbeui -i' command

If spaces occur in the name they will be misinterpreted by the command processor as an argument separator between the <name> and <interface> options.  Therefore, spaces must be entered as '.'.  Other non printable characters need to be in the Control-Alpha format or as ^<character> such as ^A for ASCII value 0x01, ^B for 0x02 and so on.  This applies to the ..\delete command as well.

Note: Leemah  	This definition is very fuzzy.  I tried to delete NetBeui route entries with little success.  The command interpreter could not understand the name as I tried to input as it was shown in figure 87.  How is the delete and add command input for the route showing "Home" in figure 87 above?  I also tried adding a Name and the interface and it adds the interface ppp0 in the middle of the Name as shown [HOME^A......ppp0^A]          ppp0               SRCH_BRD_ALLOWED.  I typed "add HOME ppp0".  This does not resembles anything shown in figure 87 in the format and display.  This is very confusing.

### \administration\path\start

Figure 90 - BandWagon's definition: \administration\path start

BandWagon:\administration\path>

start, stop, list, addport, delport

->> ? start

START                                                                      START

SYNOPSIS

start        [-v <level>] <pathname>

DESCRIPTION

Start a path, indicated by the pathname. A pathname is a string of

upto 15 characters

OPTIONS

-v <level>  start path in Verbose mode, with a level of <level>

Higher levels indicate more diagnostic output

BandWagon:\administration\path>

start, stop, list, addport, delport

Figure 91 - BandWagon example: \administration\path start

->> start -v 100 nb

starting path nb in Verbose mode

fire_up_ppd: path nb is already running

BandWagon:\administration\path>

start, stop, list, addport, delport

Note: Leemah  	I tried this command with units 0, 1, 10, and 100 and it did no make any difference.  Every time the path started it displayed the same message.  What is the point then for this command?  What can it be used for?  I tried stopping the path but then I dialed again with the client and the connection was made.  Shouldn't this command stop the path until the administrator start the path again?  Other wise the start command do not make any sense since it does not perform any useful purpose

### \administration\path\stop

Figure 92 - BandWagon's definition: \administration\path stop

->> ? stop

STOP                                                                      STOP

SYNOPSIS

stop         <pathname> [<pid>]

DESCRIPTION

Stop the specified active path. A pathname is a string of upto 15

characters. The pid if any will be used to stop only the instance

of the path with a matching pid.

OPTIONS

none

If the -v value greater than zero is used, the modem and login chat will be splayed at the administration console.  If a value greater than 10 is used, the PPP negotiation process will be displayed as well.

Figure 93 - BandWagon example: \administration\path stop

start, stop, list, addport, delport

->> stop nb 37

12:11:17 ppd: shut down ppp0 to nbopping path nb, ppp0

BandWagon:\administration\path>

start, stop, list, addport, delport

### \administration\path\list

Figure 94 - BandWagon's definition: \administration\path  list

->> ? list

LIST                                                                        LIST

SYNOPSIS

list

DESCRIPTION

List the active paths

OPTIONS

none

BandWagon:\administration\path>

start, stop, list, addport, delport

Figure 95 - BandWagon example: \administration\path list

start, stop, list, addport, delport

->> list

nb.pid = 40, ppp0 - A8

BandWagon:\administration\path>

start, stop, list, addport, delport

The number after the path name indicates the process identification (pid) running the instance at the port specified.  It is expected that multiple instances of the same path may run (different clients with the same path assigned).  In such cases, the pid and the interface/port number can be used to distinguish between the various paths.

\administration\path\addport

Figure 96 - BandWagon's definition: \administration\path addport

BandWagon:\administration\path>

start, stop, list, addport, delport

->> ? addport

ADDPORT                                                               ADDPORT

SYNOPSIS

addport  [-v <level>]  <pathname>  <port> [<pid>]

DESCRIPTION

Add a port to an active path.

This can be used to add additional ports to an active path from its

configured port pool, in case a higher throughput is desired.

Once a port has been added by addport, the Bandwidth on Demand

functionality won't delete it from the path unless it is deleted through

the delport command. This should be used only with the paths with MLPPP

protocol

OPTIONS     none

### \administration\path\delport

Figure 97- BandWagon's definition: \administration\path delport

BandWagon:\administration\path>

start, stop, list, addport, delport

->> ? delport

DELPORT                                                               DELPORT

SYNOPSIS

delport      <pathname>  <port> [<pid>]

DESCRIPTION

Delete a port from an active path.

Once a port has been deleted by delport, the Bandwidth on Demand

functionality won't add it to the path unless it is added through

the addport command. This should be used only with the paths with MLPPP

protocol

OPTIONS

### \administration\who

Figure 98 - BandWagon's definition: \administaion\who

BandWagon:\administration>

activity, wall, write, reboot, detach, date, ifconfig, reset, route, path, who

->> ? who

WHO                                                                       WHO

SYNOPSIS

who

DESCRIPTION

The who command lists the users currently logged into the system.

It gives the user's full name, the port in which he is logged in and

the date & time at which he logged in.

OPTIONS

none

Figure 99 - BandWagon example: \administration\who

->> who

root                    CO      Jan 30, 1996 00:08:49

tony                    A8      Jan 30, 1996 00:20:19

BandWagon:\administration>

activity, wall, write, reboot, detach, date, ifconfig, reset, route, path, who

->>

# Chapter 4.  \Configuration directory

The configuration path is were most of the setup and day-to-day work is done.  The configuration path is divided as follows:

- Runtime - This path provide the necessary parameters to configure ports, network protocols, RAS related parameters, and database maintenance to upload and download the hosts file.

- Permanent - Provides the same functions as runtime.  It also includes the service and WinRAS to configure the WinRAS remote configuration program.  WinRAS is a Windows™ based applications used to manage the BandWagon RAS using the Ethernet LAN port.

- Path - The path section is used to define the class of service to be provided to the users of the RAS.  Different paths can be designed to provide the necessary classes of services that best suites a particular user or groups of users.

- Security - The security directory provides the necessary tools to define individual user's accounts.  Different security schemes can be provided from simple password access to the highly secure proprietary methods developed by LeeMah Authentication Server using LeeMah Security API.

- RAS - This configures RAS related operational parameters.

- WINRAS - Provides the parameters required to allow the RAS to communicate with the RAS.

Notice that the Permanent and Runtime directories provide similar functions.  The difference is that any changes made using the runtime directory take effect immediately.  The runtime changes are stored in the volatile memory and, therefore, are only valid until the RAS is restarted.  In contrast changes made using the Permanent directory are store in non-volatile memory and only take effect after the RAS performs a reboot.  The Runtime directory allows the operator to make functional changes an verify that they work properly.

## \configuration\runtime

Figure 100 - BandWagon's definition:  \configuration\runtime

->> ? runtime

RUNTIME                                                                  RUNTIME

SYNOPSIS

runtime

DESCRIPTION

Change to runtime directory and allow access to the commands there.

All changes made here are immediately entered into the operational

database and take effect immediately, and will be lost on the next boot

OPTIONS

none

### BandWagon:\\configuration\runtime\maintenance

This directory is used to upload and download the RAS configuration database.  The command allows the operator  to specify a host and a \path\file.ext  name .  If the operator does not specify a host the RAS will use the default boot server. .  Notice that all changes made using the runtime directory will be lost after a RAS reboot.  This directory could be used to verify that changes to the RAS will work properly before making permanent changes in  non-volatile memory.

#### \configuration\runtime\maintenance\upload

Figure 101 - BandWagon's definition: \configuration\runtime\maintenance\upload

BandWagon:\configuration\runtime\maintenance>

upload, gethost

->> ? upload

UPLOAD                                                                    UPLOAD

SYNOPSIS

upload       [host:]<filename>

DESCRIPTION

Upload configuration information to the specified file on the

indicated host

OPTIONS

[host:]       Indicates the remote host to upload. If it is not

specified, the boot server will be selected

<filename>    Filename for uploading

upload, gethost

Figure 102 - BandWagon example:.-.\configuration\runtime\maintenance\upload

BandWagon:\configuration\runtime\maintenance>

upload, gethost

->> upload 192.10.1.1:\etc\config.ras

Uploading Configuration  to file \etc\config.ras on host 192.10.1.1

BandWagon:\configuration\runtime\maintenance>

upload, gethost

#### \configuration\runtime\maintenance\gethost

The gethost commands retrieves a configuration database from the specified host.  The \path\filename.ext is given with the location path specified.  The host can be specified by name or by its IP address.  If no host is specified the RAS will try to use the boot host server.

Figure 103 - BandWagon's definition:.-.\configuration\runtime\maintenance\gethost

BandWagon:\configuration\runtime\maintenance>

upload, gethost

->> ? gethost

GETHOST                                                                GETHOST

SYNOPSIS

gethost      [host:][filename]

DESCRIPTION

Download the host database from the specified file on the indicated host

OPTIONS

[host:]       Indicates the remote host. If it is not specified,

specified, the boot server will be selected

[filename]    Indicates the hosts filename. If it is not specified,

"/etc/hosts" will be chosen

BandWagon:\configuration\runtime\maintenance>

upload, gethost

Figure 104 - BandWagon example: - \configuration\runtime maintenance\gethost

->> gethost 192.10.1.1:\etc\config.ras

Downloading host database \etc\config.ras from host 192.10.1.1

isgQµPkw>-//Bhs'-,U+¤n:-+>mqx%h'`a,U+¤nMDQ{9 j-K%s&f9R\[5tU;>mnx$h%,a,U+

¤n:-+>mqx%h'`a,U+¤n:-+>mqx%h'`a,U+¤n:-+>mqx%h%,a,U+¤n:-+>mqx%h'`an:-

Note: Leemah  	Note these  commands are not working properly, the database did not downloaded properly.

### \configuration\runtime\ports

This directory is were the operator will configure the RAS port parameters.  The changes made to the ports will take effect immediately.  Notice that all changes made using the runtime directory will be lost after a RAS reboot.  This directory could be used to verify that changes to the RAS will work properly before making permanent changes in  non-volatile memory.

#### \configuration\runtime\ports\show

Figure 105 - BandWagon's definition: \configuration\runtime\ports\show

BandWagon:\configuration\runtime\ports>

show, modify, groupmod

->> ? show

SHOW                                                                        SHOW

SYNOPSIS

show         [ <portname|portrange>]

DESCRIPTION

Show the port characteristics. Example portrange is A1,A5-A8

OPTIONS

none

BandWagon:\configuration\runtime\ports>

show, modify, groupmod

Figure 106 - BandWagon example: \configuration\runtime\ports\show

BandWagon:\configuration\runtime\ports>

show, modify, groupmod

->> show

-----------------------------------------------------------------------------

FlowControl

Port   Type        BaudRate  Char  Parity  (Hardware)  (Software)    Modem

size            in/out     in/out

-----------------------------------------------------------------------------

CO    dialin       9600      7     odd     none/none   off/on   NULL_MODEM

A1    dialin       57600     8     none    rts /cts    off/off  NULL_MODEM

A2    dialin       57600     8     none    rts /cts    off/off  NULL_MODEM

A3    dialin       57600     8     none    rts /cts    off/off  NULL_MODEM

A4    dialin       57600     8     none    rts /cts    off/off  NULL_MODEM

A5    dialin       57600     8     none    rts /cts    off/off  NULL_MODEM

A6    dialin       57600     8     none    rts /cts    off/off  NULL_MODEM

A7    dialin       57600     8     none    rts /cts    off/off  NULL_MODEM

A8    dial_in_out  38400     8     none    rts /cts    off/off  NULL_MODEM

BandWagon:\configuration\runtime\ports>

show, modify, groupmod

#### \configuration\runtime\ports\modify

Figure 107 - BandWagon's definition: \configuration\runtime\ports\modify

BandWagon:\configuration\runtime\ports>

show, modify, groupmod

->> ? modify

MODIFY                                                                    MODIFY

SYNOPSIS

modify       [ <portname|portrange>]

DESCRIPTION

Modify the port characteristics. Example portrange is A1,A5-A8

OPTIONS

none

BandWagon:\configuration\runtime\ports>

show, modify, groupmod

Figure 108 - BandWagon example: \configuration\runtime\ports\modify

------------------------------------------------------------------------

->> modify A1

PORTNAME(S) ::: A1

-----------------------------------------------------------------------------

Parameter                                Current Value       New value

-----------------------------------------------------------------------------

Baud Rate                                57600               ?

Help:

Possible Baud Rate values are:

115200, 76800, 57600, 14400, 7200, 38400, 19200, 9600, 4800

3600, 2400, 2000, 1800, 1200, 600, 300, 200, 150, 134.5, 110

75, 50

-----------------------------------------------------------------------------

Parameter                                Current Value       New value

-----------------------------------------------------------------------------

Baud Rate                                57600

Character Size                           8                   ?

Help:

This field can be any of 8, 7, 6, 5

-----------------------------------------------------------------------------

Parameter                                Current Value       New value

-----------------------------------------------------------------------------

Character Size                           8

Parity                                   none                ?

Help:

Possible Parity Values are as follows:

value               meaning

-----               -------

even                EVEN parity

odd                 ODD parity

none                DISABLE parity

mark                parity is MARK

space               parity is SPACE

-----------------------------------------------------------------------------

Parameter                                Current Value       New value

-----------------------------------------------------------------------------

Parity                                   none

No of stop bits                          1                   ?

Help:

This field can be one of 1, 1.5 or 2 only

-----------------------------------------------------------------------------------------

-----------------------------------------------------------------------------

Parameter                                Current Value       New value

-----------------------------------------------------------------------------

No of stop bits                          1

Software output flow control             off                 ?

Help:

This field can have two values only: on or off

-----------------------------------------------------------------------------

Parameter                                Current Value       New value

-----------------------------------------------------------------------------

Software output flow control             off

Software input flow control              off                 ?

Help:

This field can have two values only: on or off

-----------------------------------------------------------------------------

Parameter                                Current Value       New value

-----------------------------------------------------------------------------

Software input flow control              off

Modem Name                               NULL_MODEM          ?

Help:

Input a modem name.  A modem name can be up to 14 chars.

Input NULL_MODEM if no modem is to be connected

-----------------------------------------------------------------------------

Parameter                                Current Value       New value

-----------------------------------------------------------------------------

Modem Name                               NULL_MODEM

Monitor Carrier                          on                  ?

Help:

This field can have two values only: on or off

-----------------------------------------------------------------------------

Parameter                                Current Value       New value

-----------------------------------------------------------------------------

Monitor Carrier                          on

Intr Char  (<^><char> or <\ooo>)         ^C                  ?

Help:

Input any char . Control chars can be given as ^(char) or

as \ooo, where 'o' denotes an octal digit. The octal format is

preferred for control characters and when inputting ? itself

------------------------------------------------------------------------------------------

-----------------------------------------------------------------------------

Parameter                                Current Value       New value

-----------------------------------------------------------------------------

Intr Char  (<^><char> or <\ooo>)         ^C

Stop Character  (<^><char> or <\ooo>)    ^S                  ?

Help:

Input any char . Control chars can be given as ^(char) or

as \ooo, where 'o' denotes an octal digit. The octal format is

preferred for control characters and when inputting ? itself

-----------------------------------------------------------------------------

Parameter                                Current Value       New value

-----------------------------------------------------------------------------

Stop Character  (<^><char> or <\ooo>)    ^S

Start Character  (<^><char> or <\ooo>)   ^Q                  ?

Help:

Input any char . Control chars can be given as ^(char) or

as \ooo, where 'o' denotes an octal digit. The octal format is

preferred for control characters and when inputting ? itself

-----------------------------------------------------------------------------

Parameter                                Current Value       New value

-----------------------------------------------------------------------------

Start Character  (<^><char> or <\ooo>)   ^Q

Input hardware flow control              rts                 ?

Help:

Possible Values Are : none, rts

-----------------------------------------------------------------------------

Parameter                                Current Value       New value

-----------------------------------------------------------------------------

Input hardware flow control              rts

Output hardware flow control             cts                 ?

Help:

Possible Values Are : none, cts

-----------------------------------------------------------------------------

Parameter                                Current Value       New value

-----------------------------------------------------------------------------

Output hardware flow control             cts

Port type                                dialin              Read Only

Session prompt                           ->                  ?

Help:

Prompt can be any sequence of chars upto  10 chars long

------------------------------------------------------------------------------------------

Parameter                                Current Value       New value

-----------------------------------------------------------------------------

Session prompt                           ->

Port IP Address                          0.0.0.0             ?

Help:

The IP address associated with this port.

If the "accept client IP address" option in the global RAS config

is set to "no" and the dialin path has no IP address associated

with it, then the client will he allotted this IP address.

-----------------------------------------------------------------------------

Parameter                                Current Value       New value

-----------------------------------------------------------------------------

Port IP Address                          0.0.0.0

Edit, Abort, Save : s

------------------------------------------------------------------------

#### \configuration\runtime\ports\groupmod

The command groupmod modifies a group of port, i.e. A1-A8.  The parameter options are the same as the modify command.

Figure 109 - BandWagon's definition: \configuration\runtime\ports\groupmod

BandWagon:\configuration\runtime\ports>

show, modify, groupmod

->> ? groupmod

GROUPMOD                                                                GROUPMOD

SYNOPSIS

groupmod     <portname|portgroup>

DESCRIPTION

Modify the characteristics of the Group of ports

The ports are to be of the SAME type. Example portgroup is A1,A5-A8

The "Current Value" field displays the actual parameter value or *,

depending on whether that parameter has the same value or not in each

port of the group

OPTIONS

none

BandWagon:\configuration\runtime\ports>

show, modify, groupmod

### \configuration\runtime\network\ip

This directory provides the necessary options to configure all the routing protocols parameters to properly configure the BandWagon RAS.  Notice that all changes made using the runtime directory will be lost after a RAS reboot.  This directory could be used to verify that changes to the RAS will work properly before making permanent changes in  non-volatile memory.

#### \configuration\runtime\network\ip\show

Figure 110 - BandWagon's definition: \configuration\runtime\network\ip\show

->> ? show

SHOW                                                                        SHOW

SYNOPSIS

show

DESCRIPTION

Show TCP/IP protocol suite parameters

OPTIONS

none

Figure 111 - BandWagon example: \configuration\runtime\network\ip\show

BandWagon:\configuration\runtime\network\ip>

show, modify

->> show

---------------------------------------------------------------------

Parameter                               Current Value

---------------------------------------------------------------------

Trace ARP packets (y/n)                 no

Send RWHOD packets (y/n)                no

Send routed (RIP) packets (y/n)         yes

Perform IP forwarding (y/n)             yes

Ack. TCP pkts immediately (y/n)         no

Use TCP keepalive (y/n)                 yes

Reverse Telnet load balancing (r/l)     linear

#### \configuration\runtime\network\ip\modify

Figure 112 - BandWagon definition: \configuration\runtime\network\ip\modify

BandWagon:\configuration\runtime\network\ip>

show, modify

->> modify

-----------------------------------------------------------------------------

Parameter                               Current Value  New value

-----------------------------------------------------------------------------

Trace ARP packets (y/n)                 no             ?

Help:

The Address Resolution Protocol (ARP) resolves the hostname to address

mappings.  If the ARP trace is ON, then a trace of all ARP packets sent or

received by the server is displayed on the Console.  If the trace is set to

OFF, this information is not displayed .

value               meaning

(y)es               Trace of ARP Packets will be displayed

(n)o                Suppress displaying the trace of ARP packets

-----------------------------------------------------------------------------

Send RWHOD packets (y/n)                no             ?

Help:

This field decides if rwhod packets are to be broadcast on the network

This broadcast is performed every 60 seconds.  It can be turned off to

reduce LAN traffic

value               meaning

(y)es               Rwhod packets are broadcast on the network

(n)o                No broadcasting of rwhod packets is done

-----------------------------------------------------------------------------

Send routed (RIP) packets (y/n)         yes            ?

Help:

This field decides if the RAS should transmit routed (RIP) information

Normally, RIP information should be transmitted only by nodes acting as a

gateway

value               meaning

(y)es               RAS will transmit RIP information

(n)o                RAS will NOT transmit RIP information

-----------------------------------------------------------------------------

Perform IP forwarding (y/n)             yes            ?

Help:

This field decides if the RAS can act as a gateway between two networks.

Normally this field is set to YES, since the RAS acts as a router. However

this field can be set to NO for diagnostic or other purposes

value               meaning

(y)es               Forward IP packets between networks

(n)o                No Forwarding of IP packets

-----------------------------------------------------------------------------

Ack. TCP pkts immediately (y/n)         no             ?

Help:

The RAS generates acknowledgment packets in response to incoming TCP

data. These ACK packets can be delayed for performance reasons to reduce

network traffic. This field controls whether TCP acknowledgment should

be delayed or immediate.

value               meaning

(y)es               Acknowledgment to TCP packets is IMMEDIATE

(n)o                DELAY acknowledgment to TCP packets

-----------------------------------------------------------------------------

Use TCP keepalive (y/n)                 yes            ?

Help:

TCP can perform a keepalive function by sending TCP keepalive messages

to the other end.  This ensures that TCP channels will be closed when

the peer node aborts.  This field decides whether the RAS should generate

TCP keepalive messages for its TCP channels

value               meaning

(y)es               Perform TCP keepalive function

(n)o                Do not perform TCP keepalive function

-----------------------------------------------------------------------------

Reverse Telnet load balancing (r/l)     linear         ?

Help:

Reverse Telnet is useful in sharing devices like printers, modems etc.

This field indicates which scheme is used to perform load balancing

among the shared devices.

value               meaning

r                   Load balancing is round robin

l                   Load balancing is Linear

### \configuration\runtime\network\ipx

This directory will allow  the operator to view and modify all the IPX related parameters.  Notice that all changes made using the runtime directory will be lost after a RAS reboot.  This directory could be used to verify that changes to the RAS will work properly before making permanent changes in  non-volatile memory.

#### \configuration\runtime\network\ipx\show

Figure 113 - BandWagon example: \configuration\runtime\network\ipx\show

->> show

-----------------------------------------------------------------------------

Parameter                                     Current Value

-----------------------------------------------------------------------------

Internal Network address(in hex)              0x0

Local LAN address (in hex)                    0x0

Ethernet type(Type II/Raw/802.3/Snap)         802.3

Send ipx rip broadcasts(send/quiet)           send

Forward ipx packets(yes/no)                   yes

BandWagon:\configuration\runtime\network\ipx>

show, modify

#### \configuration\runtime\network\ipx\modify

Figure 114 - BandWagon example: \configuration\runtime\network\ipx\modify

->> modify

-----------------------------------------------------------------------------

Parameter                                Current Value  New value

-----------------------------------------------------------------------------

Send ipx rip broadcasts(send/quiet)      send           ?

Help:

whether this machine should send IPX RIP broadcasts or not

value               meaning

(S)end              Send periodic IPX RIP broadcasts across network interfaces

(Q)iuet             Do NOT send any IPX RIP broadcasts at all. Be silent.

Send ipx rip broadcasts(send/quiet)      send

Forward ipx packets(yes/no)              yes            ?

Help:

Whether this machine should forward IPX from one network to another

Note that if this parameter is set to "no", this machine will

CEASE TO FUNCTION AS A ROUTER AT ALL. However for diagnostic reasons

you may want this set this parameter to "no" temporarily

value               meaning

-----               -------

(Y)es               forward IPX packets across networks

(N)o                Do NOT forward IPX packets from one network to another

Forward ipx packets(yes/no)              yes

BandWagon:\configuration\runtime\network\ipx>

show, modify

### \configuration\runtime\network\snmp

In this directory the operator sets the SNMP parameters that identifies the RAS to the SNMP management entity. Notice that all changes made using the runtime directory will be lost after a RAS reboot.  This directory could be used to verify that changes to the RAS will work properly before making permanent changes in  non-volatile memory.

#### \configuration\runtime\network\snmp\show

Figure 115 - BandWagon example: \configuration\runtime\network\snmp\show

BandWagon:\configuration\runtime\network\snmp>

show, modify

->> show

-----------------------------------------------------------------------------

Parameter                               Current Value

-----------------------------------------------------------------------------

Location of the RAS (max 31 chars)

Person in charge of RAS (max 31 chars)

Authentification traps (Enable/Disable) disabled

Trap destinations (host_id:community)

Community (Community:permissions)       public:RO

BandWagon:\configuration\runtime\network\snmp>

show, modify

#### \configuration\runtime\network\snmp\modify

The section below  shows an example of the \configuration\runtime\network\snmp\modify command.

->> modify

------------------------------------------------------------------------

Parameter                               Current Value  New value

------------------------------------------------------------------------

Location of the RAS (max 31 chars)                     ?

Help:

This field identifies the physical location of the RAS.  The name of

the location can be upto 31 characters long.  The factory default is

a null string.

------------------------------------------------------------------------------

Person in charge of RAS (max 31 chars)                 ?

Help:

This field identifies the name of the person in charge of the RAS

typically the network manager.  The name can be up to 31 characters.

The factory default is a null string.

------------------------------------------------------------------------------

Authentification traps (Enable/Disable) disabled       ?

Help:

This field controls the enabling/disabling of authentification traps.

The factory default disables this field

value               meaning

e                   enable authentication traps

d                   disable authentication traps

------------------------------------------------------------------------------

Trap destinations (host_id:community)                  ?

Help:

This option specifies the  <host_id:community>  pair, which is to be used

to select the hosts to which traps are to be sent.  The host_id can

be a valid host name or an IP address in the dot notation.  The community

string can be a maximum of 15 characters and refers to a valid community

name for the SNMP manager running on the destination host.

The host_id part is compulsory while the community part is optional.

More than one <host_id:community> pair can be specified by separating

them with commas. A maximum of FIVE such pairs can be specified.

value               meaning

+<pair[,pair]>      Add these pair(s)

-<pair[,pair]>      Delete these pair(s)

------------------------------------------------------------------------------

Community (Community:permissions)       public:RO      ?

Help:

This field specifies the  <community:permissions> pairs, which are used

to validate incoming SNMP requests.

The community half of the pair can be a string of maximum 15 characters

length. The permissions half of the pair can be one of RO, RW, WO, NA.

The significance of these permissions options is as follows :

RO  :  A SNMP Manager belonging to the specified community can only

get information and not modify the information itself.

RW  :  A SNMP Manager belonging to the specified community can both

get information as well as modify the information.

WO  :  A SNMP Manager belonging to the specified community can not

get information but can modify the information.

NA  :  A SNMP Manager belonging to the specified community can

neither get information nor can modify the information.

More than one <community:perms> pair can be specified by separating

them with commas. A maximum of FIVE such pairs can be specified.

value               meaning

+<pair[,pair]>      Add these pair(s)

-<pair[,pair]>      Delete these pair(s)

------------------------------------------------------------------------------

The "person in charge of the RAS" field lets you  identify the person in charge of the  BandWagon; normally, the this is the system administrator responsible for the day-to-day operations of the BandWagon.  You may enter up to 31 printable ASCII characters, including letters, numbers, spaces and punctuation.

By default the Disable Authentication Traps is turned disable.  This means that authentication traps are disabled.   When authentication traps are enabled, the administrator can use the SNMP manager will be notified by the BandWagon's SNP manager,  whether an illegal SNMP manager is trying to access the BandWagon's SNMP agent If you don't want to track this, set the Authentication Traps field to enable.

#### \configuration\runtime\network\isdn\digi

The ISDN Basic Rate Interface (BRI) Board is referred to as the "digi board".  This command is used to program the parameters required for proper operation of the ISDN BRI board.

The switch style defines the central office switch used to provision the ISDN service.  It is necessary to know from your ISDN service provider the manufacturer and model of the ISDN switch used by your ISDN service provider.  The ISDN service provider has to assign an SPID number  for each BRI interface.  If you are using the S/T interface and you are planning of adding more ISDN compliant devices, you need to specify the number of logical terminals assigned to the interface.  Each ISDN device will negotiate with the ISDN switch the assignment of an ISDN Terminal Identifier Number.

Figure 116 - BandWagon example: \configuration\runtime\network\isdn\digi\show

BandWagon: \configuration\runtime\network\isdn\digi

show

->>show

-------------------------------------------------------------------------

Parameter                                               Current Value

Line 1: Switch style                                    3

No. of Logical Terminals                             1

SPID 1                                               51058105231

ADDR 1                                               5810523

Line 2: Switch style                                    3

No. of Logical Terminals                             1

SPID 1                                               51058105231

ADDR 1                                               5810523

Line 3: Switch style                                    3

No. of Logical Terminals                             1

SPID 1                                               51058105231

ADDR 1                                               5810523

Line 4: Switch style                                    3

No. of Logical Terminals                             1

SPID 1                                               51058105231

ADDR 1                                               5810523

Note: Leemah  	I do not have a sample of the digi modify command display.  Need to know the CO switch number settings options.  I have no way to verify what they are since I don not have a BRI card.

#### \configuration\runtime\network\isdn\pri>

The user/exchange denotes the protocol side to be used in the ISDN D channel protocol.  The ISDN PRI protocol specify two sides in the User Network Interface.  One side of the protocol is designated as and exchange, the other side is designated as a user.  In most cases, since the PRI card is connected to a Central Office switch, it has to be configured as user.  In situations were the PRI is connected behind a Private Communications Branch Exchanges (PBX) it may have to be set as network side.  Please consult with your telecommunications department to verify if this is the case.

The PRI card supports two PRI interfaces in the card.  Line A, and line B refer to one separate ISDN interface line each.

Figure 117 - BandWagon definition: \configuration\runtime\network\isdn\pri\show

BandWagon:\configuration\runtime\network\isdn\pri>

show

->> show

-----------------------------------------------------------------------------

Parameter                               Current Value

-----------------------------------------------------------------------------

ISDN Configuration: (u)ser/(e)xchange   user

ISDN Line type : line(a)/line(b)        line A

ISDN Line Switch Style:                 0

BandWagon:\configuration\runtime\network\isdn\pri>

show

#### \configuration\runtime\network\netbeui>

NetBeui Protocol is the most commonly used protocol in LAN Manager network implementations.  It supports NetBIOS 3.0 interface standards for use on OS/2 and Windows 3.X and Windows for Workgroups.  NetBIOS provides three primary communication services, local name management, virtual circuit establishment, and datagram communication.  Local names are registered with NetBIOS and provides a means to identify network resources, users, domains, computer names, and relative paths.  An example of a name is a shared hard disk.

Virtual circuits are communication links  between tow names that provide for the reliable delivery of packets.  This implies that the receiving name most acknowledge receipt of transmitted packets.  Datagram communications is a means to transmit packets of data from one NetBIOS name to another without a guarantee of delivery.  The NetBIOS transmitting end assumes that the packets are received by the destination.

NetBeui is not a routable protocol.  It is meant to be used in directly LAN attached workstations.  Because of this design restriction, NetBeui is not tolerant of line delays found in WAN lines.  It is necessary to modify NetBeui protocol default timers for operation over WAN links or dial-up lines.  The default values provided in the BandWagon will work in most instances.  It may be necessary to adjust the timers in cases of very slow or noisy lines.  To alter the NetBeui parameters under windows the Protocol.ini file most be modified.  The [NETBEUI] section may need to have the following parameters added:

[NETBEUI]

NETBIOSTIMEOUT=3000

NETBIOSRETRIES=2

T1=5000

T2=2000

MAXOUT=40

MAXIN=40

- NETBIOSTIMEOUT - is the amount of time the NetBIOS will wait for a replay to its name inquiries in the network.

- NETBIOSRETRIES - is the number of times that NetBIOS will try again after the timeout is completed.

- T1-  this is the time the system will wait for an acknowledgment of the window packets were received by the receiving end-station.

- T2 - This is the time that the system will wait before sending an acknowledgment.

- Ti - This  how often the link layer will check an inactive link to see if it is operational.

- MAXOUT - Is equivalent to a send window size.

- MAXIN - Is equivalent to the receive window size.

The BandWagon has an equivalent set of parameters that can be set to match those set in the workstation.

The parapets shown below defines the NetBeui parameters for the BandWagon's LAN port.  The individual remote access ports are configured in the \cofiguration\path directory.

Figure 118 - BandWagon's definition: \configuratin\runtime\network\netbeui

->> modify

-----------------------------------------------------------------------------

Parameter                               Current Value  New value

-----------------------------------------------------------------------------

T1    (milliseconds)                    5000           ?

Help:

The T1 timer defines the time interval during which the LLC shall

expect to receive an acknowledgment to one or more outstanding

Information frames or an expected response frame to a sent Unnumbered

command PDU. This timer must obey the following rule

T2 <= T1 <= <TI

-----------------------------------------------------------------------------

T2    (milliseconds)                    3000           ?

Help:

The t2 timer defines the time interval during which the LLC shall

wait for either an information frame to become available for

transmission or to accumulate additional Information frame's to

be acknowledged in

a single Receive Ready Frame This timer must obey the following rule

T2 <= T1 <= <TI

-----------------------------------------------------------------------------

Ti    (milliseconds)                    30000          ?

Help:

The TI timer determines how often the LLC checks an inactive link

to see if it is operational.

This timer must obey the following rule

T2 <= T1 <= <TI

-----------------------------------------------------------------------------

Send Window Size (1 - 127)              40             ?

Help:

This field denotes the number of LLC packets to send before

expecting an acknowledgment.

-----------------------------------------------------------------------------

Receive Window Size (1 - 127)           40             ?

Help:

This field denotes the number of LLC packets to receive before

sending an acknowledgment.

-----------------------------------------------------------------------------

Lan Wan Threshold                       1              ?

Help:

Number of Netbeui Name query retries from LAN before allowing them

to be propagated to the WAN links.  This should typically not exceed 3.

### \configuration\runtime\network\acs>

The \configuration\runtime\network\acs directory provides the commands to set the BandWagon's Asynchronous Communications Services.  The information below provides some information concerning the ACS process used in the BandWagon RAS.  This is process includes Dial-Out and Dial-In services fro- and-to the LAN attached to the RAS Ethernet Port.

Dial-Out Services

To accomplish dial-out services, the serial communications that are intended to be passed to the serial port are redirected across the network to a shared modem device.  Dial-out services are accomplished by a few basic methods, which follow:

- Software-only solutions, by which a modem is shared    throughout the network by loading a TSR at a work   station that makes the modem a shared device

- Dedicated modem pooling servers, which consist of a dedicated PC running modem-sharing software

- Devices that connect shared modems attached as a virtual  network device, such as the BandWagon RAS.

Dial-out services require two correctly configured "ends." At one end of the desired communication is the user's workstation.  This end uses a redirection technique to direct signals from the PC's modem port through the network to the communication server.

At the other end is the shared modem connected to the BandWagon. This software receives the modem signals redirected from the user's PC and passes them on to the BandWagon RAS.  The result of this Process is that the shared modem in the BandWagon appears to the user to be directly connected to his own PC's serial port.

When the BandWagon is installed, the shared modem is given a name. When a user loads the sharing software, the software looks across the network for available shared modems in the BandWagon.  When it finds one, a "virtual circuit" is established between the user's workstation and the shared modem in the BandWagon RAS. This virtual circuit remains intact until the user has finished using the shared modem. When the modem is in use, the modem device is unavailable to other users throughout the network.

Redirection Serial Communications

As stated, the serial communications must be redirected across the network to the shared modem in the RAS. Redirection is done by either of two methods: Intl4 and NASI.

Intl4

Intl4, a serial communications redirection standard, operates with BIOS redirection by using Interrupt 14, which is the IBM ROM BIOS serial port interrupt.  A TSR provided, in the RAS Dial-In-Out software, is loaded that intercepts calls made to Interrupt 14, and these calls are redirected across the network into the BandWagon RAS. In turn, because the proper return codes also are made by the redirection TSR, the workstation application sees what it expects to see.

Before you can use the Intl4 interface to redirect communications, The RAS  must load  TSR for the communications application.  Typically, this TSR is configured to look for a specific server name and log in to a RAS communications server with a specific user name and password.

Novel has developed NASI (NetWare Asynchronous Services Interface), another method of serial communications redirection.

Because NASI interfaces directly to the NetWare shell and does not use the computer's BIOS, it performs better at higher communications speed.  Like Intl4, NASI consists of a TSR that must be loaded before the communications application (which must also support NASI) is loaded. After the communications are completed, the NASI TSR may be unloaded.

NASI is operated in of two modes: basic and advanced. In the basic mode of operation, the NASI command interpreter is used to send and receive commands from the NASI TSR. Advanced mode is used more often because it is application-driven (applications are programmed to use it) and--in this mod- -NASI runs within the application, unseen by the user.

Figure 119 - BandWagon example: \configuration\runtime\network\acs\modify

BandWagon:\configuration\runtime\network\acs>

->> modify

-----------------------------------------------------------------------------

Parameter                               Current Value  New value

-----------------------------------------------------------------------------

Prompt string  (max. 11 chars)          NASI           ?

Help:

The acs prompt string can be upto 11 characters long

Prompt string  (max. 11 chars)          NASI

macs port-id  (Hex)                     0x4009         Read Only

Figure 120 - BandWagon example: \configuration\runtime\network\acs\show

->> show

-----------------------------------------------------------------------------

Parameter                               Current Value

-----------------------------------------------------------------------------

Prompt string  (max. 11 chars)          NASI

macs port-id  (Hex)                     0x4009

BandWagon:\configuration\runtime\network\acs>

Notice that the Master Asynchronous Communications Services (ACS) port-ID is read-only. The  ASI.TSR provided with the RAS facilitates the asynchronous services to the users attached to the BandWagon LAN port.  The ASI.TSR also uses this port number to communicate with the RAS.  The ASI.TSR is set to the same default value.  Changing this number in the RAS, without changing the ASI.INI file accordingly, will render the ACS inoperative.  If necessary, this value can be changed using the \configuration\permanent\network\acs directory.  The \configuration\runtime\ is a subset of the \configuration\permanent subdirectory.  All the parameters available in the \configuration\runtime subdirectories can also be set in the permanent configuration database using the equivalent \configuration\permanent command.

### \configuration\runtime\ras

Use this directory to change general parameters related to the RAS object configuration.

Figure 121 - BandWagon's definitions: \configuration\runtime\ras\modify

->> modify

-----------------------------------------------------------------------------

Parameter                               Current Value  New value

-----------------------------------------------------------------------------

Server name (max 15 chars)              BandWagon      ?

Help:

This field specifies the name of the server.  The name should not exceed

15 characters, and should not have control characters or intervening white

spaces.  The default name is BandWagon.

Console error messages level (10-100)   66             ?

Help:

This field indicates the Console error message level.  This parameter

defines the extent to which detailed messages should be displayed on

the console. A larger value for level indicates more detailed messages.

The possible values for this field are between 10 and 100

Console messages buffer level (10-100)  65             ?

Help:

This field indicates the extent to which the console messages should be

buffered for later viewing (with the kmesg command).  A larger value for

level indicates more detailed messages.  The possible values for this

field are between 10 and 100

BandWagon:\configuration\runtime\ras>

show, modify

Figure 122 - BandWagon's example:  \configuration\runtime\ras\show

->> show

-----------------------------------------------------------------------------

Parameter                               Current Value

-----------------------------------------------------------------------------

Server name (max 15 chars)              BandWagon

Console error messages level (10-100)   66

Console messages buffer level (10-100)  65

BandWagon:\configuration\runtime\ras>

show, modify

## \configuration\permanent

This is the directory that needs to be used to make permanent changes to the database of the BandWagon RAS.  The changes made to the BandWagon's database with the commands in this directory will only take effect after a reboot is performed.  This can be done by turning the power off then on again, or by issuing the \administration\reboot command.

The configuration\permanent directory provides the same functions as the \configuration\runtime.  The \configuration\permanent directory, however, contains additional commands not available in the \configuration\runtime these are:

### \configuration\permanent\maintenance

This directory is used to upload and download the RAS configuration database.  The command allows the operator  to specify a host and a \path\file.ext  name .  If the operator does not specify a host the RAS will use the default boot server.  Notice that all changes made using the permanent directory will not take effect until the RAS reboots.

#### \configuration\permanent\maintenance\upload

Figure 123 - BandWagon's definition: \configuration\permanent\maintenance\upload

BandWagon:\configuration\permanent\maintenance\>

upload, gethost

->> ? upload

UPLOAD                                                                    UPLOAD

SYNOPSIS

upload       [host:]<filename>

DESCRIPTION

Upload configuration information to the specified file on the

indicated host

OPTIONS

[host:]       Indicates the remote host to upload. If it is not

specified, the boot server will be selected

<filename>    Filename for uploading

BandWagon:\configuration\runtime\maintenance>

upload, gethost

Figure 124 - BandWagon example:.-.\configuration\permanent\maintenance\upload

BandWagon:\configuration\runtime\permanent>

upload, gethost

->> upload 192.10.1.1:\etc\config.ras

Uploading Configuration  to file \etc\config.ras on host 192.10.1.1

BandWagon:\configuration\runtime\maintenance>

upload, gethost

#### \configuration\permanent\maintenance\download

Downloads the specified configuration file from the specified host server.  Reloads a previously saved  configuration database, using the \configuration\permanent\upload command, from a file in the host path.

Figure 125 - BandWagon's definition: \configuration\permanent\maintenance\download

BandWagon:\configuration\permanent\maintenance>

upload, download, factory

->> ? download

DOWNLOAD                                                                DOWNLOAD

SYNOPSIS

download     [host:][filename]

DESCRIPTION

Download configuration information from the specified file on the

indicated host

OPTIONS

[host:]       Indicates the remote host to download from. If it is not

specified, the boot server will be selected

[filename]    Filename for downloading. If it is not specified,

the System Setup Filename specified in the Boot

Parameters menu will be chosen

BandWagon:\configuration\permanent\maintenance>

upload, download, factory

Figure 126 - BandWagon's example: \configuration\permanent\maintenance\download

BandWagon:\configuration\permanent\maintenance>

upload, download, factory

->> download 192.10.1.1:config.ras

Downloading Configuration  from file config.ras on host 192.10.1.1

add: Pathname 'tacp' already exists

add: Pathname 'nb' already exists

BandWagon:\configuration\permanent\maintenance>

upload, download, factory

#### \configuration\runtime\permanent\factory

Resets database with factory defaults.  All information in the current database is replaced with the factory defaults.  User, port configuration, and path definitions are also erased.

Figure 127 - BandWagon's definition: \configuration\permanent\maintenance\factory

BandWagon:\configuration\permanent\maintenance>

upload, download, factory

->> ? factory

FACTORY                                                                  FACTORY

SYNOPSIS

factory

DESCRIPTION

Restore all settings to factory defaults.

OPTIONS

none

BandWagon:\configuration\permanent\maintenance>

upload, download, factory

Figure 128 - BandWagon example: \configuration\permanent\maintenance\factory

BandWagon:\configuration\permanent\maintenance>

upload, download, factory

->> factory

Do you really wish to restore configuration to factory settings ? (y/n) y

changed all system parameters to factory defaults

BandWagon:\configuration\permanent\maintenance>

upload, download, factory

### \configuration\permanent\ports

This directory is were the operator will configure the RAS port parameters.  The changes made to the BandWagon's database with the commands in this directory will only take effect after a reboot is performed.  This can be done by turning the power off then on again, or by issuing the \administration\reboot command.

#### \configuration\permanent\ports\show

Figure 129 - BandWagon's definition: \configuration\permanent\ports\show

BandWagon:\configuration\permanent\ports>

show, modify, groupmod

->> ? show

SHOW                                                                        SHOW

SYNOPSIS

show         [ <portname|portrange>]

DESCRIPTION

Show the port characteristics. Example portrange is A1,A5-A8

OPTIONS

none

BandWagon:\configuration\permanent\ports>

show, modify, groupmod

Figure 130 - BandWagon example: \configuration\permanent\ports\show

BandWagon:\configuration\permanent\ports>

show, modify, groupmod

->> show

-----------------------------------------------------------------------------

FlowControl

Port   Type        BaudRate  Char  Parity  (Hardware)  (Software)    Modem

size            in/out     in/out

-----------------------------------------------------------------------------

CO    dialin       9600      7     odd     none/none   off/on   NULL_MODEM

A1    dialin       57600     8     none    rts /cts    off/off  NULL_MODEM

A2    dialin       57600     8     none    rts /cts    off/off  NULL_MODEM

A3    dialin       57600     8     none    rts /cts    off/off  NULL_MODEM

A4    dialin       57600     8     none    rts /cts    off/off  NULL_MODEM

A5    dialin       57600     8     none    rts /cts    off/off  NULL_MODEM

A6    dialin       57600     8     none    rts /cts    off/off  NULL_MODEM

A7    dialin       57600     8     none    rts /cts    off/off  NULL_MODEM

A8    dial_in_out  38400     8     none    rts /cts    off/off  NULL_MODEM

#### \configuration\permanent\ports\modify

Figure 131 - BandWagon's definition: \configuration\permanent\ports\modify

BandWagon:\configuration\permanent\ports>

show, modify, groupmod

->> ? modify

MODIFY                                                                    MODIFY

SYNOPSIS

modify       [ <portname|portrange>]

DESCRIPTION

Modify the port characteristics. Example portrange is A1,A5-A8

OPTIONS

none

BandWagon:\configuration\permanent\ports>

show, modify, groupmod

Figure 132 - BandWagon example: \configuration\permanent\ports\modify

-----------------------------------------------------------------------------

->> modify A1

PORTNAME(S) ::: A1

-----------------------------------------------------------------------------

Parameter                                Current Value       New value

-----------------------------------------------------------------------------

Baud Rate                                57600               ?

Help:

Possible Baud Rate values are:

115200, 76800, 57600, 14400, 7200, 38400, 19200, 9600, 4800

3600, 2400, 2000, 1800, 1200, 600, 300, 200, 150, 134.5, 110

75, 50

-----------------------------------------------------------------------------

Parameter                                Current Value       New value

-----------------------------------------------------------------------------

Baud Rate                                57600

Character Size                           8                   ?

Help:

This field can be any of 8, 7, 6, 5

-----------------------------------------------------------------------------

Parameter                                Current Value       New value

-----------------------------------------------------------------------------

Character Size                           8

Parity                                   none                ?

Help:

Possible Parity Values are as follows:

value               meaning

even                EVEN parity

odd                 ODD parity

none                DISABLE parity

mark                parity is MARK

space               parity is SPACE

-----------------------------------------------------------------------------

Parameter                                Current Value       New value

-----------------------------------------------------------------------------

Parity                                   none

No of stop bits                          1                   ?

Help:

This field can be one of 1, 1.5 or 2 only

-----------------------------------------------------------------------------

Parameter                                Current Value       New value

-----------------------------------------------------------------------------

No of stop bits                          1

Software output flow control             off                 ?

Help:

This field can have two values only: on or off

-----------------------------------------------------------------------------

Parameter                                Current Value       New value

-----------------------------------------------------------------------------

Software output flow control             off

Software input flow control              off                 ?

Help:

This field can have two values only: on or off

-----------------------------------------------------------------------------

Parameter                                Current Value       New value

-----------------------------------------------------------------------------

Software input flow control              off

Modem Name                               NULL_MODEM          ?

Help:

Input a modem name.  A modem name can be upto 14 chars.

Input NULL_MODEM if no modem is to be connected

-----------------------------------------------------------------------------

Parameter                                Current Value       New value

-----------------------------------------------------------------------------

Modem Name                               NULL_MODEM

Monitor Carrier                          on                  ?

Help:

This field can have two values only: on or off

-----------------------------------------------------------------------------

Parameter                                Current Value       New value

-----------------------------------------------------------------------------

Monitor Carrier                          on

Intr Char  (<^><char> or <\ooo>)         ^C                  ?

Help:

Input any char . Control chars can be given as ^(char) or

as \ooo, where 'o' denotes an octal digit. The octal format is

preferred for control characters and when inputting ? itself

-----------------------------------------------------------------------------

-----------------------------------------------------------------------------

Parameter                                Current Value       New value

-----------------------------------------------------------------------------

Intr Char  (<^><char> or <\ooo>)         ^C

Stop Character  (<^><char> or <\ooo>)    ^S                  ?

Help:

Input any char . Control chars can be given as ^(char) or

as \ooo, where 'o' denotes an octal digit. The octal format is

preferred for control characters and when inputting ? itself

-----------------------------------------------------------------------------

Parameter                                Current Value       New value

-----------------------------------------------------------------------------

Stop Character  (<^><char> or <\ooo>)    ^S

Start Character  (<^><char> or <\ooo>)   ^Q                  ?

Help:

Input any char . Control chars can be given as ^(char) or

as \ooo, where 'o' denotes an octal digit. The octal format is

preferred for control characters and when inputting ? itself

-----------------------------------------------------------------------------

Parameter                                Current Value       New value

-----------------------------------------------------------------------------

Start Character  (<^><char> or <\ooo>)   ^Q

Input hardware flow control              rts                 ?

Help:

Possible Values Are : none, rts

-----------------------------------------------------------------------------

Parameter                                Current Value       New value

-----------------------------------------------------------------------------

Input hardware flow control              rts

Output hardware flow control             cts                 ?

Help:

Possible Values Are : none, cts

-----------------------------------------------------------------------------

Parameter                                Current Value       New value

-----------------------------------------------------------------------------

Output hardware flow control             cts

Port type                                dialin              Read Only

Session prompt                           ->                  ?

Help:

Prompt can be any sequence of chars upto  10 chars long

-----------------------------------------------------------------------------

Parameter                                Current Value       New value

-----------------------------------------------------------------------------

Session prompt                           ->

Port IP Address                          0.0.0.0             ?

Help:

The IP address associated with this port.

If the "accept client IP address" option in the global RAS config

is set to "no" and the dialin path has no IP address associated

with it, then the client will he allotted this IP address.

-----------------------------------------------------------------------------

Parameter                                Current Value       New value

-----------------------------------------------------------------------------

Port IP Address                          0.0.0.0

#### \configuration\permanent\ports\groupmod

The command groupmod modifies a group of port, i.e. A1-A8.  The parameter options are the same as the modify command.

Figure 133 - BandWagon's definition: \configuration\permanent\ports\groupmod

BandWagon:\configuration\permanent\ports>

show, modify, groupmod

->> ? groupmod

GROUPMOD                                                                GROUPMOD

SYNOPSIS

groupmod     <portname|portgroup>

DESCRIPTION

Modify the characteristics of the Group of ports

The ports are to be of the SAME type. Example portgroup is A1,A5-A8

The "Current Value" field displays the actual parameter value or *,

depending on whether that parameter has the same value or not in each

port of the group

OPTIONS

none

BandWagon:\configuration\permanent\ports>

show, modify, groupmod

### \configuration\permanent\network\ip

This directory provides the necessary options to configure all the routing protocols parameters to properly configure the BandWagon RAS.  The changes made to the BandWagon's database with the commands in this directory will only take effect after a reboot is performed.  This can be done by turning the power off then on again, or by issuing the \administration\reboot command.

#### \configuration\permanent\network\ip\show

Figure 134 - BandWagon's definition: \configuration\permanent\network\ip\show

->> ? show

SHOW                                                                        SHOW

SYNOPSIS

show

DESCRIPTION

Show TCP/IP protocol suite parameters

OPTIONS

none

Figure 135 - BandWagon example: \configuration\permanent\network\ip\show

BandWagon:\configuration\permanent\network\ip>

show, modify

->> show

-----------------------------------------------------------------------------

Parameter                               Current Value

-----------------------------------------------------------------------------

Trace ARP packets (y/n)                 no

Send RWHOD packets (y/n)                no

Send routed (RIP) packets (y/n)         yes

Perform IP forwarding (y/n)             yes

Ack. TCP pkts immediately (y/n)         no

Use TCP keepalive (y/n)                 yes

Reverse Telnet load balancing (r/l)     linear

#### \configuration\permanent\network\ip\modify

Figure 136 - BandWagon definition: \configuration\permanent\network\ip\modify

BandWagon:\configuration\permanent\network\ip>

show, modify

->> modify

-----------------------------------------------------------------------------

Parameter                               Current Value  New value

-----------------------------------------------------------------------------

Trace ARP packets (y/n)                 no             ?

Help:

The Address Resolution Protocol (ARP) resolves the hostname to address

mappings.  If the ARP trace is ON, then a trace of all ARP packets sent or

received by the server is displayed on the Console.  If the trace is set to

OFF, this information is not displayed .

value               meaning

(y)es               Trace of ARP Packets will be displayed

(n)o                Suppress displaying the trace of ARP packets

-----------------------------------------------------------------------------

Send RWHOD packets (y/n)                no             ?

Help:

This field decides if rwhod packets are to be broadcast on the network

This broadcast is performed every 60 seconds.  It can be turned off to

reduce LAN traffic

value               meaning

(y)es               Rwhod packets are broadcast on the network

(n)o                No broadcasting of rwhod packets is done

-----------------------------------------------------------------------------

Send routed (RIP) packets (y/n)         yes            ?

Help:

This field decides if the RAS should transmit routed (RIP) information

Normally, RIP information should be transmitted only by nodes acting as a

gateway

value               meaning

(y)es               RAS will transmit RIP information

(n)o                RAS will NOT transmit RIP information

-----------------------------------------------------------------------------

Perform IP forwarding (y/n)             yes            ?

Help:

This field decides if the RAS can act as a gateway between two networks.

Normally this field is set to YES, since the RAS acts as a router. However

this field can be set to NO for diagnostic or other purposes

value               meaning

(y)es               Forward IP packets between networks

(n)o                No Forwarding of IP packets

-----------------------------------------------------------------------------

Ack. TCP pkts immediately (y/n)         no             ?

Help:

The RAS generates acknowledgment packets in response to incoming TCP

data. These ACK packets can be delayed for performance reasons to reduce

network traffic. This field controls whether TCP acknowledgment should

be delayed or immediate.

value               meaning

(y)es               Acknowledgment to TCP packets is IMMEDIATE

(n)o                DELAY acknowledgment to TCP packets

-----------------------------------------------------------------------------

Use TCP keepalive (y/n)                 yes            ?

Help:

TCP can perform a keepalive function by sending TCP keepalive messages

to the other end.  This ensures that TCP channels will be closed when

the peer node aborts.  This field decides whether the RAS should generate

TCP keepalive messages for its TCP channels

value               meaning

(y)es               Perform TCP keepalive function

(n)o                Do not perform TCP keepalive function

-----------------------------------------------------------------------------

Reverse Telnet load balancing (r/l)     linear         ?

Help:

Reverse Telnet is useful in sharing devices like printers, modems etc.

This field indicates which scheme is used to perform load balancing

among the shared devices.

value               meaning

r                   Load balancing is round robin

l                   Load balancing is Linear

Reverse Telnet load balancing (r/l)     linear

### \configuration\permanent\network\ipx

This directory will allow  the operator to view and modify all the IPX related parameters. The changes made to the BandWagon's database with the commands in this directory will only take effect after a reboot is performed.  This can be done by turning the power off then on again, or by issuing the \administration\reboot command.

#### \configuration\permanent\network\ipx\show

Figure 137 - BandWagon example: \configuration\permanent\network\ipx\show

->> show

-----------------------------------------------------------------------------

Parameter                                     Current Value

-----------------------------------------------------------------------------

Internal Network address(in hex)              0x0

Local LAN address (in hex)                    0x0

Ethernet type(Type II/Raw/802.3/Snap)         802.3

Send ipx rip broadcasts(send/quiet)           send

Forward ipx packets(yes/no)                   yes

BandWagon:\configuration\permanent\network\ipx>

show, modify

#### \configuration\permanent\network\ipx\modify

Figure 138 - BandWagon example: \configuration\permanent\network\ipx\modify

->> modify

-----------------------------------------------------------------------------

Parameter                                Current Value  New value

-----------------------------------------------------------------------------

Send ipx rip broadcasts(send/quiet)      send           ?

Help:

whether this machine should send IPX RIP broadcasts or not

value               meaning

(S)end              Send periodic IPX RIP broadcasts across network interfaces

(Q)iuet             Do NOT send any IPX RIP broadcasts at all. Be silent.

-----------------------------------------------------------------------------

Forward ipx packets(yes/no)              yes            ?

Help:

Whether this machine should forward IPX from one network to another

Note that if this parameter is set to "no", this machine will

CEASE TO FUNCTION AS A ROUTER AT ALL. However for diagnostic reasons

you may want this set this parameter to "no" temporarily

value               meaning

(Y)es               forward IPX packets across networks

(N)o                Do NOT forward IPX packets from one network to another

-----------------------------------------------------------------------------

BandWagon:\configuration\permanent\network\ipx>

show, modify

### \configuration\permanent\network\snmp

In this directory the operator sets the SNMP parameters that identifies the RAS to the SNMP management entity.  The changes made to the BandWagon's database with the commands in this directory will only take effect after a reboot is performed.  This can be done by turning the power off then on again, or by issuing the \administration\reboot command.

#### \configuration\permanent\network\snmp\show

Figure 139 - BandWagon example: \configuration\permanent\network\snmp\show

BandWagon:\configuration\permanent\network\snmp>

show, modify

->> show

-----------------------------------------------------------------------------

Parameter                               Current Value

-----------------------------------------------------------------------------

Location of the RAS (max 31 chars)

Person in charge of RAS (max 31 chars)

Authentification traps (Enable/Disable) disabled

Trap destinations (host_id:community)

Community (Community:permissions)       public:RO

BandWagon:\configuration\permanent\network\snmp>

show, modify

#### \configuration\permanent\network\snmp\modify

The section below  shows an example of the \configuration\permanent\network\snmp\modify command.  You can type a ? in each of the input fields to obtain detailed help as shown below.

->> modify

------------------------------------------------------------------------

Parameter                               Current Value  New value

------------------------------------------------------------------------

Location of the RAS (max 31 chars)                     ?

Help:

This field identifies the physical location of the RAS.  The name of

the location can be upto 31 characters long.  The factory default is

a null string.

------------------------------------------------------------------------

Person in charge of RAS (max 31 chars)                 ?

Help:

This field identifies the name of the person in charge of the RAS

typically the network manager.  The name can be up to 31 characters.

The factory default is a null string.

------------------------------------------------------------------------

Authentification traps (Enable/Disable) disabled       ?

Help:

This field controls the enabling/disabling of authentification traps.

The factory default disables this field

value               meaning

e                   enable authentication traps

d                   disable authentication traps

------------------------------------------------------------------------

Trap destinations (host_id:community)                  ?

Help:

This option specifies the  <host_id:community>  pair, which is to be used

to select the hosts to which traps are to be sent.  The host_id can

be a valid host name or an IP address in the dot notation.  The community

string can be a maximum of 15 characters and refers to a valid community

name for the SNMP manager running on the destination host.

The host_id part is compulsory while the community part is optional.

More than one <host_id:community> pair can be specified by separating

them with commas. A maximum of FIVE such pairs can be specified.

value               meaning

+<pair[,pair]>      Add these pair(s)

-<pair[,pair]>      Delete these pair(s)

Trap destinations (host_id:community)

------------------------------------------------------------------------

Community (Community:permissions)       public:RO                  ?

Help:

This field specifies the  <community:permissions> pairs, which are used

to validate incoming SNMP requests.

The community half of the pair can be a string of maximum 15 characters

length. The permissions half of the pair can be one of RO, RW, WO, NA.

The significance of these permissions options is as follows :

RO  :  A SNMP Manager belonging to the specified community can only

get information and not modify the information itself.

RW  :  A SNMP Manager belonging to the specified community can both

get information as well as modify the information.

WO  :  A SNMP Manager belonging to the specified community can not

get information but can modify the information.

NA  :  A SNMP Manager belonging to the specified community can

neither get information nor can modify the information.

More than one <community:perms> pair can be specified by separating

them with commas. A maximum of FIVE such pairs can be specified.

value               meaning

+<pair[,pair]>      Add these pair(s)

-<pair[,pair]>      Delete these pair(s)

BandWagon:\configuration\permanent\network\snmp>

------------------------------------------------------------------------

The "person in-charge of the RAS" field lets you  identify the person in charge of the  BandWagon; normally, the this is the system administrator responsible for the day-to-day operations of the BandWagon.  You may enter up to 31 printable ASCII characters, including letters, numbers, spaces and punctuation.

By default the Disable Authentication Traps is turned disable.  This means that authentication traps are disabled.   When authentication traps are enabled, the administrator can use the SNMP manager will be notified by the BandWagon's SNP manager,  whether an illegal SNMP manager is trying to access the BandWagon's SNMP agent If you don't want to track this, set the Authentication Traps field to enable.

When authentication trap checking is enabled the Trap Destinations... button

lets you specify trap hosts.

#### \configuration\permanent\network\iptunnel

IP tunnel allows for bridging of IPX networks over an IP internet.  The RAS can act as a gateway to IPX only networks.  Two RAS systems, attached each to their own IPX networks can be bridged using the RAS IP routing protocols.  In addition a RAS configured for IP protocol, can use IPX tunneling to forward IPX packets to other IPX networks connected by IP routers.  The IP routers act as gateways to the interconnected IPX networks.  Although the RAS will find the routers using the RIP protocol, the RAS can be programmed to forward the IPX packets to specific router's IP addresses, in this case, RIP could be turned off to reduce traffic over the dial-up link.

The RAS \configuration\permanent\network\iptunnel path allows configuration of IP tunnels within the RAS.  In RAS-to-RAS configuration, the called and calling RASs have each other's  IP address and IPX network addresses programmed in their RAS tunnel tables.  Up to 3 additional IP routing gateways can be specified to allow reaching the desired IPX network.  Using IP tunnelling saves processing load in the RAS, reduces the traffic in the WAN links (RIP protocol  off), and provides faster response to internet traffic.

Note: Leemah  	I think this is what IPTUNNEL could be used for, nor am I sure if this is what it is intended for.  Please verify if this is correct, or if I am blowing smoke here.

Figure 140 - BandWagon example: \configuration\permanent\network\iptunnel\show

BandWagon:\configuration\permanent\network\iptunnel>

show, add, modify, delete, clear

->> show

IPX network                 IP tunnel peer list

0xfffffff1                  193.1.2.1

0xfffffff2                  193.1.1.2

0xfffffff3                  193.1.1.3

0xfffffff4                  193.1.1.4

0xfffffff5                  193.1.1.5

0xfffffff6

BandWagon:\configuration\permanent\network\iptunnel>

show, add, modify, delete, clear

Note: Leemah  	Tried to add more than one IP address to a single ipx net and it did not work.  If what I said above about the use of this feature, then it is very important that the IPX entry allows to specify more than one gateway.  Although it may work if the router has RIP turned on or if it has a static route to forward the packet to its next neighbor, therefore, it may reach its destination.  This is something to check and verify.  Again I may still be blowing smoke.

The IPTUNNEL tables can be modified using the ..\add, \modify, and ..\delete commands.  The ..\clear commands erases all the entries in the table.  In the examples below entries are made to the IPTUNNEL table and the ..\show command is used to verify the changes.  At the end the ..\clear command is used to erase the tables.

Figure 141 - BandWagon example: \configuration\permanent\network\iptunnel\add

BandWagon:\configuration\permanent\network\iptunnel>

show, add, modify, delete, clear

->> add

-----------------------------------------------------------------------

Parameter                                Current Value  New value

-----------------------------------------------------------------------

IPX network(hex) to be IPtunneled        -              fffffff7

IP tunnel peer list(name/ip address)     -

193.1.1.7

Edit, Abort, Save : s

BandWagon:\configuration\permanent\network\iptunnel>

show, add, modify, delete, clear

->> show

IPX network                 IP tunnel peer list

0xfffffff1                  193.1.2.1

0xfffffff2                  193.1.1.2

0xfffffff3                  193.1.1.3

0xfffffff4                  193.1.1.4

0xfffffff5                  193.1.1.5

0xfffffff6

0xfffffff7                  193.1.1.7

BandWagon:\configuration\permanent\network\iptunnel>

show, add, modify, delete, clear

->>

Figure 142 - BandWagon example:  \configuration\permanent\network\iptunnel\modify

BandWagon:\configuration\permanent\network\iptunnel>

show, add, modify, delete, clear

->> modify fffffff6

------------------------------------------------------------------------

Parameter                                Current Value  New value

------------------------------------------------------------------------

IP tunnel peer list(name/ip address)

193.1.1.6

Edit, Abort, Save : s

BandWagon:\configuration\permanent\network\iptunnel>

show, add, modify, delete, clear

->> show

IPX network                 IP tunnel peer list

0xfffffff1                  193.1.2.1

0xfffffff2                  193.1.1.2

0xfffffff3                  193.1.1.3

0xfffffff4                  193.1.1.4

0xfffffff5                  193.1.1.5

0xfffffff6                  193.1.1.6

0xfffffff7                  193.1.1.7

BandWagon:\configuration\permanent\network\iptunnel>

show, add, modify, delete, clear

->>

Figure 143 - BandWagon example:  \configuration\permanent\network\iptunnel\delete

->> delete fffffff7

-----------------------------------------------------------------------

Parameter                                     Current Value

-----------------------------------------------------------------------

IPX network(hex) to be IPtunneled             0xfffffff7

IP tunnel peer list(name/ip address)          193.1.1.7

Delete this entry(y/n)(n)? y

ipx_net 'fffffff7' deleted

BandWagon:\configuration\permanent\network\iptunnel>

show, add, modify, delete, clear

->> show

IPX network                 IP tunnel peer list

0xfffffff1                  193.1.2.1

0xfffffff2                  193.1.1.2

0xfffffff3                  193.1.1.3

0xfffffff4                  193.1.1.4

0xfffffff5                  193.1.1.5

0xfffffff6                  193.1.1.6

BandWagon:\configuration\permanent\network\iptunnel>

show, add, modify, delete, clear

Figure 144 - BandWagon example:  \configuration\permanent\network\iptunnel\clear

BandWagon:\configuration\permanent\network\iptunnel>

show, add, modify, delete, clear

->> clear

This command will clear the whole IP tunneling information database.

Proceed(y/n)(n)? y

BandWagon:\configuration\permanent\network\iptunnel>

show, add, modify, delete, clear

->> show

IP tunneling information database empty

BandWagon:\configuration\permanent\network\iptunnel>

show, add, modify, delete, clear

#### \configuration\permanent\network\isdn\digi

The ISDN Basic Rate Interface (BRI) Board is referred to as the "digi board".  This command is used to program the parameters required for proper operation of the ISDN BRI board.

The switch style defines the central office switch used to provision the ISDN service.  It is necessary to know from your ISDN service provider the manufacturer and model of the ISDN switch used by your ISDN service provider.  The ISDN service provider has to assign an SPID number  for each BRI interface.  If you are using the S/T interface and you are planning of adding more ISDN compliant devices, you need to specify the number of logical terminals assigned to the interface.  Each ISDN device will negotiate with the ISDN switch the assignment of an ISDN Terminal Identifier Number.

Figure 145 - BandWagon example: \configuration\permanent\network\isdn\digi\show

BandWagon: \configuration\permanent\network\isdn\digi

show

->>show

-------------------------------------------------------------------------

Parameter                                               Current Value

Line 1: Switch style                                    3

No. of Logical Terminals                             1

SPID 1                                               51058105231

ADDR 1                                               5810523

Line 2: Switch style                                    3

No. of Logical Terminals                             1

SPID 1                                               51058105231

ADDR 1                                               5810523

Line 3: Switch style                                    3

No. of Logical Terminals                             1

SPID 1                                               51058105231

ADDR 1                                               5810523

Line 4: Switch style                                    3

No. of Logical Terminals                             1

SPID 1                                               51058105231

ADDR 1                                               5810523

Note: Leemah  	I do not have a sample of the digi modify command display.  Need to know the CO switch number settings options.  I have no way to verify what they are since I don not have a BRI card.

#### \configuration\permanent\network\isdn\pri>

The user/exchange denotes the protocol side to be used in the ISDN D channel protocol.  The ISDN PRI protocol specify two sides in the User Network Interface.  One side of the protocol is designated as and exchange, the other side is designated as a user.  In most cases, since the PRI card is connected to a Central Office switch, it has to be configured as user.  In situations were the PRI is connected behind a Private Communications Branch Exchanges (PBX) it may have to be set as network side.  Please consult with your telecommunications department to verify if this is the case.

The PRI card supports two PRI interfaces in the card.  Line A, and line B refer to one separate ISDN interface line each.

Figure 146 - BandWagon definition: \configuration\permanent\network\isdn\pri\show

BandWagon:\configuration\permanent\network\isdn\pri>

show

->> show

-----------------------------------------------------------------------------

Parameter                               Current Value

-----------------------------------------------------------------------------

ISDN Configuration: (u)ser/(e)xchange   user

ISDN Line type : line(a)/line(b)        line A

ISDN Line Switch Style:                 0

BandWagon:\configuration\permanent\network\isdn\pri>

show

#### \configuration\permanent\network\netbeui>

NetBeui Protocol is the most commonly used protocol in LAN Manager network implementations.  It supports NetBIOS 3.0 interface standards for use on OS/2 and Windows 3.X and Windows for Workgroups.  NetBIOS provides three primary communication services, local name management, virtual circuit establishment, and datagram communication.  Local names are registered with NetBIOS and provides a means to identify network resources, users, domains, computer names, and relative paths.  An example of a name is a shared hard disk.

Virtual circuits are communication links  between tow names that provide for the reliable delivery of packets.  This implies that the receiving name most acknowledge receipt of transmitted packets.  Datagram communications is a means to transmit packets of data from one NetBIOS name to another without a guarantee of delivery.  The NetBIOS transmitting end assumes that the packets are received by the destination.

NetBeui is not a routable protocol.  It is meant to be used in directly LAN attached workstations.  Because of this design restriction, NetBeui is not tolerant of line delays found in WAN lines.  It is necessary to modify NetBeui protocol default timers for operation over WAN links or dial-up lines.  The default values provided in the BandWagon will work in most instances.  It may be necessary to adjust the timers in cases of very slow or noisy lines.  To alter the NetBeui parameters under windows the Protocol.ini file most be modified.  The [NETBEUI] section may need to have the following parameters added:

[NETBEUI]

NETBIOSTIMEOUT=3000

NETBIOSRETRIES=2

T1=5000

T2=2000

MAXOUT=40

MAXIN=40

- NETBIOSTIMEOUT - is the amount of time the NetBIOS will wait for a replay to its name inquiries in the network.

- NETBIOSRETRIES - is the number of times that NetBIOS will try again after the timeout is completed.

- T1-  this is the time the system will wait for an acknowledgment of the window packets were received by the receiving end-station.

- T2 - This is the time that the system will wait before sending an acknowledgment.

- Ti - This  how often the link layer will check an inactive link to see if it is operational.

- MAXOUT - Is equivalent to a send window size.

- MAXIN - Is equivalent to the receive window size.

The BandWagon has an equivalent set of parameters that can be set to match those set in the workstation.

The parapets shown below defines the NetBeui parameters for the BandWagon's LAN port.  The individual remote access ports are configured in the \cofiguration\path directory.

Figure 147 - BandWagon's definition: \configuratin\permanent\network\netbeui

->> modify

------------------------------------------------------------------------

Parameter                               Current Value  New value

------------------------------------------------------------------------

T1    (milliseconds)                    5000           ?

Help:

The T1 timer defines the time interval during which the LLC shall

expect to receive an acknowledgment to one or more outstanding

Information frames or an expected response frame to a sent Unnumbered

command PDU. This timer must obey the following rule

T2 <= T1 <= <TI

------------------------------------------------------------------------

T2    (milliseconds)                    3000           ?

Help:

The t2 timer defines the time interval during which the LLC shall

wait for either an information frame to become available for

transmission or to accumulate additional Information frame's to

be acknowledged in

a single Receive Ready Frame This timer must obey the following rule

T2 <= T1 <= <TI

------------------------------------------------------------------------

Ti    (milliseconds)                    30000          ?

Help:

The TI timer determines how often the LLC checks an inactive link

to see if it is operational.

This timer must obey the following rule

T2 <= T1 <= <TI

------------------------------------------------------------------------

Send Window Size (1 - 127)              40             ?

Help:

This field denotes the number of LLC packets to send before

expecting an acknowledgment.

------------------------------------------------------------------------

Receive Window Size (1 - 127)           40             ?

Help:

This field denotes the number of LLC packets to receive before

sending an acknowledgment.

------------------------------------------------------------------------

Lan Wan Threshold                       1              ?

Help:

Number of Netbeui Name query retries from LAN before allowing them

to be propagated to the WAN links.  This should typically not exceed 3.

------------------------------------------------------------------------

### \configuration\permanent\network\acs>

For more information on the BandWagon's ACS services refer to the section \configuration\runtime\network\acs.

The \configuration\permanent\network\acs directory provides the commands to set the BandWagon's Asynchronous Communications Services.  The information below provides some information concerning the ACS process used in the BandWagon RAS.  This is process includes Dial-Out and Dial-In services fro- and-to the LAN attached to the RAS Ethernet Port.

Figure 148 - BandWagon example: \configuration\permanent\network\acs\modify

BandWagon:\configuration\permanent\network\acs>

->> modify

----------------------------------------------------------------------

Parameter                               Current Value  New value

----------------------------------------------------------------------

Prompt string  (max. 11 chars)          NASI           ?

Help:

The acs prompt string can be upto 11 characters long

Prompt string  (max. 11 chars)          NASI

macs port-id  (Hex)                     0x4009         Read Only

Figure 149 - BandWagon example: \configuration\permanent\network\acs\show

->> show

----------------------------------------------------------------------

Parameter                               Current Value

----------------------------------------------------------------------

Prompt string  (max. 11 chars)          NASI

macs port-id  (Hex)                     0x4009

BandWagon:\configuration\permanent\network\acs>

### \configuration\permanent\ras

Use this directory to change general parameters related to the RAS object configuration.

Figure 150 - BandWagon's definitions: \configuration\permanent\ras\modify

->> modify

------------------------------------------------------------------------

Parameter                               Current Value  New value

------------------------------------------------------------------------

Server name (max 15 chars)              BandWagon      ?

Help:

This field specifies the name of the server.  The name should not exceed

15 characters, and should not have control characters or intervening white

spaces.  The default name is BandWagon.

Console error messages level (10-100)   66             ?

Help:

This field indicates the Console error message level.  This parameter

defines the extent to which detailed messages should be displayed on

the console. A larger value for level indicates more detailed messages.

The possible values for this field are between 10 and 100

Console messages buffer level (10-100)  65             ?

Help:

This field indicates the extent to which the console messages should be

buffered for later viewing (with the kmesg command).  A larger value for

level indicates more detailed messages.  The possible values for this

field are between 10 and 100

Figure 151 - BandWagon's example:  \configuration\permanent\ras\show

->> show

----------------------------------------------------------------------

Parameter                               Current Value

----------------------------------------------------------------------

Server name (max 15 chars)              BandWagon

Console error messages level (10-100)   66

Console messages buffer level (10-100)  65

### \configuration\permanent\service

This directory is used to set the TCP password daemon socket number.  This number is used by the TCP protocol to link the login services between the RAS TCP server and the Client's TCP server.  The default number should work for standard installations.  This number is also used by the WinRAS program. WinRAS is a Windows™  based program used to configure the BandWagon RAS from a PC connected to the LAN port.  Both, the WinRAS and the BandWagon RAS have to have the same port number.  Please refer to the WinRAS configuration manual for more information.

F Note: 	If you do not have a copy of the WinRAS program, contact LeeMah Datacom for information.

Figure 152 - BandWagon example: \configuration\permanent\service

BandWagon:\configuration\permanent>

maintenance, ports, network, ras, service, winras

->> service

-----------------------------------------------------------------------

Parameter                               Current Value  New value

-----------------------------------------------------------------------

TCP port of password daemon             1657

Edit, Abort, Save : a

service ABORTED...

BandWagon:\configuration\permanent>

maintenance, ports, network, ras, service, winras

->>

### \configuration\permanent\winras

This directory is used to program the parameters required to communicated with the WinRAS. The parameters programmed here have to agree with the parameters set in the WinRAS.

Figure 153 - BandWagon example: \configuration\permanent\winras

->> winras

------------------------------------------------------------------------

Parameter                               Current Value  New value

------------------------------------------------------------------------

WinRAS address                          0.0.0.0        ?

Help:

----

IP address of the WinRAS PC

WinRAS address                          0.0.0.0        192.10.1.1

TCP port for WinRAS commn               1658           ?

Help:

----

Configure an unused TCP port for winras communication. The TCP port

can range between 1 and 65535

TCP port for WinRAS commn               1658

Shared Secret                           -              *

Help:

----

The Shared Secret of WinRAS. It is a string of upto 16 chars

Shared Secret                           -              *****

Please re-enter                         *****          *****

Edit, Abort, Save : s

BandWagon:\configuration\permanent>

maintenance, ports, network, ras, service, winras

->>

The shared secret is part of the BandWagon's standard RADIUS security administration complex.  RADIUS is a system of distributed security that protects remote access to networks and network services against unauthorized access.  RADIUS authentication includes two process: authentication server process and the authentication client process.  All user authentication and network service access information is located on the authentication servers. Server may run on a RAS or a security authentication server (host SAS) in the network.  The user trying to access the network via the RAS is required to identify himself by giving his access code (user name) and password.  The client on the RAS, interacts with he servers to authenticate a user.  Transactions between the client and server are authenticated by means of a shared secret, which is never sent over the network.  In addition, any user passwords are sent encrypted between the client and server, to eliminate the possibility that someone snooping on an access line could determine a user's password.

It is important that for proper operation of the BandWagon's security system that the parameters in the winras and service directories be set in the same way at both, the BandWagon RAS and the Client/WinRAS sides.

## \configuration\path

The \configuration\path is critical for the proper operation of the BandWagon RAS.  Each user account has a path definition assigned to it.  The path defines important protocol parameters to be used for the connection assigned to a given user.  The ability to define multiple paths allows for many different combinations that can be tailored to satisfy from the most simple remote access application to the most sophisticated one according to the requirements needed for each application.  In summary the path defines a "class-of-service" to be assigned to a particular user or users.  The path command allows to show, add, delete and modify the class-of-service attached to the path's name.  The following features are defined in the class-of-service path name:

- Type of connection

- Compression

- Protocol class parameters

- Address information

- Protocol negotiation parameters

- Security negotiation parameters

- Protocol timing parameters.

Explanation of the path directory follows below.

Figure 154 - BandWagon example: \configuration\path

-----------------------------------------------------------------------

Parameter                                   Current Value       New value

-----------------------------------------------------------------------

Status (Enabled/Disabled)                   Disabled

Connection Type (Demand/Persistent/Client)  Persistent          ?

Help:

Enter the type of connection with the destination

value       meaning

P           Persistent - Connection is active always

C           Client  - The connection is with a RAS client and is started

by the login process

D           Demand  - The connection can be started either by the login

process or by the administrator through the start

command

------------------------------------------------------------------------

Destination Type (Host/Gateway)             Gateway             ?

Help:

Enter the type of destination

value       meaning

H           Destination is a host

G           Destination is a gateway

------------------------------------------------------------------------

Logchat                                     Null

Logchat file must be loaded

User Name                                   reagan

Password                                    *****

Site Key                                    11111111

User Key                                    22222222            ?

Help:

Infocard User cipher key.

------------------------------------------------------------------------

Spoofing type(None/Triggered)               Triggered           ?

Help:

Whether Spoofing is to be done on RIP/SAP broadcasts. Spoofing reduces

cost by reducing/avoiding periodic RIP/SAP broadcasts on WAN links

value               meaning

Triggered           Triggered spoofing as per RFC 1582

None                No Spoofing will be done

------------------------------------------------------------------------

Data Compression (None/RLE/LZW)             RLE                 ?

Help:

Enter the data compression method to be used over the path.

Should be used only for RAS - RAS or RAS - RAS Client

value               meaning

None                Disable data compression

RLE                 Use Run Length Encoding method for compression

LZW                 Use LZW method for compression

------------------------------------------------------------------------

Protocol (SLIP/PPP/MLPPP)                   MLPPP               ?

Help:

Enter the data link protocol to be used over this path

value               meaning

M                   Multi Link Point to Point Protocol(MLPPP)

S                   Serial Line Internet Protocol(SLIP)

P                   Point to Point Protocol(PPP)

------------------------------------------------------------------------

High Water Mark (1 - 100)                   80                  ?

Help:

Enter the High Water Mark - if the traffic on a link increases beyond the high water mark (in % of capacity), another port will be added to the link

------------------------------------------------------------------------

Low Water Mark (1 - 100)                    20                  ?

Help:

Enter the Low Water Mark - if the traffic on a link decreases beyond the

low water mark (in % of capacity), a port will be deleted from the link

------------------------------------------------------------------------

Time Slice(min)                             30                  ?

Help:

Enter the Time Slice - it is the time for which a link will be monitored as well as the time interval between two successive monitors.  If a triggered request is not received during this interval, the BandWagon assumes the logical connection is disconnected.

------------------------------------------------------------------------

IP Status (oN/oFf)                          on                  ?

Help:

Whether IP connectivity is to be enabled on this path or not

value               meaning

oN                  IP connectivity is enabled

oFf                 IP connectivity is disabled

------------------------------------------------------------------------

Destination IP Address                      REMOTE              ?

Help:

The IP destination address of this path.

It is RECOMMENDED that this field be left at REMOTE which is the default

value. In such a case, the address of the destination will be obtained by PPP negotiation with the other end.  If desired, DYNAMIC can be used to indicate that this RAS should assign an IP address from the port on which the other end has called.  This is meaningful only when this path has received a call, typically Client

value               meaning

REMOTE              Remote peer assigns an address

DYNAMIC             This RAS assigns an address from the port

a.b.c.d             The IP address itself

------------------------------------------------------------------------

VJ Compression Status (oN/oFf)              on                  ?

Help:

Enter whether the Van Jacobson compression is to be enabled or not

value               meaning

oN                  VJ compression is enabled

oFf                 VJ compression is disabled

------------------------------------------------------------------------

Maximum Slot Index(1-256)                   16                  ?

Help:

Enter the maximum slot. Should be between 1 and 256. Default of 16

Maximum Slot Index(1-256)                   16

------------------------------------------------------------------------

IPX Status (oN/oFf)                         on                  ?

Help:

Whether IPX connectivity is to be enabled on this path or not

value               meaning

oN                  IPX connectivity is enabled

oFf                 IPX connectivity is disabled

------------------------------------------------------------------------

Destination IPX Address                     *.*                 ?

Help:

The IPX destination address of this path. It is RECOMMENDED that this

field be left as *.* which is the default value. In such a case, the

address of the destination will be obtained by PPP negotiation with the

other end. If desired, an IPX address can be given in net.mac_address

format in hex notation

------------------------------------------------------------------------

NetBeui Status (oN/oFf)                     on                  ?

Help:

Whether Netbeui connectivity is to be enabled on this path or not

value               meaning

oN                  NetBeui connectivity is enabled

oFf                 NetBeui connectivity is disabled

------------------------------------------------------------------------

T1 Timer(in millisecs)                      6000                ?

Help:

The T1 timer defines the time interval during which the LLC shall

expect to receive an acknowledgment to one or more outstanding

frames. This timer must obey the following rule T2 <= T1 <= <TI

------------------------------------------------------------------------

T2 Timer(in millisecs)                      1000                ?

Help:

The t2 timer defines the time interval during which the LLC shall

wait before acknowledging the receipt of a frame

This timer must obey the following rule T2 <= T1 <= <TI

------------------------------------------------------------------------

TI Timer(in millisecs)                      120000              ?

Help:

The TI timer determines how often the LLC checks an inactive link

to see if it is operational.  This timer must obey the following rule

T2 <= T1 <= <TI

------------------------------------------------------------------------

Send Window Size(1-127)                     40                  ?

Help:

This field denotes the number of LLC packets to send before

expecting an acknowledgment.

------------------------------------------------------------------------

Receive Window Size(1-127)                  40                  ?

Help:

This field denotes the number of LLC packets to receive before

sending an acknowledgment.

------------------------------------------------------------------------

Retry count                                 6                   ?

Help:

This field specifies the maximum number of times that a frame is sent

following the running out of T1 timer

------------------------------------------------------------------------

Search Broadcast allowed(Yes/No)            Yes                 ?

Help:

Enter whether Netbeui name search broadcast allowed. This controls the flow of Netbeui NAME_QUERY packets TO the concerned path.  It is recommended that this option be kept to "no" for links not requiring the starting of sessions TO machine(s) located on the side dialing in. For instance PC dialin clients.  Please note that this option does not control the flow of such packets FROM the concerned path to the network dialed into.

value               meaning

Yes                 Name search broadcast will be allowed

No                  Name search broadcast will not be allowed

------------------------------------------------------------------------

Name traffic allowed(Yes/No)                Yes                 ?

Help:

Enter whether Netbeui datagram traffic allowed.  This controls the flow of NAME DATAGRAMS TO the concerned link.  Certain Netbeui applications generate name datagrams at regular intervals.  This will keep the dialup link busy, and prevent an idle timeout.  Hence "no" is recommended for dialup links which require an idle timeout

value               meaning

Yes                 Netbeui datagram traffic will be allowed

No                  Netbeui datagram traffic will not be allowed

------------------------------------------------------------------------

Port Pool 1                                 A1-A3               ?

Help:

Enter the ports in the port pool.  All the ports in the pool should be of same type.  A "-" (minus) would clear the current and subsequent pools.

------------------------------------------------------------------------

Pool Type (Demand/Persistent)               Demand              ?

Help:

Enter the type of pool: (d)emand/(p)ersistent

------------------------------------------------------------------------

Telephone Number                            9991235555

Port Pool 2                                 A4-A6

Pool Type (Demand/Persistent)               Demand

Telephone Number                            9991234444

Port Pool 3                                 -

Change PPP negotiation parameters? (Yes/No) No                  ?

Help:

Enter "yes" to change the PPP negotiation parameters

Change PPP negotiation parameters? (Yes/No) No                  Y

Negotiate Maximum Receive Unit (MRU)(Yes/No)Yes                 ?

Help:

Enter whether Maximum Receive Unit negotiation(MRU) is to be done or not

value               meaning

Yes                 Maximum Receive Unit negotiation is enabled

No                  Maximum Receive Unit negotiation is disabled

------------------------------------------------------------------------

Maximum Receive Unit in number of octets    1500                ?

Help:

Enter the maximum number of octets per PPP packet you are willing

to receive/send.  Maximum Receive Unit in number of octets    1500

------------------------------------------------------------------------

Negotiate Async map(Yes/No)                 Yes                 ?

Help:

Enter whether asynchronous control map negotiation is to be done or not

value               meaning

Yes                 Async map negotiation is enabled

No                  Async map negotiation is disabled

------------------------------------------------------------------------

Async control character map                 ffffffff            ?

Help:

Type the hex number representing the asynchronous control character map.  The default is ffffffff. PPP can map any ASCII control character into an appropriate two character sequence.  The ACCM field consists of four octets with each bit representing one control character which should be mapped.  The least significant bit corresponds to ASCII 0 and so on. If a bit is zero then that control character is not mapped. If a bit is one, then the corresponding control character must be mapped. If you wish to use software flow control, you will need to map control-S and Control-Q to avoid confusing data with flow control characters.

------------------------------------------------------------------------

This is used only with ASYNC lines, and is not applicable to other

non-ASYNC lines

------------------------------------------------------------------------

Negotiate Authentication(Yes/No)            Yes

Authentication Protocol(PAP/CHAP)           CHAP                ?

Help:

Choose the password authentication protocol. This has to be

agreed to by the peer during LCP negotiation

value               meaning

P                   PAP

C                   CHAP

------------------------------------------------------------------------

Secret                                      *****

Protocol Field Compression(oN/oFf)          On                  ?

Help:

Enter whether to enable Protocol Field compression or not.

Protocol compression will limit any PPP protocol id whose value

is less than 256 to a single byte, thus saving a byte in the PPP header.

This has to be agreed to by the peer during LCP negotiation.

value               meaning

oN                  Protocol Field Compression is enabled

oFf                 Protocol Field Compression is disabled

------------------------------------------------------------------------

Address Control Field Compression(oN/oFf)   On                  ?

Help:

Enter whether to enable Address & Control Field compression or not.

Address & Control field compression will eliminate the respective

bytes from the PPP header. This has to be agreed to by the peer

during LCP negotiation.

value               meaning

oN                  Address Control Field Compression is enabled

oFf                 Address Control Field Compression is disabled

## \configuration\security

The configuration\security directory configures the users database, allows specifying Security Access Servers to be defined, and provides the commands to display and define audittrials.

For normal operation of the RAS it is necessary to at least define all the user's access codes, passwords and assigned a path to each user.  The following sections shows the different options in this directory.

### \configuration\security\users

Define the users database.  Define a user access code for each remote access user, or for users dialing out from the LAN attached to the BandWagon's Ethernet port.

#### \configuration\security\users\show

This section shows how to add a user to the BandWagon RAS database.  All remote access users need to have an entry in the users database.  The figures below show a display of the users database, and a session showing the process of adding a user and requesting help for each entry.  The section closes with an explanation of the process.

Figure 155 - BandWagon's example: \configuration\security\users\show

BandWagon:\configuration\security\users>

show, add, modify, delete, reset, clear, upload, download

->> show

--------------------------------------------------------------------

AccessCode      User ID         Authentication  Login-Session

--------------------------------------------------------------------

root            root            CR2             Admin

willie          willie          PT              Lpath

bush            bush            PT              Lpath

test            tesdt           PT              Lpath

barbara         barbara         CR1             Lpath

admin           admin           CR2             Admin

BandWagon:\configuration\security\users>

show, add, modify, delete, reset, clear, upload, download

->>

#### \configuration\security\users\add

Figure 156 - BandWagon's user's database: adding a user

->> add ana

Parameter                                     Current Value   New value

-----------------------------------------------------------------------

User ID                                       -               ?

-----------------------------------------------------------------------

Help:

This field indicates the long name of the user

User ID                                       -               ana

Max. connection time (mins)                   0               30

Session(None/Admin/Path/Lpath/aCs)            None            ?

-----------------------------------------------------------------------

Help:

Type of session that this account is entitled to.

value               meaning

(N)one      NOT entitled to any sessions, though this is a valid account

(A)dmin             entitled to an administrative shell

(P)ath              entitled to open a path

(L)path             entitled to a login path account

a(C)s               entitled to access ACS resources

Authentication type

(PT/CR1/CR2/RO1/RO2/MRO1/MRO2)            PT              ?

-----------------------------------------------------------------------

Help:

This field indicates the Authentication type of the user.

value               meaning

PT                  Pass Through - No further authentication

CR1                 Infocard - I with Challenge/Response Mode

CR2                 Infocard - II with Challenge/Response Mode

RO1                 Single Key Infocard - I with Response Only Mode

RO2                 Single Key Infocard - II with Response Only Mode

MRO1                Multikey Infocard - I with Response Only Mode

MRO2                Multikey Infocard - II with Response Only Mode

password                                      -               ***

Please re-enter                               ***             ***

Call back type(None,Fixed,Variable)           None            ?

-----------------------------------------------------------------------

Help:

This field indicates the type of call back to be done.

value               meaning

(N)one              No call back is done

(F)ixed             Fixed call back

(V)ariable          Variable call back

Call back type(None,Fixed,Variable)           None            V

No. of call back retries(0-25)                4

Delay between call back retries(in secs)      20

#### \configuration\security\user\modify

Figure 157 - BandWagon's definition: \configuration\security\users\modify

BandWagon:\configuration\security\users>

show, add, modify, delete, reset, clear, upload, download

->> ? modify

MODIFY                                                            MODIFY

SYNOPSIS

modify       <AccessCode>

DESCRIPTION

Modifies the details of an AccessCode in the AccessCode

information database

OPTIONS

<AccessCode>  Indicates AccessCode whose details are to be modified.

BandWagon:\configuration\security\users>

show, add, modify, delete, reset, clear, upload, download

Figure 158 - BandWagon example: \configuration\security\users\modify

BandWagon:\configuration\security\users>

show, add, modify, delete, reset, clear, upload, download

->> modify ana

-----------------------------------------------------------------------

Parameter                                     Current Value   New value

-----------------------------------------------------------------------

User ID                                       ana

Max. connection time (mins)                   30

Session(None/Admin/Path/Lpath/aCs)            None            L

path name                                     -               nb

User type(Local,External)                     External

Authentication type

(PT/CR1/CR2/RO1/RO2/MRO1/MRO2)            PT              cr2

Site Key                                      -               77777777

User Key                                      -               88888888

Call back type(None,Fixed,Variable)           None

Edit, Abort, Save : s

In the example above the user account ana was modified.  The modifications are shown in bold for clarity.  Notice at the Edit, Abort, Save : s  saves the changes made to the user account.  The Edit allows you to go back to the beginning and do more changes, the Abort, exits and discards any changes made.

#### \configuration\security\user\delete

Figure 159 - BandWagon's definition: \configuration\security\users\delete

BandWagon:\configuration\security\users>

show, add, modify, delete, reset, clear, upload, download

->> ? delete

DELETE                                                           DELETE

SYNOPSIS

delete       <AccessCode>

DESCRIPTION

Deletes an AccessCode from the AccessCode information database

OPTIONS

<AccessCode>  Indicates AccessCode to be deleted.

BandWagon:\configuration\security\users>

show, add, modify, delete, reset, clear, upload, download

Figure 160 - BandWagon example: \configuration\security\users\delete

->> delete barbara

Do you really wish to delete the entry barbara ? (y/n) y

AccessCode 'barbara' deleted

BandWagon:\configuration\security\users>

show, add, modify, delete, reset, clear, upload, download

#### \configuration\security\user\reset

This command resets the users account security response code.  It is necessary to reset the ROx and MROx type of cards after the user account is reset.  This will synchronize the infocards with the BandWagon RAS.

Figure 161 - BandWagon's definition: \configuration\security\user\reset

->> ? reset

RESET                                                          RESET

SYNOPSIS

reset        <AccessCode>

DESCRIPTION

Resets the response field for a given AccessCode

This command fills with the response field with an initial value.

OPTIONS

<AccessCode>  Indicates AccessCode whose response is to be reset.

BandWagon:\configuration\security\users>

show, add, modify, delete, reset, clear, upload, download

#### \configuration\security\user\clear

The clear commands erases the user database entirely.  Use this command only if you have the need to reconfigure the system with an entire new set of user accounts

Figure 162 - BandWagon's definition: \configuration\security\user\clear

->> ? clear

CLEAR                                                             CLEAR

SYNOPSIS

clear

DESCRIPTION

Clears the AccessCode information database.

OPTIONS

none

BandWagon:\configuration\security\users>

show, add, modify, delete, reset, clear, upload, download

Figure 163 - BandWagon example: \configuration\security\user\clear

->> clear

This command will clear the whole AccessCode information database.

Proceed(y/n)(n)? y

clearing user database

BandWagon:\configuration\security\users>

show, add, modify, delete, reset, clear, upload, download

BandWagon:\configuration\security\users>

show, add, modify, delete, reset, clear, upload, download

->> show

--------------------------------------------------------------------

AccessCode      User ID         Authentication  Login-Session

--------------------------------------------------------------------

BandWagon:\configuration\security\users>

show, add, modify, delete, reset, clear, upload, download

In the example above the user’s database is first cleared and the show command is used to verify that the database was completely cleared.

#### \configuration\security\user\upload

Uploads the user's data base to a specified file and host PC.

Figure 164 - BandWagon's definition: \configuration\security\user\upload

BandWagon:\configuration\security\users>

show, add, modify, delete, reset, clear, upload, download

->> ? upload

UPLOAD                                                       UPLOAD

SYNOPSIS

upload       [host:]<filename> [<AccessCode>]

DESCRIPTION

Upload User Configuration to the specified file on the

indicated host

OPTIONS

[host:]       Indicates the remote host to upload. If it is not

specified, the boot server will be selected

<filename>    Filename for uploading

[AccessCode]  Indicates AccessCode whose details are to be uploaded.

BandWagon:\configuration\security\users>

show, add, modify, delete, reset, clear, upload, download

Figure 165 - BandWagon example: \configuration\security\user\upload

->> upload 192.10.1.1:\etc\users.ras

Uploading User Configuration  to file \etc\users.ras on host 192.10.1.1

BandWagon:\configuration\security\users>

show, add, modify, delete, reset, clear, upload, download

#### \configuration\security\user\download

Figure 166 - BandWagon's definition: \configuration\security\user\download

->> ? download

DOWNLOAD                                                                DOWNLOAD

SYNOPSIS

download     [host:][filename] [<AccessCode>]

DESCRIPTION

Download User Configuration information from the specified file on

the indicated host

OPTIONS

[host:]      indicates the remote host to download from. If it is not

specified, the boot server will be selected

[filename]   Filename for downloading

Default is /etc/ras/users.ras

[AccessCode] Indicates AccessCode whose details are to be downloaded.

BandWagon:\configuration\security\users>

show, add, modify, delete, reset, clear, upload, download

Figure 167 - BandWagon example: \configuration\security\user\download

->> download

Downloading User Configuration  from file /etc/ras/users.ras on host 192.10.1.1

/etc/ras/users.ras: Error code 0x1 File not found

BandWagon:\configuration\security\users>

show, add, modify, delete, reset, clear, upload, download

->> download 192.10.1.1:\etc\users.ras

Downloading User Configuration  from file \etc\users.ras on host 192.10.1.1

BandWagon:\configuration\security\users>

show, add, modify, delete, reset, clear, upload, download

->> show

--------------------------------------------------------------------

AccessCode      User ID         Authentication  Login-Session

--------------------------------------------------------------------

root            root            CR2             Admin

willie          willie          PT              Lpath

bush            bush            PT              Lpath

test            tesdt           PT              Lpath

ana             ana             CR2             Lpath

admin           admin           CR2             Admin

BandWagon:\configuration\security\users>

show, add, modify, delete, reset, clear, upload, download

In the example above the upload command is issued without any arguments.  Notice the RAS tries to download the file from the default host and default path \etc\ras\users.ras.  Since the file was not uploaded to the default directory, the command was issued specifying the host and file name with the path used in the upload example.  The show command is used to verify that the users database was received successfully.  The upload and download commands can also be used to upload and download specific user's names and accounts.

### \configuration\security\sas (security access server)

The \configuration\security\sas provides the command to show, add, modify, delete, and clear the Security Access Server database.  These commands allows the administrator to create and maintain a SAS database.

#### \configuration\security\sas\show

Figure 168 - BandWagon's definition:  \configuration\security\sas\show

->> ? show

SHOW                                                             SHOW

SYNOPSIS

show         [<SAS Name>]

DESCRIPTION

Displays the SAS database

Without argument, displays SAS database summary

OPTIONS

<SAS Name>  Indicates the name of the SAS whose details are to be shown.

#### \configuration\security\sas\add

Figure 169 - BandWagon example:  \configuration\security\sas\add

BandWagon:\configuration\security\sas>

show, add, modify, delete, clear

->> add 192.10.1.1

-----------------------------------------------------------------------

Parameter                                   Current Value    New value

-----------------------------------------------------------------------

Authentication Protocol(Radius/Tacacs/Both) R                b

Shared Secret                               -                ***

Please re-enter                             ***              ***

Edit, Abort, Save : s

->> show

--------------------------------------

SAS Name             Protocol

--------------------------------------

192.10.2.1           RADIUS & TACACS

192.10.1.1           RADIUS & TACACS

BandWagon:\configuration\security\sas>

show, add, modify, delete, clear

In the example above the add commands is used to add a SAS host with address 192.10.2.1.  The show command is used to verify the entry is added correctly.

#### \configuration\security\sas\modify

Figure 170 - BandWagon example:  \configuration\security\sas\modify

BandWagon:\configuration\security\sas>

show, add, modify, delete, clear

->> modify 192.10.2.1

-----------------------------------------------------------------------

Parameter                                   Current Value    New value

-----------------------------------------------------------------------

Authentication Protocol(Radius/Tacacs/Both) Both             R

Shared Secret                               -                ***

Please re-enter                             ***              ***

Edit, Abort, Save : s

BandWagon:\configuration\security\sas>

show, add, modify, delete, clear

->> show

--------------------------------------

SAS Name             Protocol

--------------------------------------

192.10.2.1           RADIUS

192.10.1.1           RADIUS & TACACS

#### \configuration\security\sas\delete

Figure 171 - BandWagon example:  \configuration\security\sas\delete

BandWagon:\configuration\security\sas>

show, add, modify, delete, clear

->> show

--------------------------------------

SAS Name             Protocol

--------------------------------------

192.10.2.1           RADIUS

192.10.1.1           RADIUS & TACACS

BandWagon:\configuration\security\sas>

show, add, modify, delete, clear

->> delete 192.10.1.1

SAS entry for host '192.10.1.1' deleted

BandWagon:\configuration\security\sas>

show, add, modify, delete, clear

->> show

--------------------------------------

SAS Name             Protocol

--------------------------------------

192.10.2.1           RADIUS

#### \configuration\security\sas\clear

->> show

--------------------------------------

SAS Name             Protocol

--------------------------------------

192.10.2.1           RADIUS

BandWagon:\configuration\security\sas>

show, add, modify, delete, clear

->> clear

This command will clear the whole SAS database.

Proceed(y/n)(n)? y

Cleared SAS database

BandWagon:\configuration\security\sas>

show, add, modify, delete, clear

->>

BandWagon:\configuration\security\sas>

show, add, modify, delete, clear

->> show

--------------------------------------

SAS Name             Protocol

--------------------------------------

BandWagon:\configuration\security\sas>

### \configuration\security\audittrial

The audit trial database provides the system administrator with information about the usage of the BandWagon.  The audit trial tells how frequently users are dialing in to the BandWagon, status of their calls, and any administrative changes done to the configuration of the system.

The administrator can show, flush and configure the audit trial database.  In the configure directory the administrator can enable sending audit trial program to the Windows based WinRAS program, it can print the audit trial to a specific port and it can upload the audit trial to a specific host:filename path.

#### \configuration\security\audittrial\show

When the ..\show command is issued the administrator can view a listing of all audit records, displayed in spreadsheet-style format.  Each record is displayed in a separate row.

Figure 172 - BandWagon example:  \configuration\security\audittrial\show

BandWagon:\configuration\security\audittrail>

show, flush, configure

->> show

----------------------------------------------------------------------------

Identifier     Port Event  Start Time            End Time              Status

-----------------------------------------------------------------------------

root            CO  Login  Feb  4, 1996 12:09:14 -                     JCR

barbara         -   UsrDel Feb  4, 1996 12:40:40 -                     -

ana             -   UsrMod Feb  4, 1996 12:43:45 -                     -

* DOWNLOADING * -   UsrDow Feb  4, 1996 14:09:45 -                     -

* CLEARING *    -   UsrClr Feb  4, 1996 14:10:07 -                     -

* DOWNLOADING * -   UsrDow Feb  4, 1996 14:18:19 -                     -

bryan           -   UsrAdd Feb  4, 1996 15:51:49 -                     -

willie..........A8..Login  Feb  4, 1996 16:15:35 Feb  4, 1996  16:4515 -PT

------------------------------------------------------------------------------

BandWagon:\configuration\security\audittrail>

show, flush, configure

Mode field codes:

- Login - User logging into the RAS

- Logout - User is logging out

- Rlogin - User is remote logging into the RAS

- Rlogot - User is logging out after a remote login

- UsrAdd -Administrator has added a new user entry   UsrDel - Administrator has deleted a user entry

- UsrDel - Administrator deleted a user entry

- UsrMod -Administrator has modified a user entry   UsrUpl - Administrator has uploaded the user database   UsrDow - 	Administrator has downloaded the user database

- UsrUpl - Administrator uploaded the user database

- UsrDw - Administration downloaded the user database

- UsrClr - Administrator has cleared the user database from the NVRAM

- PathSt - A Path has been started

- PathSp -Path has been stopped

- PaliSt - Line of a path has been brought up PaliSp - Line of a path has been brought down

Mode field codes:  Continued.

- PaliSp - Line of a path has been brought down

- Unknown - Unknown event

Possible Status types and their meanings.

- FAC  - False Access Code

- CBI - Call back Incomplete NPWD - Password Error

- NPWD - Password error

- FRSP - False Response

- NRSP - No response from the BandWagon

- AB - Process Aborted

- IU   - User Already In Use LCL  - User is not a Local User

- LCL - User is not a local user

- PT   - Pass Through User

- ICR  - Info card I Challenge Response User

- JCR - Infocard II challenge Response User

- IRO - Single Button Infer Card I Response Only user

- JRO - Single Button Info card II Response Only user

- IMRO - Multiple Button Infer card I Response Only user

- JMRO - Multiple Button Infocard II Response Only user

- CB   - Fixed Call back user (followed by call back number)

- VC   - Variable call back (followed by call back number)

- EXT  -External user

#### \configuration\security\audittrial\flush

Figure 173 - BandWagon's definition:  \configuration\security\audittrial\flush

BandWagon:\configuration\security\audittrail>

show, flush, configure

->> ? flush

FLUSH                                                                  FLUSH

SYNOPSIS

flush

DESCRIPTION

Flushes the Audit Trail database.

OPTIONS

none

BandWagon:\configuration\security\audittrail>

show, flush, configure

#### \configuration\security\audittrial\configure

Configures the audit trial operation to enable reports to the WinRAS, print reports to a port, or upload the audittrial database to a host\filename.

##### \configuration\security\audittrial\configure\loadpc

Figure 174 - BandWagon's definition:  \configuration\security\audittrail\configure\loadpc

BandWagon:\configuration\security\audittrail>

show, flush, configure

->> configure

BandWagon:\configuration\security\audittrail\configure>

loadpc, print, upload

->> ? loadpc

LOADPC                                                                LOADPC

SYNOPSIS

loadpc       [enable|disable]

DESCRIPTION

Enable/disable Audit Trail upload to WinRAS. If no argument

is given, the current status is shown

OPTIONS

enable      Uploading is enabled

disable     Uploading is disabled

##### \configuration\security\audittrial\configure\print

Figure 175 - BandWagon's definition:  \configuration\security\audittrail\configure\print

BandWagon:\configuration\security\audittrail\configure>

loadpc, print, upload

->> ? print

PRINT                                                                  PRINT

SYNOPSIS

print        [<Port Name>|disable]

DESCRIPTION

Configure Audit Trail print on the port specified. If no argument

is given, the current status is shown

OPTIONS

<Port Name>  Name of the port where the printer is connected.

disable      Printing is disabled

BandWagon:\configuration\security\audittrail\configure>

##### \configuration\security\audittrial\configure\upload

Figure 176 - BandWagon's definition:  \configuration\security\audittrail\configure\upload

BandWagon:\configuration\security\audittrail\configure>

loadpc, print, upload

->> ? upload

UPLOAD                                                                 UPLOAD

SYNOPSIS

upload       [[host:]<hostpath>|disable]

DESCRIPTION

Configure Audit Trail Upload to the specified path on the indicated

host. If no argument is given, the current status is shown

OPTIONS

[host:]     Indicates the remote host to upload. If it is not

specified, the boot server will be selected

<hostpath>  hostpath can be a file or a directory. The hostpath is

concatenated with the timestamp to generate an audit

file on the host. If hostpath is directory, the audit

files are created in that directory. If hostpath is

not a directory, it will be the prefix to the created

audit filenames.

eg: upload aranya:/tmp/

Creates audit trail files in /tmp of aranya. If upload

takes places at "20 Aug 1995 10:10:10", the file

/tmp/95820101.010 will be created in aranya(host).

disable     Disables uploading to host

BandWagon:\configuration\security\audittrail\configure>

loadpc, print, upload

The filename format has been designed to account for DOS file name convention.  The file is in yymmdd format, so that the file list of files, when sorted lexically, will be in increasing order of time.  The us mmddyy or the UK ddmmyy format will result in random lexical ordering in terms of chronology.

Notice that the month is only a single digit, and represented in hexadecimal notation to overcome the DOS eight and three characters format.  In this case October is represented as 'a', November as 'b', and December as 'c'.

Figure 177 - BandWagon example:  \configuration\security\audittrail\configure\upload

->> upload disable

Audit Trail Uploading to Host Disabled.

BandWagon:\configuration\security\audittrail\configure>

loadpc, print, upload

->> upload enable

Audit Trail Uploading to Host Enabled.

BandWagon:\configuration\security\audittrail\configure>

loadpc, print, upload

->> upload 192.10.1.1:\etc

Audit Trail Uploading to Host Enabled.

BandWagon:\configuration\security\audittrail\configure>

loadpc, print, upload

->> upload 192.10.1.1:\etc

Audit Trail Uploading to Host Enabled.

BandWagon:\configuration\security\audittrail\configure>

loadpc, print, upload

->> upload

Audit Trail Uploading to Host : 192.10.1.1:\etc

Note: Leemah  	I tried the commands above and it says that it is uploading but it did not upload anything to the specified host directory.

### \configuration\modemchats

The BandWagon downloads modem control files from the boot server during the boot process.  The \configuration\modemchats directory provides the necessary commands to manage the modemchats file.

#### \configuration\modemchats\show

Figure 178 - BandWagon example:  \configuration\modemchats\show

BandWagon:\configuration\modemchats>

show, add, delete, clear, upload, download

->> show

-----------------------------------------------------------------------------

Chat Name               Chat Script

-----------------------------------------------------------------------------

FasTraq                \r  AT&F\r OK ATQ0\r OK ATE1\r OK ATV1\r OK AT&D2\r OK AT&C1&R1&L0\\G1\\Q2\r OK ATS0=0\r OK AT\r OK ATDT$t\r CONNECT  \r\n

_FasTraq               \d\r  AT&F\r \dOK ATQ0\r \dOK AT&D2\r \dOK AT&C1&R1&L0\\G

1\\Q2\r \dOK ATS0=1\r \dOK ATS2=0\r \dOK AT\r \dOK ATE0\r \dOK ATQ1\r \d

MT1432PCS              \r  AT&FX4\r OK AT&E1&E4\r OK ATS7=60S0=0E1&B\r OK AT\r O

K ATDT$t\r CONNECT  \r\n

_MT1432PCS     \r  AT&FX4\r OK AT&E1&E4\r OK ATS7=60S0=1E1&B\r OK ATE0Q1

Radicom        \r  AT&F0\r OK ATQ0\r OK ATE1\r OK ATV1\r OK AT&D2\r OK A

T&C1&K3\\G1\r OK ATS0=0\r OK AT\r OK ATDT$t\r CONNECT  \r\n

_Radicom       \d\r  AT&F0\r \dOK ATQ0\r \dOK AT&D2\r \dOK AT&C1&K3\\G1\

r \dOK ATS0=1\r \dOK ATS2=0\r \dOK AT\r \dOK ATE0\r \dOK ATQ1\r \d\n

FasTraqSec     \r \d ATQ0S0=0\r \dOK ATDT$t\r CONNECT  \r\n

_FasTraqSec    \d\r \d ATQ0S0=1\r \dOK ATQ1\r \d\n

HayesAcu       \r  AT&F\r OK ATQ0\r OK ATE1\r OK ATV1\r OK AT&D2\r OK AT

&C1\r OK AT&K3\r OK ATS0=0\r OK AT\r OK ATDT$t\r CONNECT  \r\n

_HayesAcu      \d\r  AT&F\r \dOK ATQ0\r \dOK AT&D2\r \dOK AT&C1\r \dOK A

T&K3\r \dOK ATS0=1\r \dOK ATS2=0\r \dOK AT\r \dOK ATE0\r \dOK ATQ1\r \d\n

BandWagon:\configuration\modemchats>

show, add, delete, clear, upload, download

In the example above the modemchat can be read as follows:

The \d stands for a 50-milliseconds delay, the \r stands for an enter or carriage return (CR) key, the \ sends a space character to the modem, and \n ends the script.  The chatscript syntax consists of Send-Expect strings, separated by SPACES.  The chat script is executed by the RAS sending the Send string and looking for the Expect string from the modem.  In the example below:

_FasTraqSec    \d\r \d ATQ0S0=1\r \dOK ATQ1\r \d\n

The RAS delays 50 milli seconds,  sends a CR to the modem.  It waits another 50 milli seconds and sends the string ATQ0S0=1 a CR and waits for another 50 milli seconds.  It then waits for the OK string to return back from the modem to send the ATQ1 and a CR.  The actual execution of the chatscript will be very fast, in the order of a few seconds.  However if the modem goes off and on, the chatscript will fail, it will be retried after a 3 minutes time-out.  The command\administration\reset can be used to reset the port.

#### \configuration\modemchats\add

Figure 179 - BandWagon's definition: \configuration\modemchats\add

BandWagon:\configuration\modemchats>

show, add, delete, clear, upload, download

->> ? add

ADD                                                                       ADD

SYNOPSIS

add          [host:]<filename>

DESCRIPTION

Adds the chats specified in the file to the modemchats database

OPTIONS

[host:]      Indicates the remote host to download from. If it

is not specified, the boot server will be selected

<filename>   Name of file containing chats to be added

Figure 180 - BandWagon example:  \configuration\modemchats\add

->> add 192.10.1.1:\etc\ras\null.scr

Adding chat info (file \etc\ras\null.scr) from host 192.10.1.1

added chat info: 'null_modem'

->> show

-----------------------------------------------------------------------------

Chat Name               Chat Script

-----------------------------------------------------------------------------

FasTraq        \r  AT&F\r OK ATQ0\r OK ATE1\r OK ATV1\r OK AT&D2\r OK AT

&C1&R1&L0\\G1\\Q2\r OK ATS0=0\r OK AT\r OK ATDT$t\r CONNECT  \r\n

_FasTraq       \d\r  AT&F\r \dOK ATQ0\r \dOK AT&D2\r \dOK AT&C1&R1&L0\\G

1\\Q2\r \dOK ATS0=1\r \dOK ATS2=0\r \dOK AT\r \dOK ATE0\r \dOK ATQ1\r \d

MT1432PCS      \r  AT&FX4\r OK AT&E1&E4\r OK ATS7=60S0=0E1&B\r OK AT\r O

K ATDT$t\r CONNECT  \r\n

_MT1432PCS     \r  AT&FX4\r OK AT&E1&E4\r OK ATS7=60S0=1E1&B\r OK ATE0Q1

Radicom        \r  AT&F0\r OK ATQ0\r OK ATE1\r OK ATV1\r OK AT&D2\r OK A

T&C1&K3\\G1\r OK ATS0=0\r OK AT\r OK ATDT$t\r CONNECT  \r\n

_Radicom       \d\r  AT&F0\r \dOK ATQ0\r \dOK AT&D2\r \dOK AT&C1&K3\\G1\

r \dOK ATS0=1\r \dOK ATS2=0\r \dOK AT\r \dOK ATE0\r \dOK ATQ1\r \d

FasTraqSec     \r \d ATQ0S0=0\r \dOK ATDT$t\r CONNECT  \r\n

_FasTraqSec    \d\r \d ATQ0S0=1\r \dOK ATQ1\r \d

HayesAcu       \r  AT&F\r OK ATQ0\r OK ATE1\r OK ATV1\r OK AT&D2\r OK AT

&C1\r OK AT&K3\r OK ATS0=0\r OK AT\r OK ATDT$t\r CONNECT  \r\n

_HayesAcu      \d\r  AT&F\r \dOK ATQ0\r \dOK AT&D2\r \dOK AT&C1\r \dOK A

T&K3\r \dOK ATS0=1\r \dOK ATS2=0\r \dOK AT\r \dOK ATE0\r \dOK ATQ1\r \d

null_modem                     \d\n

BandWagon:\configuration\modemchats>

show, add, delete, clear, upload, download

Notice that the null_modem string was added to the database.

#### \configuration\modemchats\delete

Figure 181 - BandWagon's definition:  \configuration\modemchats\delete

BandWagon:\configuration\modemchats>

show, add, delete, clear, upload, download

->> ? delete

DELETE                                                                 DELETE

SYNOPSIS

delete       <chat_name>

DESCRIPTION

Deletes a modemchat entry from the modemchats database

OPTIONS

none

BandWagon:\configuration\modemchats>

show, add, delete, clear, upload, download

Figure 182 - BandWagon example:  \configuration\modemchats\delete

->> delete null_modem

chat name 'null_modem' deleted

BandWagon:\configuration\modemchats>

show, add, delete, clear, upload, download

->> show

-----------------------------------------------------------------------------

Chat Name               Chat Script

-----------------------------------------------------------------------------

FasTraq        \r  AT&F\r OK ATQ0\r OK ATE1\r OK ATV1\r OK AT&D2\r OK AT

&C1&R1&L0\\G1\\Q2\r OK ATS0=0\r OK AT\r OK ATDT$t\r CONNECT  \r\n

_FasTraq       \d\r  AT&F\r \dOK ATQ0\r \dOK AT&D2\r \dOK AT&C1&R1&L0\\G

1\\Q2\r \dOK ATS0=1\r \dOK ATS2=0\r \dOK AT\r \dOK ATE0\r \dOK ATQ1\r \d

MT1432PCS      \r  AT&FX4\r OK AT&E1&E4\r OK ATS7=60S0=0E1&B\r OK AT\r O

K ATDT$t\r CONNECT  \r\n

_MT1432PCS     \r  AT&FX4\r OK AT&E1&E4\r OK ATS7=60S0=1E1&B\r OK ATE0Q1

Radicom        \r  AT&F0\r OK ATQ0\r OK ATE1\r OK ATV1\r OK AT&D2\r OK A

T&C1&K3\\G1\r OK ATS0=0\r OK AT\r OK ATDT$t\r CONNECT  \r\n

_Radicom       \d\r  AT&F0\r \dOK ATQ0\r \dOK AT&D2\r \dOK AT&C1&K3\\G1\

r \dOK ATS0=1\r \dOK ATS2=0\r \dOK AT\r \dOK ATE0\r \dOK ATQ1\r \d

FasTraqSec     \r \d ATQ0S0=0\r \dOK ATDT$t\r CONNECT  \r\n

_FasTraqSec    \d\r \d ATQ0S0=1\r \dOK ATQ1\r \d

HayesAcu       \r  AT&F\r OK ATQ0\r OK ATE1\r OK ATV1\r OK AT&D2\r OK AT

&C1\r OK AT&K3\r OK ATS0=0\r OK AT\r OK ATDT$t\r CONNECT  \r\n

_HayesAcu      \d\r  AT&F\r \dOK ATQ0\r \dOK AT&D2\r \dOK AT&C1\r \dOK A

T&K3\r \dOK ATS0=1\r \dOK ATS2=0\r \dOK AT\r \dOK ATE0\r \dOK ATQ1\r \d

BandWagon:\configuration\modemchats>

show, add, delete, clear, upload, download

Notice that the null_mode script was removed from the modemchats database.

#### \configuration\modemchats\upload

Figure 183 - BandWagon's definition:  \configuration\modemchats\upload

BandWagon:\configuration\modemchats>

show, add, delete, clear, upload, download

->> ? upload

UPLOAD                                                                UPLOAD

SYNOPSIS

upload       [host:]<filename>

DESCRIPTION

Upload chats information to the specified file on the

indicated host

OPTIONS

[host:]       Indicates the remote host to upload. If it is not

specified, the boot server will be selected

<filename>    Filename for uploading

Figure 184 - BandWagon example:  \configuration\modemchats\upload

BandWagon:\configuration\modemchats>

show, add, delete, clear, upload, download

->> upload \etc\ras\modembak.scr

Uploading Modem Info  to file \etc\ras\modembak.scr on host 192.10.1.1

BandWagon:\configuration\modemchats>

show, add, delete, clear, upload, download

BandWagon:\configuration\modemchats>

show, add, delete, clear, upload, download

->> upload 192.10.1.1:\etc\modembak.scr

Uploading Modem Info  to file \etc\modembak.scr on host 192.10.1.1

BandWagon:\configuration\modemchats>

Notice that in the first example, the file is downloaded to the default boot server to the specified directory.  In the second example a host is explicitly defined.

#### \configuration\modemchats\clear

Figure 185 - BandWagon's definition:  \configuration\modemchats\clear

BandWagon:\configuration\modemchats>

show, add, delete, clear, upload, download

->> ? clear

CLEAR                                                                 CLEAR

SYNOPSIS

clear

DESCRIPTION

Clears the modemchats database.

OPTIONS

none

BandWagon:\configuration\modemchats>

show, add, delete, clear, upload, download

Figure 186 - BandWagon example:  \configuration\modemchats\clear

->> clear

This command will clear the modem chat information database.

Proceed(y/n)(n)? y

clearing modem chats database

BandWagon:\configuration\modemchats>

show, add, delete, clear, upload, download

->>

->> show

-----------------------------------------------------------------------------

Chat Name               Chat Script

-----------------------------------------------------------------------------

BandWagon:\configuration\modemchats>

show, add, delete, clear, upload, download

#### \configuration\modemchats\download

Figure 187 - BandWagon's definition:  \configuration\modemchats\download

BandWagon:\configuration\modemchats>

show, add, delete, clear, upload, download

->> ? download

DOWNLOAD                                                          DOWNLOAD

SYNOPSIS

download     [host:][filename]

DESCRIPTION

Download modemchats information from the specified file on the

indicated host

OPTIONS

[host:]       Indicates the remote host to download from. If it is not

specified, the boot server will be selected

[filename]    Filename for downloading

Default is /etc/ras/modem.scr

BandWagon:\configuration\modemchats>

show, add, delete, clear, upload, download

Figure 188 - BandWagon example:  \configuration\modemchats\download

->> download 192.10.1.1:\etc\ras\modembak.scr

Downloading Modem Info  from file \etc\ras\modembak.scr on host 192.10.1.1

downloaded chat info: 'FasTraq'

downloaded chat info: '_FasTraq'

downloaded chat info: 'MT1432PCS'

downloaded chat info: '_MT1432PCS'

downloaded chat info: 'Radicom'

downloaded chat info: '_Radicom'

downloaded chat info: 'FasTraqSec'

downloaded chat info: '_FasTraqSec'

downloaded chat info: 'HayesAcu'

downloaded chat info: '_HayesAcu'

BandWagon:\configuration\modemchats>

### \configuration\logchats

Logchats are scripts used by the RAS to dial out to another system using a PPP connection as a result of the \administration\path\start -v 10 command.  It tells the RAS what to do when it receives the login:prompt from the remote system.

#### \configuration\logchats\show

Figure 189 - BandWagon's definition:  \configuration\logchats\show

BandWagon:\configuration\logchats>

show, add, delete, clear, upload, download

->> ? show

SHOW                                                                        SHOW

SYNOPSIS

show

DESCRIPTION

Displays the Logchat information database

OPTIONS

none

BandWagon:\configuration\logchats>

show, add, delete, clear, upload, download

Figure 190 - BandWagon example:  \configuration\logchats\show

BandWagon:\configuration\logchats>

show, add, delete, clear, upload, download

->> show

-----------------------------------------------------------------------------

Chat Name               Chat Script

-----------------------------------------------------------------------------

raslogin          Login: $u Password: $p Login\ Successful\r\n

pclogin        \r Login: $u Password: $p Login\ Successful\r\n

n\ Successful\r\n

BandWagon:\configuration\logchats>

show, add, delete, clear, upload, download

The logchat script interpretation and syntax is similar to that of the modem chat scripts.  The $u stands for the UserName specified in the path management directory.  The $p stands for the Password as specified in the path management directory.  The $cChallenge:$c instructs the RAS to wait for the challenge from the remote RAS.  The $r stands for the response which the calling RAS will calculate from the received Challenge (making use of the Site key and Userkey stored in the path management).  The LAN-to-LAN (a RAS connecting to another RAS) always uses CR-II authentication.

The "pclogin" is interpreted as follows:  send a CR and wait for the Login: prompt.  Send the UserName specified in the path management.  Wait for Password: prompt from the remote host to send the password as specified in the path management.  Wait for the "Login ... Successful" key words to return from the host, to send a CR and end the script.

#### \configuration\logchats\add

Figure 191 - BandWagon's definition:  \configuration\logchats\add

BandWagon:\configuration\logchats>

show, add, delete, clear, upload, download

->> ? add

ADD                                                                      ADD

SYNOPSIS

add          [host:]<filename>

DESCRIPTION

Adds the chats specified in the file to the logchats database

OPTIONS

[host:]      Indicates the remote host to download from. If it

is not specified, the boot server will be selected

<filename>   Name of file containing chats to be added

BandWagon:\configuration\logchats>

show, add, delete, clear, upload, download

Figure 192 - BandWagon example:  \configuration\logchats\add

show, add, delete, clear, upload, download

->> show

-----------------------------------------------------------------------------

Chat Name               Chat Script

-----------------------------------------------------------------------------

raslogin              Login: $u Password: $p Login\ Successful\r\n

pclogin            \r Login: $u Password: $p Login\ Successful\r\n

BandWagon:\configuration\logchats>

show, add, delete, clear, upload, download

->> add 192.10.1.1:\etc\ras\mychat.scr

Adding chat info (file \etc\ras\mychat.scr) from host 192.10.1.1

added chat info: 'seclogin'

BandWagon:\configuration\logchats>

show, add, delete, clear, upload, download

->> show

-----------------------------------------------------------------------------

Chat Name               Chat Script

-----------------------------------------------------------------------------

raslogin            Login: $u Password: $p Login\ Successful\r\n

pclogin          \r Login: $u Password: $p Login\ Successful\r\n

seclogin            Login: $u Password: \r $cChallenge:$c\r\nResponse: $r Logi

n\ Successful\r\n

BandWagon:\configuration\logchats>

show, add, delete, clear, upload, download

In the example above the "show" command is used to look at the logchat database.  The file \etc\ras\mychat.scr, containing the seclogin script, is added to the logchat database.  The second show command verifies that the seclogin script entry was added successfully.

#### \configuration\logchats\delete

The delete command deletes specific chat scripts from the logchat database.

Figure 193 - BandWagon example:  \configuration\logchats\delete

BandWagon:\configuration\logchats>

show, add, delete, clear, upload, download

->> delete seclogin

chat name 'seclogin' deleted

BandWagon:\configuration\logchats>

show, add, delete, clear, upload, download

->> show

-----------------------------------------------------------------------------

Chat Name               Chat Script

-----------------------------------------------------------------------------

raslogin              Login: $u Password: $p Login\ Successful\r\n

pclogin                \r Login: $u Password: $p Login\ Successful\r\n

BandWagon:\configuration\logchats>

show, add, delete, clear, upload, download

In the example above the delete command is used to remove the seclogin chat script from the logchat database.

#### \configuration\logchats\clear

Figure 194 - BandWagon example:  \configuration\logchats\clear

show, add, delete, clear, upload, download

->> show

-----------------------------------------------------------------------------

Chat Name               Chat Script

-----------------------------------------------------------------------------

raslogin           Login: $u Password: $p Login\ Successful\r\n

pclogin         \r Login: $u Password: $p Login\ Successful\r\n

BandWagon:\configuration\logchats>

show, add, delete, clear, upload, download

->> clear

This command will clear the logchat information database.

Proceed(y/n)(n)? y

clearing logchats database

BandWagon:\configuration\logchats>

show, add, delete, clear, upload, download

->> show

-----------------------------------------------------------------------------

Chat Name               Chat Script

-----------------------------------------------------------------------------

BandWagon:\configuration\logchats>

show, add, delete, clear, upload, download

#### \configuration\logchats\download

Figure 195 - BandWagon example:  \configuration\logchats\download

show, add, delete, clear, upload, download

->> show

-----------------------------------------------------------------------------

Chat Name               Chat Script

-----------------------------------------------------------------------------

BandWagon:\configuration\logchats>

show, add, delete, clear, upload, download

->> download 192.10.1.1:\etc\ras\logchat.scr

Downloading Login chat  from file \etc\ras\logchat.scr on host 192.10.1.1

downloaded chat info: 'raslogin'

downloaded chat info: 'pclogin'

BandWagon:\configuration\logchats>

show, add, delete, clear, upload, download

->> show

-----------------------------------------------------------------------------

Chat Name               Chat Script

-----------------------------------------------------------------------------

raslogin              Login: $u Password: $p Login\ Successful\r\n

pclogin            \r Login: $u Password: $p Login\ Successful\r\n

BandWagon:\configuration\logchats>

show, add, delete, clear, upload, download

#### \configuration\logchats\upload

Figure 196 - BandWagon example:  \configuration\logchats\upload

->> show

------------------------------------------------------------------------

Chat Name               Chat Script

------------------------------------------------------------------------

raslogin              Login: $u Password: $p Login\ Successful\r\n

pclogin                \r Login: $u Password: $p Login\ Successful\r\n

BandWagon:\configuration\logchats>

show, add, delete, clear, upload, download

->> upload 192.10.1.1:\etc\ras\logchat.bak

Uploading Login chat  to file \etc\ras\logchat.bak on host 192.10.1.1

BandWagon:\configuration\logchats>

show, add, delete, clear, upload, download

In the example above the "show" command is used to verify the validity of the logchat database entries.  The upload command is used to upload the logchat.bak backup file to the host 192.10.1.1 at the \etc\ras subdirectory.

# Chapter 5.  Configuration Guidelines

There are many ways that the user administrator can begin a configuration session.  These guidelines are intended to give an idea on how to start the process.  You may decide to start in a different way and that may very well be fine.  If you fell that way after reading these guidelines then, you should go ahead and follow your approach.  The guidelines are not step-by-step instructions on how to set the BandWagon.  It is very difficult to design such a set of instructions since the BandWagon can be configured in many different ways.  The steps provided below are general in nature, it will guide you in the process.

The first step in configuring your BandWagon configuration is to try standardize in the way your users will access the system.  It is recommended that the users standardize in the minimum amount of modems.  This will minimize the number of modem chats that you may have to minimize and maintain.  Once you decide in the number of modems that you will standardize, begin by writing the initialization strings to be used.

Try to stay within the following recommendations:

1.  Use secure modems as much as possible.  LeeMah Datacom has a line of state-of-the art modems with built-in security.  These include challenge response modems such as the FasTrak and TraKnet modem lines.  They support V.42 compression, and V32/V34 standards, with MNP level 5.

2.  It is recommended to program the modems to keep constant DTE speed.  Set at leas three pools of modems, at different speeds.  The first pool shall be set at 38.4 Kbps, the second at 57.6 Kbps, and the third one at 115.2 Kbps.  Program the modems for hardware flow control and for auto-answer for the Dial-in modem pool.  This will allow users to try from the highest speed to the lowest, until they find the connection that will work for them.  The reason for this is that different users may have difficulties in connecting at higher speeds because their calls may be routed over noisy lines.  In addition some users may call using V32bis, V32, and V34 type modems and those speeds represent the top speeds for each class of modem.  Modify your modenchat scripts to enable MNP error correction, fixed DTE port, and disable auto-speed fallback.  Enable hardware flow control, and disable the software flow control.  The reason is that during file transfer the data stream may contain the Xon/Xoff characters and halt the modems during transfer.  Your telephone service provider can arrange to set-up modem lines in circular hunt groups.  Each hunt group can be set with their own listed number.  This will keep the dial numbers equal to the number of pools you have designed and not multiple modem numbers.

3.  If you have users that are going to use a BRI ISDN line, It is necessary to connect BRI ISDN or PRI WAN links

to the BandWagon since these users will not be able to access the modem pools.

4.  At this point you can set the BandWagon Asynchronous ports according to the modem pool characteristics that you have used.  It is also recommended to set the parity to none, and the character size to 8.  This will provide the best line efficiency.  The parity is not needed since the modem will be doing error detection and correction.

5.  Define the community of users that will be calling in and out of the BandWagon system.  Define the protocol suite that they will be using.  It is recommended to try to keep standard configurations.  Stay with one WAN protocol.  If possible standardize in the use of the IP protocol.  The IP protocol was designed for WAN applications.  Microsoft has acknowledge its intentions to standardize on the IP protocol.  You can download all the necessary drivers from the Microsoft bulletin board, or accessing their WWW Page at www.microsoft.com.  Configure the BandWagon Client software according to the protocol that you decided to standardize.

6.  Configure the BandWagon WAN protocols using the \configure\permanent\network ip, ipx or netbeui directories.  Be sure to program the IP, IPX and NetBeui timers as required.

7.  Define the classes of services to provided to your community of users.  Follow the recommendations in the section " Adding a Path" in the next section of this manual.  Program the configuration\paths that you will be using.

8.  Download the modem chats scripts and the logchats scripts if you are going to program a LAN=to-LAN connection.

9.  At this point in time you are ready to set your users accounts.  Follow the recommendations in the section "Adding a user" in the section ahead.

10.  Proceed to connect the modems to the BandWagon and test your configurations.

These steps are simple but they require you to access different directories in the BandWagon to complete the process.  Plan along these steps before you attempt to start the process.

## Adding a user

The Add command lets you add a new user definition to the database.

Enter the user's lD in the User ID field. in the Access Code field, enter the code the user will type in when they log on to the BandWagon.

The Duration of Connection field shows the maximum length (in seconds) for any of this user's sessions.  Normally, this field should be set to zero. This setting tells the BandWagon not to limit the connection time for this user. The connection can be of any length.  The user will disconnect the call at the end of the session.

Each user must have a Usage Type. Select one of the following options:

None:  Although the user is valid, the user is not entitled to any BandWagon sessions.  This setting lets you create a "place holder" for a user. At a later time, you can give the user another usage type.  You may also use this setting to temporarily stop a particular user from using the BandWagon without being required to delete the entry.

Admin:  This user has all administrative privileges.  For example only a user with this usage type can add, modify or delete user records.  While other types of users can use the BandWagon, they cannot configure the BandWagon.

Note:  Administrator accounts have more power;   therefore, they must be given tighter security.     You must set the authentication type to CR2     (InfoCard-Il with challenge-response mode).  A challenge alone is not enough. There must  always be a challenge/response sequence.

Path:  When a Path user logs in, the BandWagon will attempt to start the path specified in the user's Path Name field. (Paths contain port and configuration information which the BandWagon will use to bring up SLIP or PPP and connect with a remote system.

Lpath:  An Lpath user can log in to the BandWagon simply by dialing in to it. When this user ions in. the BandWagon will automatically start SLIP or PPP on the path specified in the user's Path Name field.

ACS:  An ACS user access ports configured as ACS resources.  A local LAN user who tries to use the NASI/INT14 dial out redirection, will need to be authenticated.  Normally only NASI users (LAN PCs running ASI.EXE) should be given this user type.

Enter the user's InfoCard cipher key in the Cipher Key field.  The Site Key field holds the InfoCard-II cipher Key

Enter the user's authentication type in the Security Mode field Choose from the following options:

- PT:  		Pass through.  Only password r authentication required

- CRl:  		I InfoCard-I with challenge-response mode.

- CR2:  		I InfoCard-II with challenge-response mode

- ROl:   		Single key InfoCard-I with response only mode.

- R02:   		Single key InfoCard-II with response only mode.

- MROI:		Multiple key InfoCard-I with response only mode.

- MR02:  	Muliple key InfoCard-II with response only mode.

The Password field holds the user's password

Use the Path Name field to enter the name of the path which will  be automatically started up when the user logs in.  This path should be already configured in the path information database.

The callback Type field tells the BandWagon whether to call a particular user back after that user logs in.  Use this field to assign each user one of these three callback types:

None:  No call back. The BandWagon will not return this user's calls.  This means that the user must pay any long-distance charges incurred while accessing the network.

Fixed:  Fixed call back.  The BandWagon can return this user's calls.. However, the BandWagon can only call a pre-specified number.   If you assign this callback type to a user, you will also need to specify the telephone number that the BandWagon will call.  Enter the user's telephone number in the Call-back Number Field.

Variable:  Variable call back.  The BandWagon can return this user's calls to any telephone number.  The user will enter the number before connecting to the BandWagon.  This option is specially useful for user that travel frequently.  The variable call back feature lets these users enter any telephone number  for the callback.  This can provide dramatic cost savings in long distance phone calls.

If you set a user's callback type to either Fixed or Variable, you will also need to fill in both the Retry Count field and the Delay Between Retries field.

The BandWagon server may not always be able to successfully call the user back.  This can happen if the wrong telephone number has been entered, or if the user has accepted another call on that telephone number.  The Retry Count field tells the BandWagon how many times it should attempt to call the user back before abandoning the process.  You may at Retry Count to any number   between 0 and 25; by default, this field is set to permit four retries.

If a call back attempt has been unsuccessful, the Delay Between Retries field tells the BandWagon how many seconds to wait before attempting another call back. This field may be set to any number between 20 and 9999 (167 minutes).  By default, it is set to 20 seconds.

## Adding a PATH

Enter the path name of your choice in the Path Name field.

The Data Compression field  indicates the type of compression the  BandWagon will use. Normally, this field is set to None; however, you may  also choose LZW (Leimp  Zie Wig) or RZE (Run Length Encoding)  compression.

If the data to be transmitted to the remote system is highly compressible, then  it may be advisable to enable data compression. However, the BandWagon will  negotiate with the remote end before actually doing the compression.  In  addition, most modems normally perform data compression dynamically as  data is sent.  It is strongly recommend that you use V.42 compatible modems.  These modems provide data compression as high as 4 to 1 ratios.  V.42 modems  provide for error correction during transmission.  These modems can achieve speeds up to 115,200 KBPS.  Programming the modems to a fixed DTE port speed set at this speed, and programming the BandWagon for RTS/CTS flow control, will provide better results.  In this case it is recommended to set the BandWagon compression to none.

The Idle Time-out field tells the BandWagon when it should assume that the  connection can be terminated.  If there is no traffic on the connection for the number of  minutes set in the Idle Time-out  field, the BandWagon will take the action  specified in the On Time-out Close field.  By default this field is set to zero. This setting tells the BandWagon that it  should never time-out the connection, even if there is no traffic.  However, you  may prefer to define a idle time-out period so that you can minimize the cost off  your communication media over WAN links. You may define an idle time-out  period between 1 and 21,474,836,638 minutes long.  Typically this value is set between 3 to -30 minutes.  If a BRI link is used at the other end you may want to set this timer as short as possible since most BRI lines rates are metered.

The Connect Type field tells the BandWagon how to bring up the path

F  Note:  This tells the BandWagon when to start performing PPP/SLIP  negotiation with the remote system. After successful negotiation, a  network interface will be created using the appropriate IP, IPX or  NetBEUI protocols. The BandWagon can then route packets over this  interface.

Choose one of the following options:

Demand:  Bring up the path on demand (manually).

Client:  Bring up the path when the BandWagon client connects. The BandWagon will not dial out to the client.  After the  client logs in to the BandWagon, the BandWagon will start negotiating a PPP/SLIP connection.

Persistent:  Bring up the path immediately after booting.  The BandWagon will not wait for a client to log in.  As soon as the BandWagon is powered on and enters multi-user mode, it will automatically dial-out to the remote system and begin negotiating the PPP/SLIP connection.

Use the Destination Type field to tell the BandWagon whether the path's destination is a Gateway or a Host.  Select Host if the destination system is a host system. Select Gateway if the destination system is actually a gateway that Will route your packet to another destination or network.  For example, you should select Gateway if this path is part of a LAN-to-LAN configuration, in which the BandWagon will be dialing-out to another BandWagon to reach a remote LAN.  Selecting Host will connect to a specific Host and made available any shared resources in the Host.  Users in the "calling" BandWagon will not see any other Hosts in the connected LAN.

The Spoofing field tells the BandWagon whether or not to send keep-alive packets to remote systems. By default, spoofing is set to None.  IP and IPX networks normally send periodic keep-alive packets to their peers on the LAN. However, when systems are connected remotely (for example, through telephone lines), these packets keep the line busy without actually doing useful work. If you have defined an idle time-out period (as described above), keep-alive packets force the BandWagon to keep the connection active even when there is no user traffic on the connection.  Under these conditions, spoofing should be triggered.  Spoofing will disconnect the line and send acknowledge packets to the local LAN.  This will fool hosts and gateways attached to the LAN to maintain their routing and services tables as if the link were still active.  The BandWagon will establish the connection as soon as a service request is received, or if a user has packets destined to the remote LAN.

FNote:      Modem chatscripts must be present in the modem chat database.  Please call customer support to add chatscripts and a modem chat database for a modem not currently supported.

Enter the name of the login chat you want to use in the Login Chat field. The User Name field contains the user name that will be used during the login chat, and the Password field contains the password that will be used during the login chat.

Enter the user's InfoCard cipher key in the User Key field. The Site Key field holds the InfoCard-II cipher key.

Port pools allow WinRAS to dial a second telephone number when  necessary. All the ports in a single port pool share the same telephone number.  If all the ports in the first pool are busy, the BandWagon will move to the  second pool. The BandWagon will repeat this process until it has cycled  through all 8 pools.  This allows the BandWagon to dial two different telephone numbers for each  Port.

All ports in a pool must be of the same type.  For example, one port pool might  include ports A4-A8, since these are all asynchronous.  Another port pool  might include ports 12-15, since these are all ISDN ports.  However, you cannot  define a port pool that includes both A2 and 13. Since one of these ports is asynchronous and the other is ISDN, this pool would be invalid.

Enter the name of the port pool in the Port Pool field. You may configure up  to 8 port pools.  If the ports in the pool are asynchronous, then you must enter the telephone number of a destination with an asynchronous modem in the Telephone Number field. If the ports are synchronous ports,  then you must enter a telephone number of a synchronous modem.

The Data Link Protocol field lets you choose between PPP, SLIP, and MLPP. By  default the BandWagon client uses PPP, so this field is also set to PPP by  default.

FNote: If you prefer to use SLIP,  the Clients Software or the remote BandWagon, must be also configured with a slip connection.

Specify the Use VJ Compression or not. Since most PPP implementations disable compression, this field is  disabled by default.  The VJ compression algorithm uses a hash table structure. if you enable VJ  compression, Maximum Slot Index tells the BandWagon  how many entries to use for  compression. You may specify any number between 1 and 256;  by default, this  index is set to 16.

FNote: For PP negotiations normally, you should have one VJ slot index for each TCP channel working over the PPP link. If you have more TCP channels than slots, then VJ compression will be less efficient. Therefore, you should configure the Maximum Slot index to match the number of TCP channels on your system.

If you have set the Data Link Protocol field on the Protocol dialog box to PPP, you will be prompted to configure additional PPP parameters.  The Negotiate Maximum Receive Unit field tells the BandWagon whether or not to negotiate the MRU. ("Maximum Receive Unit" refers to the maximum packet size that your PPP implementation can receive.) Since many PPP implementations don't need to negotiate MRU, this field should normally disabled.  If MRU negotiation is enabled, you will also need to set the MRU size. The MRU in Number of Octets field tells the BandWagon how many octets to allow in a PPP packet. Valid only if MRU negotiation is enabled. You may specify any number between 600 and 1500; by default, this field is set to 1500.

The Negotiate Asynchronous Map field tells the BandWagon whether ACCM negotiation should be enabled or disabled. ("ACCM' means "Async Control Character Map".) Normally, this field should be disabled.  If ACCM negotiation is enabled you will need to define the Async Control Character Map. This is a hexadecimal number between 00000000 and FFFFFFFF. By default, it is set to FFFFFFFF;  normally, there is no reason to change this parameter.

Authentication Protocol may be set to None, PAP (UPAP) or CHAP. (The terms "PAP" and "UPAP" both refer to "Password Authentication Protocol".)  The BandWagon uses password and security tokens to perform authentication while the user is logging in. If you also want to perform periodic user authentication, then CHAP may be enabled. However, this will increase line traffic, and may slow down the BandWagon's performance.

### IP connection

If you set the Data Link Protocol field on the Protocol choice. you can configure additional IP parameters.   If you want to connect to a network using IP host, then this field should be enabled.  Enter the IP address of the destination with which you want to connect in the Destination IP Address field.

### IPX

If  you have set the Data Link Protocol choice, then IPX Connectivity is enabled for this path.

Enter the IPX address of the destination with which you want to connect in the Destination IPX Address field.  You can leave the default of "*.*", and the BandWagon will find the correct address.

### NetBEUI

It may be necessary to modify the PROTOCOL.INI file, in the Windows Client PC to match the Parameters set in the path..  This is done by editing the Protocol.ini file in the \windows directory.  In addition it may be necessary to add the NETBIOSTIMEOUT=3000 and NETBIOSRETRIES=2 to the [NETBEUI] will slow down the Windows start-up time.  This is required in order to use the NETBEUI protocol over dial-up links.  These links are slow compared to normal Ethernet speeds.  Since NetBEUI is not a routable protocol, these parameters have to be changed.  The specified parameters should work in most cases.  If this is not the case, it is recommended to try different values until a satisfactory connection is obtained.  It will be necessary for the RAS administrator to set the corresponding values in the user's path to match the values used in the Client's PC.  Other parameters have to be added as indicated below.

If you have set the Data Link Protocol choice, then a NetBEUI connection is enabled for this path.

The DLC Timer Values fields let you set NetBEUI timers. Each of these three timers may be set to a value of one or more milliseconds.  The Delayed Acknowledgment (T2) timer defines the time interval during which the LLC shall either wait for an "Information Frame" to become available for transmission or accumulate additional information frames to be acknowledged in a single RR frame. (Received  Response frames are NetBEUI Supervisory frames, used to acknowledge Information frames.) By default this field is set to 1000; this is equivalent to one second.  Add the line "T2=1000" in Windows' the PROTOL.INI file, under the [NETBIOS] section.

The Retransmission (T1) timer defines the time interval during which the LLC ("Logical Link Control") will expect to receive an acknowledgment to one of more Information frames (NetBEUI data packets) or an expected response frame to a Unnumbered command PDU (NetBEUI datagram packet). By default, this field is set to 6000; this is equivalent to six seconds. Add the line "T1=6000" in the Windows' PROTOL.INI file, under the [NETBIOS] section.

The Inactivity (TI) timer tells the LLC how often to check an inactive link and verify whether it is operational.  By default, this field is set to 120000; this is equivalent to 120 seconds.

Caution: These timers must be set according to the following rule:

T2 <= T1 <= TI

In other words, the settings for these timers must have the following relationship:

Delayed Acknowledgment (T2) must be less than or equal to

Retransmission (T1) which must be less than or equal to

Inactivity TI timer)

You may want to change these timers, depending on the speed and efficiency of your PPP link. However, normally there is no need to change these settings.  In addition the PROTOCOL.INI file, in the Windows program running in the LAN's interface HOST, will need to be set as well.  The timers in the \configuration\permanent\network\netbios directory need to be changed accordingly.

Use the Send Window Size field to specify the number of LLC packets to send before expecting an acknowledgment. You may set this field to a value between 1 and 127; by default, it is set to 40.  Add the MAXOUT=40 parameter in Windows' PROTOCOL.INI file under the [NETBIOS] section.

The Receive Window Size field indicates the number of LLC packets to receive before sending an acknowledgment. You may set this field to a value between 1 and 127; by default, it is set to 40. .  Add the MAXIN=40 parameter in Windows' PROTOCOL.INI file under the [NETBIOS] section.

Use the Allow Name Traffic to enable or disable the flow of NetBEUI Name Datagrams to the path. (Name Datagrams are NetBEUI packets containing Name Query or Name Response supervisory frames.)  Normally, this field should be enabled.

The Allow Search Broadcast field tells the BandWagon whether to enable or disable the flow of NetBEUI Name-Query packets to the concerned path. (NetBEUI sends Name-Query packets, containing source and destination names, to verify whether a destination is alive; Name Response packets are the response to Name Query packets.) Normally, this field should be enabled.

## Setup the RAS

rwhodSend          This field decides if rwhod packets are to be broadcast on the network .This broadcast is performed every 60 seconds. It can be turned off to reduce LAN traffic.

routedSend          This field decides if the RAS should transmit routed (RIP) formation. Normally, RIP information should be transmitted only by nodes acting as a gateway.

ipforward            This field decides if the RAS can act as a gateway between two networks. Normally this field is set to YES, since the RAS acts as a router. However this field can be set to NO for diagnostic or other purposes.

tcpNodelack        The RAS generates acknowledgment packets in response to incoming TCP data. These ACK packets can be delayed for performance reasons to reduce network traffic. This field controls whether TCP acknowledgment should be delayed or immediate.

tcpKeepAlive     TCP can perform a keep alive function by sending TCP keep alive messages to the other end. This ensures that TCPchannels will be closed when the peer node aborts. This field decides whether the RAS should generate TCP keep alive messages for its TCP channels.

rtelnetloadBal ·    Reverse Telnet is useful in sharing devices like printers, modems etc. This field indicates which scheme is used to effect load  balancing among the shared devices.

### IPX Configuration

int net     The IPX network address advertised by this server as its internal network address.

local net   The local IPX network address of the LAN interface.

ether_type  Ethernet encapsulation type of the LAN interface

ipxrip_send Whether this machine should send IPX RIP broadcasts or not

ipxforward  Whether this machine should forward IPX from one network to another. Note that if this parameter is set to "no" this machine will CEASE TO FUNCTION AS A ROUTER AT ALL. However for diagnostic reasons you may want this set this parameter to "no"

### NetBEUI Configuration

T1 Timer            The T? timer defines the time interval during which the LLC shall expect to receive an acknowledgment to one or more outstanding Information frames or an expected response frame to a Unnumbered command PDU. This timer must obey the rule T2<= T1 <= TI

TZ Timer           The T2 timer defines the time interval during which the LLC shall wait for either an Information frame to become availabIe'for transmission or to accumulate additional Information frames to be acknowledged in a single RR frame. This timer must obey the rule T2<= T1 <= TI

TI Timer            This timer determines how often the LLC checks an inactive link to see if it is operational. This timer must obey the rule T2<= T1 <= TI send Window Size Indicates the number of LLC packets to send before expecting an acknowledgment

Receive Window Size    Indicates the number of LLC packets to receive before sending an acknowledgment

LAN WAN Threshold     Indicates the number of NetBEUI NAME QUERY retries from LAN before allowing them to be propagated to the WAN Links. This should typically not exceed 3.

### ISDN Configuration

isdnRasType           This field is used to configure the RAS as either an ISDN USER or an ISDN EXCHANGE

isdnlineType          This field is used to choose one of Line A or Line B of the ISDN Card.

### ACS Configuration

Prompt String         ACS prompt stn'ng. It can be up to 11 characters long.

MACS Port Id        The MACS port id in hexadecimal format. This id should be the same on all ACS (NASI) clients also.

### SNMP Configuration

syslocation           This field identifies the physical location of the RAS. The name of the location can be up to 31 characters long. The factory default is a null string.

sysContact            This field identifies the name of the person in charge of the RAS typically the network manager. The name can be up to 31 characters. The factory default is a null string.

authTraps             This field controls the enabling/disabling of authentication traps.  The factory default disables this field

trapDests              This option specifies the <host_id:community> pair, which is to be used to select the hosts to which traps are to be sent.  The host id can be a valid host name or an IP address in the dot notation.  The community string can be a maximum of 15 characters and refers to a valid community name for the SNMP manager running on the destination host. The host_id part is compulsory while the community part is optional.  More than one <host_id:com mu n ity> pair can be specified by separating them with commas. A maximum of FIVE such pairs can be specified.

community           This field specifies the <community:permissions> pairs, which are used to validate incoming SNMP requests. The community half of the pair can be a string of maximum 15 characters length.  The permissions half of the pair can be one of RO, RW, WO, NA.  The significance of these permissions options is as follows :

RO :   A SNMP Manager belonging to the specified community  can only get information and not modify the information itself.

RW : A SNMP Manager belonging to the specified community can both get information as well as modify the information.

WO : A SNMP Manager belonging to the specified community can not get information but can modify the information.

NA :   A SNMP Manager belonging to the specified community can neither get information nor can modify the information.

More than one <community:perms> pair can be specified by separating them with commas. A maximum of FIVE such pairs can be specified.

### IP tunnelling Configuration

ipx_net                Destination IPX network address which requires IP tunneling

ipt_addr               The IP gateway which supports tunneling and can route. IPX packets to the given IPX destination network

A unique IPX network number must be used for the Tunnel Network. The same IPX net number must be used in all the nodes which form the IPX Tunnel Network. e.g. if RAS1 and RAS2 are to form an IPX Tunnel network, then on each of these nodes specify the IPX Tunnel Net as (say) 0x55151234. This must be a unique network number. On RAS1, specify the gateway (peers) as the IP addresses of RAS2, and viceversa. RAS1 and RAS2 must not be on the same LAN, and must not have any other IPX communications path.

### Modem Chat Configuration

RAS uses modem chat scripts for configuring the modems. These scripts consists of a series of AT commands and the expected response. Each modem will have two entries, one for dialin and another for dialout. If the modem name is FasTraq, the script FasTraq will be used for dialout, where as the script _FasTraq will be used for dialin. Both these scripts differ as they are meant for different but similar tasks. The special symbol \d can be used in the chatscript, which will introduce a delay of 20 millisecs. The keyword st will be substituted by the telephone number in the dialout script.

RAS provides command to download these scripts from a host. There are no commands available to edit these scripts at the RAS. The administrator should change the MODEM.SCR file at the primary boot server (where RAS boots from or any other TFTP server), and download the file. Later using the commands, a particular script can be deleted, added from host. All these commands are available under /configuration/modemchats and are self-explanatory.

### Log Chat Configuration

RAS uses log chat scripts for logging procedures. This script typically consists of a set of two commands in the form $u and $p)where $u is substituted with username and $p is substituted with password. LAN-LAN path is using Challenge Response based security, the keywords $cXXX$c encloses a challenge, while $r will  be substituted with the appropriate response. These scripts are associated with a path. When a path is activated from one end, the path knows (from logchat) what kind of login procedure should hand-shaked. As the activating path contains the user name and password, it can substitute them, when the prompts are prompted by other RAS at the time of connection establishment. As mentioned previously, these chats are required only at the originating end of a path.

RAS provides command to download these scripts from a host. There are no commands available to edit these scripts at the RAS. The administrator should change the logchat.scr file at the primary boot server (where RAS boots from or any other TFTP server), and download the file. Later using the commands, a particular script can be deleted, added from host. All these commands are available under /configuration/loginchats and ale self-explanatory.

Two chatscripts have been provided for PT LAN-LAN login, and for CR-II LAN-LAN login. These are sufficient for all RAS - RAS communication. Modifications may be required only when talking to other third-party systems.

FNote: This might seem tricky by looking at the above path profiles. One might get an obvious doubt of how to connect to other routers as they don't understand the chat mechanisms (log chats). For such kind of configurations, the logchat can be left blank which forces the RAS not to play the logchat procedure with other routers.

### SAS Configuration

In \configuration\security\sas, add the IP address of each of the SAS servers. The protocol RADIUS or TACACS or both can be chosen. If RADIUS is enabled, then a RADIUS shared secret must also be provided.

Client Configuration

RAS supports three kinds of clients to access the RAS services. These are

TCP/IP client (Windows & DOS)

IPX client (Windows & DOS)

NetBEUI client on Windows For Workgroup - WFG)

To support these clients, drivers and dial front ends are provided under DOS and Windows. Apart from this, in NetWare environment, NASI redirector and INT14 redirector software is also provided to let the NetWare users use the modem pool services available at the RAS. These four clients will be discussed in detail in the following sections.

### TCP/IP Client

There are many TCP/IP packages available in the industry. Currently the RAS client software has been tested to work with PC/TTCP.DOS and Windows versions only. HCL team in India is working on NetManage and Microsoft TCP/IP computability with the RAS client software.

The following procedure should be followed for configuring and using PC/TCP under DOS or Windows with RAS :

a)  Install PC/TCP for DOS or Windows, if not already done. In windows, choose custom setup and Windows (VxD kemel) option. Give the appropriate node address and router address (this depends on the network, it should be connected to). The router address should be the RAS on that network. Make sure to select the packet driver as the driver type. At the end, when it prompts to select a packet driver, choose any driver and exit.

b) Make the following entries in the CONFIG.SYS file. This will be automated later. device=c:\leemah\lanman\protman.dos /I:c:\leemah\lanman(this entry must precede the following entry) device=c:\leemah\lanman\dis_pkt.dos

c) Make the following changes in AUTOEXEC.BAT file.  This will be automated later.  Comment the pctcp driver. It will be like c:\pctcp\ne2000.dos, in case of NE2000 adapter Comment the c:\pctcp\ethdrv (DOS) or c:\pctcp\vxdinit (Windows)

d) Create a IP.BAT file for loading the files. The entries should be as given below. This will be automated later.

For DOS :

cd \leemah\bin

ndisppp

cd \leemah\lanman

netbind

For Windows:

cd \leemah\bin

ndisppp

cd \leemah\lanman

netbind

cd \pctcp (assuming that PC/TCP is installed in c:\pctcp directory)

vxdinit

e) Load DOS or Windows dial front-end and configure the following parameters :

In the Port settings menu, select the com port and set the baud, flow control etc. All the com ports can configured, but only one com port should be used at any time to connect to RAS. Hence, one of the ports configured should be enabled for connectivity by clicking the Set default port.

- In the protocols menu, enable TCP/IP protocol. Other options like VJ compression and other PPP negotiation parameters can also be tried. But the default parameters should be sufficient. Currently, the IP address can be dynamically assigned to clients. If dynamic IP address is being used, then the RAS will assign an address to the client (based on the port on which he has dialed in). This will be displayed in the RAS Client front end after dialup is completed. At present, due to limitation, the user will need to modify PC/TCP configuration to use the same address as is assigned by the RAS. Later on, this step will be automated and the dynamically-assigned IP address will be directly communicated to the PC/TCP stack by the dial front-end. Alternatively, the user can assign the node (IF) address here itself. The default interrupt vector for PPP driver is 0x60. If it happens to be used by anybody else, which will be reported at the loading time, this should be changed to a unique value.

- In the Modem menu, select a modem type from the list of modems displayed   and attach it to the selected com port. The modem chat is also displayed   when the modem is selected. This can be modified on-line by positioning the   cursor there and editing the entry. After the editing is done, to save the   changes, click Modify and OK buttons. In case of Null modem, select the Null   modem.

- In the users menu, create a user with user name, phone number and security mechanism to be used (Infocard or disc etc).

- Exit from the dial front end. Go to DOS (in case of Windows)

f) Reboot the client system (PC).

g) RAS side configuration:

Connect the PC with modem or null modem to any of the async ports on the RAS. The port should be configured using the command /configuration/permanent/ports/modify with the same set of parameters as the com port (baud, parity, data bits, stop bits, flow-control, modem type etc.).

Configure the user using the /configuration/security/users/add command and providing all the necessary replies for the prompts like password, authentication type (PT, CR1, CR2 etc.) and path etc.

j) Configure the path using /configuration/path/add command and providing all the necessary replies for the prompts like status, connect type, destination type, protocol, IP status, IP address, etc.

The following is the sample port, user and path configuration for the TCP/IP client (It's provided in sample configuration files with RAS software):

Path Configuration (for IP)

Parameter                                                  Values

Status (Enabled/ Disabled)                                 Enabled

Connection Type (Demand / Persistent/ Client)              Client

Data Compression (None / RLE / LZW)                        None

Protocol (SLIP/ PPP)                                       PPP

IP Status (On /Off)                                        On

estination IP Address

VJ Compression Status (On / Off)                           Off

IPX Status (On / Off)                                      Off

NetBEUI Status (On/ Off)                                   Off

Negotiate Maximum Receive Unit (Yes / No)                  No

Negotiate Async Map (Yes / No)                             No

Negotiate Authentication ( Yes / No)                       No

Protocol Field Compression (On / Off)                      Off

Address Field Compression (On / Off)                       Off

Idle Timeout (in mins)                                     0

### User Configuration (for ip)

Parameter                                                     Values

Full Name                                                     IP user

Max. Connection time (in mins)                                0

Session (None/ Admin/ Path/ loath/ aCs)                       Lpath

Path Name                                                     ip

User Type (Extemal / Local)                                   External

Authentication type (PT/C R2/CR 1 /R01 /R02/M R01 /M R02)     PT

Password                                                      **

Call back type (None, Fixed, Variable)                        None

Port Configuration (for A2)

Parameter                                Values (no modem) Values(modem)

Baud Rate                                9600              9600

Character size                            8                 8

Panty                                     None              None

No of stop bits                           1                 1

SNV output flow control                   Off               Off

SNV input flow control                    Off               Off

Modem Name                              NULL MODEM        FasTraq

Monitor Carrier                           Off               On

Intr Char (<"> <char> or <\000>)         ^C                ^C

Stop Char (<A> <char, or <\000>)         ^S                ^S

StartChar(<A> <char> or<\000>)           ^Q                ^Q

HWV output flow control                  CTS               CTS

HWV input flow control                   RTS               RTS

Port Type                               dialin             dialin

Session Prompt                           ->                ->

As we discussed in RAS configuration, the user is linked with the path over a  particular port.

h)  Reboot the RAS for the configuration to become effective.  Remember that the changes made an are in /configuration/permanent tree will be stored  NVRAM will be effective when the RAS boots next time. The changes made in /configuration/runtime will be valid for the active session of the RAS only.

I) Load IP.BAT.

In case of DOS PC/TCP, use either DOS or Windows dial front-end and dial the  RAS to establish a PPP connection.  To do this, select the user from the menu  on the top right hand side and click the dial box from the tool bar. The dial  window specifies the summary of the selected user.  Make sure that the  configuration is proper and enter the password and click OK button. This starts  the dialing process and prompts for any necessary items like response for  challenge etc,. Provide appropriate answers to the prompts. Once the user is  authenticated, the PPP negotiation will start. After the connection is successful,  exit the front-end (come back to DOS, in case of Windows) and load  c:\pctcp\ethdrv.

In case of Windows PC/TCP, use Windows dial front-end and dial the RAS to  make a PPP connection. The rest of the login procedure is as described above  for the DOS dial front-end. After the connection is established, use any of the  PC/TCP Win applications.

FNote : The challenge-response prompts are given only for Windows dial front end.

### IPX Client

The following procedure should be followed for configuring and using Novell IPX under  DOS with the RAS:

a) Install NetWare client for DOS, if not already done.

b) Create a IPX.BAT file for loading the files.  The entries should be as given below. This will be automated later.

For DOS :

\netware\lsl (assuming that netware client is in netware directory)

cd \leemah\bin

odippp

c) Load DOS or Windows dial front-end and configure the following parameters:

- In the Port settings menu, select the com port and set the baud, flow control etc.  All the com ports can configured, but only one com port should be used at any time to connect to RAS.  Hence, one of the ports configured should be enabled for connectivity by clicking the Set default  port.

- In the protocols menu, enable IPX protocol only.  Other options like PPP negotiation parameters can also be tried.  But the default parameters should be sufficient.  The IPX address will be assigned dynamically. Hence it need not be configured here.  The default interrupt vector for PPP driver is 0x60. If it happens to be used by anybody else, which will be reported at the loading time, this should be changed to a unique value.

- If IPX is being used with VLMs, then the Local IPX address must be assigned in the front-end itself.  It cannot be negotiated.  The Local IPX Network   Address must be the same as the IPX internal Network Address of the RAS to   which this client will be dialing into.  The Local IPX Node Address (MAC) must   he unique for this client among all the users.

- In the Modem menu, select a modem type from the list of modems displayed  and attach it to the selected com port. The modem chat is also displayed  when the modem is selected. This can be modified on-line by positioning the  cursor there and editing the entry. After the editing is done, to save the  changes, click Modify and OK buttons. In case of Null modem, select the Null  modem .

- In the users menu, create a user with user name, phone number and security   mechanism to be used (Infocard or disc etc).

Exit from the dial front end. Go to DOS (in case of Windows).

d)Reboot the system.

e) RAS side configuration:

Connect the PC with modem or null modem to any of the async ports on the RAS. The port should be configured using the command /configuration/permanent/ports/modify with the same set of parameters as the com port (baud, parity, data bits,stop bits, flow-control, modem type etc.).

Configure the user using the /configuration/security/users/add command and providing all the necessary replies for the prompts like password, authentication type (PT, CR1, CR2 etc.) and path etc.

Configure the path using /configuration/path/add command and providing all the necessary replies for the prompts like status, connect type, destination type, protocol, IP status, IP address, etc.

The following is the sample port, user and path configuration for the IPX client (It's provided in sample configuration files with RAS software):

Path Configuration (for ipx)

Parameter                                                  Values

Status(Enabled / Disabled)                                 Enabled

Connection Type (Demand / Persistent / Client             Client ·

Data Compression (None / RLE / LZW)                       None

Protocol (SLIP / PPP)                                     PPP

IP Status (On / Off)                                      Off

IPX Status (On / Off)                                     On

Destination IPX Address                                   *.*

NetBEUI Status (On / Off)                                 Off

Negotiate Maximum Receive Unit (Yes / No)                 No

Negotiate Async Map (Yes / No)                            No

Negotiate Authentication (Yes / No)                       No

Protocol Field Compression (On /Off)                      Off

Address Field Compression (On /Off)                       Off

Idle Timeout (in mins)                                     0

User Configuration (for ipx)

Parameter                                                  Values

Full Name                                                  ipx user

Max.Connection time (in mins)                              0

Session (None/Admin/Path/Lpath/aCs)                        Lpath

PathName                                                   ipx

User Type (External / Local)                               Extemal

Authentication type (PT/CR2/CR1/RO1/R02/MR01/MR02)         PT

Password                                                   **

Call back type (None, Fixed, Variable)                     None

The same port configuration as described for TCP/IP client applies here also.

f) Reboot the RAS for the configuration to become effective. Remember that the changes made anywhere in /configuration/permanent tree will be stored in NVRAM and will be effective when the RAS boots next time. The changes made in /configuration/untime will be valid for this session of RAS only.

g) Load IPX.BAT.

Use either DOS or Windows dial front-end and dial the RAS to establish a PPP connection.  To do this, select the user from the menu on the top right hand side and click the dial box from the tool bar.  The dial window specifies the summary of the selected user.  Make sure that the configuration is proper and enter the password and click OK button.  This starts the dialing process and prompts for any necessary items like response for challenge etc.  Provide appropriate answers to the prompts.  Once the user is authenticated, the PPP negotiation will start.  After the connection is successful, exit the front-end (come back to DOS, in case of Windows) and load the following:

\netware\ipxodi

\netware\netx

FNote: The challenge-response prompts are given only for Windows dial frontend.

### NetBEUI Client

The following procedure should be followed for configuring and using WFG NetBEUI with RAS:

a) Install WFG, if not already done.  Enable the Microsoft network and follow the following steps to add the RAS NDIS-PPP driver.

- Run the WFG network setup

- Select the drivers

- Select the Add Adapter button

- Select the Unlisted or updated drivers

- Insert the RAS client diskette and click OK

- HCL-HP NDIS-PPP will be shown in the list. Click OK and close

- Remove the SPXIIPX compatible with NDIS.

- Continue with the setup

- When it prompts to load the enhanced mode protocol manager files, follow the directions and insert the appropriate diskettes and load the files.

- After successful installation, close network setup and continue.

- Exit to DOS

- Make sure that the CONFIG.SYS contains the c:\windows\ifshlp.sys entry. This will be added by the WFG.

b) Make the following changes in the SYSTEM.INI file. This will be automated later.  Comment the line netcard=ndisppp.exe in [network drivers] section Comment the line device=*vcd (if there is one) in [386enh] section

c) Make the following changes in AUTOEXEC.BAT file. This will be automated later.  Comment the net start line.

d) Create a NB.BAT file for loading the files. The entries should be as given below.  This will be automated later.

cd \windows (assuming that the windows is loaded in windows directory)

net init

cd leemah\bin

ndisppp

cd \windows

net start

e) Load Windows dial front-end and configure the following parameters:

- In the Port settings menu, select the com port and set the baud, flow control etc. All the com ports can configured, but only.one com port should be used at any time to connect to RAS. Hence, one of the ports configured should be enabled for connectivity by clicking the Set default port.

- In the protocols menu, enable NetBEUI protocol only. Other options like PPP negotiation parameters can also be tried. But the default parameters should be sufficient. The default interrupt vector for PPP driver is 0x60. If it happens to be used by anybody else, which will be reported at the loading time, this should be changed to a unique value.

- In the Modem menu, select a modem type from the list of modems displayed and attach it to the selected com port. The modem chat is also displayed when the modem is selected.  This can be modified on-line by positioning the cursor there and editing the entry. After the editing is done, to save the changes, click Modify and OK buttons.  In case of Null modem, select the Null modem.

- In the users menu, create a user with user name, phone number and security mechanism to be used (Infocard or disc etc).

- Exit from the dial front end. Go to DOS.

f) Reboot the system.

g) RAS side configuration :

Connect the PC with modem or null modem to any of the async ports on the RAS. The port should be configured using the command configuration/permanent/ports/modify with the same set of parameters as the com port (baud, parity, data bits, stop bits, flow-control, modem type etc.).

Configure the user using the /configuration/security/users/add command and providing all the necessary replies for the prompts like password, authentication type (PT, CR1, CR2 etc.) and path etc.

Configure the path using /configuration/path/add command and providing all the necessary replies for the prompts like status, connect type, destination type, protocol, IP status, IP address, etc.  The following is the sample port, user and path configuration for the NetBEUI client (It's provided in sample configuration files with RAS software):

Path Configuration (for nb)

Parameter                                                  Values

Status (Enabled / Disabled)                                Enabled

Connection Type (Demand / Persistent/ Client)              Client

Data Compression (None / RLE/ LZW)                         None

Protocol (SLIP / PPP)                                      PPP

IP Status (On / Off)                                       Off

IPX Status (On / Off)                                      Off

NetBEUI Status (On/ Off)                                   On

T1 Timer (in milliseconds)                                 6000

T2 Timer (in milliseconds)                                 1000

TI Timer (in milliseconds)                                 120000

Send Window Size (1-127)                                   40

Receive Window Size (1-127)                                40

Retry count                                                6

Search broadcast allowed (Yes / No)                        Yes

Name Traffic allowed (Yes / No)                            Yes

Negotiate Maximum Receive Unit (Yes / No)                  No

Negotiate Async Map (Yes / No)                             No

Negotiate Authentication (Yes / No)                        No

Protocol Field Compression (On / Off)                      Off

Address Field Compression (On / Off)                       Off

Idle Timeout (in mins)                                     0

User Configuration (for nb)

Parameter                                                  Values

Pull Name                                                  nb

user Max. Connection time (in mins)                        0

Session (None/Admin/ Path/ Lpath/ aCs)                     Lpath

Path Name                                                  nb

User Type (External / Local)                               Extemal

Authentication type (PT/CR2/CR1/RO1/R02/MR01/MR02)         PT

Password                                                   **

Call back type (None, Fixed, Variable)                     None

The same port configuration as described for TCP/IP client applies here also.

h) Reboot the RAS for the configuration to become effective.  Remember that the changes made anywhere in /configuration/permanent will be stored in NVRAM and will be effective when the RAS boots next time.  The changes made in /configuration/runtime will be valid for this session of RAS only.

I) Load NB.BAT.

Use Windows dial front-end and dial the RAS to make a PPP connection.  To do this, select the user from the menu on the top right hand side and click the dial box from the tool bar. The dial window specifies the summary of the selected user. Make sure that the configuration is proper and enter the password and click OK button. This starts the dialing process and prompts for any necessary items like response for challenge etc,. Provide appropriate answers to the prompts. Once the user is authenticated, the PPP negotiation will start. After the connection is established, try connecting to the WFG nodes on the networks connected to the RAS.

F Note : The challenge-response prompts are given only for Windows dial front end.

j) NetBEUI uses LLC protocol for carrying the NetBIOS commands. This protocol is extremely sensitive to time-outs. The time-outs are in milliseconds. Some of the time-outs are not configurable as the support is not provided in WFG. Yet, some of the timers can be changed by editing the [NETBEUI] section in protocol.ini file. For instance, when we browse for the list of WFG nodes from a remote NetBEUi client, we may not see the WFG nodes on networks connected to RAS due to time out problems. Yet, if we try connecting to a specific node on the network from the remote node, it gets connected sometimes (if the responses comes before the time-out happens). If it doesn't connect, we can tailor the following timers until we feel it's sufficient. In most situations, the following values are sufficient.  As explained before make the necessary changes to the Procol.ini file.  Include at least the statementd below:

NETBIOSTIMEOUT=4000

NETBIOSRETRIES=4

### NASI Client

RAS client software provides the NASI redirector software for providing the modem pooling services. This software works only in the NetWare environment. NetWare client users using this software can use the modem resources available on the RAS. The following procedure should be followed for configuring the ACS services on the RAS and clients :

a) Configure a set of async ports on the RAS for ACS use. The following sample confiauration explains the use of A2 port RAS for ACS.

Port Configuration (for A2)

Parameter                          Values

Baud Rate                           9600

Character size                      7

Parity                              Odd

No of stop bits                     2

SW output flow control              Off

SW input flow control               Off

Modem Name                          FasTraq

Monitor Carrier                     Off

Intr Char (<"> <char> or <\000>)    ^C

StopChar(<"> <char>or<\000>)        ^S

StartChar(<"> <char>or<\000>)       ^Q

H/V output flow control             None

H/V input flow control              None

Port Type                           acs

ACS Specific Port Name              ACSPORT_2

ACS Group Port Name                 POOL_1

Init Chat Script

Always provide a proper Group Port Name for ACS ports

b) Attach modems (FasTraq) to the ports configured for ACS service

c) Configure an acs user on the RAS

d) Reboot the RAS for the configuration to take effect.

e) Load LSL.COM

f) Load the LAN MLID.

g) Load the IPXODI.

h) Load the ASI.EXE (a DOS TSR providing NASI services).  This uses the configuration provided in ASI.1NI.  This file contains the IPX socket number of the RAS ACS services and the preferred ACS server to connect to.  The socket number should match with the RAS ACS socket number.  The preferred ACS server need not be specified if it should be connected to whatever is available at the time of loading ASI.  The following prompts should be answered correctly.

Give the acs user name configured on the RAS at the user prompt

Give the password at the password prompt

Give some session name at the session name prompt

i) Start Windows and load any NASI application (PC AnyWhere and ProComm) to use the ACS ports configured above. Configure the applications to use NASI interface.

j) The NASI redirector software (ASI) provides the NASI API. As some of the typical dialout applications use INT 14 also, the PC com ports can be redirected to the ACS ports available on RAS. This can be by the following procedure.

Load LSL, MLID, IPXODI and ASI as mentioned above

Load INT14.EXE (DOS TSR) which provides the com port redirection sen/ices.

Load Windows, select LeeMah Group and INT14 item.

Configure the redirection of com ports using WINT14.

Load any INT14 applications like PC Anywhere and ProComm to use the

RAS ACS services.

# Chapter 6. BandWagon RAS Main Board Diagnostics & Power On Self Test Document

Introduction

This section describes the functionality of the Diagnostics & Power On Self Test POST) utility for the BandWagon RAS Main board.  This utility will be present  the boot EPROM of the Main Board Processor.  This utility can be functionally divided into two parts

- POST

- Diagnostic Utility

The POST does an extensive self test on the various functional modules of the Main Board and reports the results of the self test on power UP. The diagnostics utility will help a test engineer to test & debug various functional modules after the Main Board powers up.

## User Interface

The Diagnostics & POST utility would interact with the user through a terminal. connected to the console port of the Main board.

Invoking Diagnostics Utility

The Diagnostics can be invoked, during power up by two means.

1. By hardware

The Main board has a 8 position DIP switch(SW 1).  If all the positions in the DIP switch are kept open during power up, the Diagnostics utility will be invoked.

2. By software

On every power up, the Diagnostics & POST utility displays the message: " RAS POST  Ver 1.1  June 1995 "and waits 5 seconds for the user to press a particular key sequence (^C, conrtol+C key). If the user presses the key sequence within the 5 seconds duration, the diagnostics utility will be invoked.

## Power on Sequence

The Diagnostics & POST utility is present in the higher end of the BOOT EPROM (at real address location 0xF000:0x6000 to 0xF000:0xFFFF).  After RESET the control is passed on to the Diagnostics and POST utility at location 0xF000:FFF0.

The Power On Self Test (POST) performs the following sequence:

- The Register EDX contains the CPU Revision.  This is saved in the RTC NVRAM to be used later.

- Calculates the EPROM Check Sum.

- Initializes the NEPTUNE Chip Set.  This involves programming the PCMC, PCEB and the ESC Registers.

- Initializes the Global Descriptor Table.

- Initializes the Segment Registers.

- Switches to Protected Mode.

- Masks all the Interrupts.

- initializes the COM PORT.  If the COM PORT initialization fails, then it flashes the LED to indicate the failed condition and enters into a tight loop.  If the COM PORT initialization is successful then display the message:

" RAS POST  Ver 1.1  June 1995 "

- Determines the amount of Memory present in the system and programs the PCMC Registers.  The Memory Size is stored in the RTC NVRAM.

- Checks for the DIP SWITCH positions, if all contacts are open then it enters Diagnostics.

- Prompts the user to press a key sequence (^C) to enter  diagnostics.  Keeps polling the console port for, looking for ^C, for 5 seconds.  If the user has pressed ^C then it enters Diagnostics.

- Displays the message

"Starting Self Test Sequence..."

- Displays the amount of Memory present in the system.

- Tests the memory for address shorts, by testing  with Address = Data Pattern.  If memory is not present or if the test fails, then it displays the message:

" Memory Test... Failed "

and enters into a tight loop.

- If the test passes then it displays the message:

"Memory Test..... Passed"

- Checks for the presence of the Second Level Cache.  If the Cache is present then it checks for the size of the Cache (256KB or 512KB).  Displays the amount of Cache present in the system.

- Senses the amount of NVRAM present in the system and displays the size.

- Checks for the presence of the FLASH by testing whether the FLASH ID is correct

- Tests for the presence of each of the ASIO Cards.  For each ASIO card present in the system, it tests the DPRAM and MailBox. If the DPRAM or the MailBox fails then the ASIO Card is not Reset.

If the DPRAM and MailBox Tests pass,  then the ASIO Card is Reset by the Main Board Processor.  The Main Board Processor waits for the ASIO Card to complete its POST.  The ASIO Card completes its POST and generates an Interrupt to the Main Board Processor.  The Interrupt Path between the two Processors is then tested.  If the test passes then it displays the message:

"ASIO Card Number X.......Passed"

If the test fails then it displays the message:

"ASIO Card Number X......Failed"

- Tests the LAN Card.  If the LAN Card Test fails then it displays the message:

"LAN Card Test........Failed"

If the test pass then it displays the message:

"LAN Card Test......;Passed"

- Tests the Sync Card.  If the Sync Card Test fails then it displays the message:

"Sync Card Test........Failed"

If the test pass then it displays the message

"Sync Card Test.......Passed"

- Prepares the Diagnostics - Driver structure.

- Enables the First Level Cache and the Second Level Cache (if present in the system).

- Enables the DMA Controllers

- Initializes the Segment Registers.

- Passes Control to the Upper Layer Software.

## DIAGNOSTICS

The RAS Diagnostics utility  tests and debugs various modules of the Main board.  This could be invoked by the user, during power up, either by hardware or software as described in the previous section.

The Diagnostic is interactive and the user interface is through a terminal connected to the Console Port.  The Diagnostics  presents a sequence of menus to the user to select and execute the tests.

Whenever the user exits from the Diagnostic utility, by pressing ^C from the Main Menu, the Self Test Sequence described in the previous section continues and finally transfers control to the Upper Layer Software.

### Main Menu of the BandWagon RAS Diagnostics

The Main Menu of the RAS Diagnostics  shall have the following options,

MAIN BOARD LOGIC TESTS

ASIO ADAPTER TESTS

NETWORK TESTS

SYNC CARD TESTS

SYSTEM DEBUG AID

The MAIN BOARD LOGIC TESTS MENU allows the user to test the DRAM, CACHE, RTC, NVRAM, FLASH and the COMMUNICATION PORT.

The ASIO ADAPTER TESTS allows the user to test the ASIO cards present on the extended BUS.

The NETWORK TESTS allows the user to test the LAN Card.

The SYNC CARD TESTS allows the user to test the SYNC Card.

The SYSTEM DEBUG AID assist  user in debugging the system by providing features such as continuous access of memory or I/O location,  viewing the contents of memory or I/O locations and changing their contents.

### Main Board Logic Tests Menu

The MAIN BOARD LOGIC TESTS menu provides the following options:

- MEMORY TEST

- CACHE TEST

- RTC TEST

- FLASH TEST

- NVRAM TEST

- COMMUNICATION INTERRUPT TEST

- CONTINUOUS TEST

The MEMORY TEST tests the DRAM, by writing and checking different patterns such as 0x0000, 0xFFFF, 0xAAAA, 0x5555, walking 1’s, walking 0's, address as data.

The CACHE TEST tests the CACHING PORTION.  It tests the external cache by

conducting the following tests:

- Line Fill Test

- FlushTest

- Write Back Test

The Real Time Clock (RTC) TEST tests the Main board RTC.  Before proceeding with the test it warns the user that the contents of the RTC NVRAM will be destroyed.

The FLASH TEST tests the FLASH after warning the user that the contents will be destroyed.

The NVRAM TEST tests the NVRAM after warning the user that the contents will be destroyed.

The COMMUNICATION INTERRUPT TEST tests the Receive Interrupt and the Transmit Interrupt path for the Communication Port.

The CONTINUOUS TEST allows the user to test the Main Memory and the NVRAM in a continuous mode.

### ASIO Adapter Tests Menu

Once this option is chosen from the MAIN MENU, the diagnostics asks the user to enter the ASIO Card Number to be tested.  Since the maximum number of ASIO Cards present in the system is four, the valid options are from 1-4.  Afterwards,  the following MENU shall be presented to the user:

DPRAM TEST

MAIL BOX TEST'

HOST-ASIO COMMUNICATION TEST

RESET ASIO

CONTINUOUS TEST

The Dynamic Port Random Access Memory (DPRAM) TEST  tests the Main board processor's accessibility of the DPRAM.  The DPRAM TEST  tests the DPRAM, by writing and checking different patterns such as 0x0000, 0xFFFF, 0xAAAA, 0x5555, walking 1's, walking O's, address as data.

The MAIL BOX TEST tests the Main board processor's accessibility of the MAIL BOX registers.  The MAIL BOX TEST also tests the MAIL BOX, by writing and checking different patterns such as 0x0000, 0xFFFF; 0xAAAA, 0x5555, walking 1's, walking 0's, address as data.

The HOST-ASIO COMMUNICATIONS TEST tests the interface between the Main Board Processor and the ASIO Processor.  The Main Board Processor Resets the ASIO Processor and waits for it to complete its POST and generate an interrupt to the Main Board Processor.  It then checks the following:

- It checks whether the ASIO processor generated interrupt reaches the Main Board Processor.

- It checks whether the Main Board generated interrupt reaches the ASIO Processor.

- It checks whether the Main Board generated NMI interrupt reaches the ASIO Processor.

The RESET ASIO option helps the user to reset the 386 processor on the ASIO board.

The CONTINUOUS TEST allows the user to test the DPRAM, MAIL BOX and the HOST-ASIO Communication path in continuous mode.

### Network TESTS

Once this option is chosen From the MAIN MENU, the diagnostics software checks, for the presence of the Board, the physical connection between the Controller & the PCB and the functionality of the Internal Configuration Register .

The above tests are carried out along with the "Reset State Check" followed by the configuration of I/O space, interrupt routing and finally sets the Controller to access the I/O space.

"NETWORK TESTS"

After the successful test, the following messages are displayed.

" AMD'S PCI-PCNET DETECTED "

" Silicon revision number : x "

79C970 reset success

" PRESS ANY KEY TO CONTINUE .... "

In case of failure in the preliminary tests. the following message is displayed.

"CONTROLLER I/O ACCESS FAILURE"

Data written :ww

Data read :rr

"PRESS ANY KEY TO CONTINUE....."

Once a key is pressed, control returns to MAIN MENU

After the successful preliminary tests the NETWORK MENU is displayed.

NETWORK TESTS

79C970 INTERNAL LOOP BACK TEST

79C970 MENDEC LOOP BACK TEST

EXTERNAL LOOP BACK TEST

DISPLAY STATION ADDRESS

SINGLE/CONTINUOUS MODE

The 79C970 INTERNAL LOOP BACK TEST is carried out to check the controller for accessibility, internal signal path, master mode DMA access and interrupt path.

The 79C970 MENDEC LOOP BACK TEST is carried out to check controller for accessibility, internal signal path, master mode DMA access, interrupt path and MANCHESTER ENCODER/DECODER

The EXTERNAL LOOP BACK TEST is carried out to check controller for accessibility, internal signal path, master mode DMA access, interrupt path, MANCHESTER ENCODER/DECODER and signal path up to external connector.

The SINGLE/CONTINUOUS TEST allows the user to choose the  option of running the NETWORK TESTS in single or in continuous mode.

The DISPLAY STATION ADDRESS checks the Ethernet address in EEPROM for accessibility through network controller and validity by checksum.

### Sync Card Tests

The Sync Card TEST MENU has the following options

- SYNC CARD ADAPTER I/O TEST

- SYNC CARD DMA TEST

- SYNC CARD INTERNAL LOOPBACK TEST

- SYNC CARD EXTERNAL LOOPBACK TEST

- SYNC CARD INTERRUPT TEST

The SYNC CARD ADAPTER I/0 TEST tests the basic I/O operation of the card. The routine a writes Control Word into the Control Port and reads the Status through the Status Port.

The SYNC CARD DMA TEST tests the Bus Master operation of the SYNC CARD protocol control. The XPC accesses the ISA Bus in DMA mode and transfers a specified packet length of data from a specified source memory location to another especified destination.

The SYNC CARD INTERNAL LOOPBACK TEST  tests the clock circuitry which converts parallel data into serial format before sending the data to the transmit and receive  circuitry.  This serial data is looped back internally and the data is verified to check the integrity of the circuit.

The SYNC CARD EXTERNAL LOOPBACK TEST tests the transmit and receive circuitry on the board, which is an interface between the External agents and the XPC. The external loopback test is performed on RS232, RS422 and V.35 physical interfaces, by selecting the jumper setting on the board.

The SYNC CARD INTERRUPT TEST tests the interrupt set by the XPX.  Basically it performs a DMA test, in addition, the interrupt is serviced to do a specified task.

### System Debug Aid

The SYSTEM DEBUG AID MENU has the following options:

- CONTINUOUS READ/WRITE - MEMORY

- CONTINUOUS READ/WRITE - I/O

- EXAMINE MEMORY

- EXAMINE I/O

- DUMP MEMORY

- DUMP I/O

The CONTINUOUS READ/WRITE - MEMORY helps the user by continuously accessing a particular memory location specified by the user.

The CONTINUOUS READ/WRITE - I/O  helps the user by continuously accessing a particular I/O location entered by the user.

The EXAMINE MEMORY option helps the user to examine a particular location in memory, and change its contents.  The contents of the location shall be 16 bit long.

The EXAMINE I/O option helps the user to examine a particular I/O location and change it's contents.  The contents of the I/O location shall be 8 bit long.

The DUMP MEMORY option DUMPs the contents of a memory location starting from the location specified by the user.

The DUMP I/O option  DUMPs the contents of an I/O location starting from the location

specified by the user.

## Description Of Tests

### Memory Test

The following sequence is followed by the MEMORY TEST:

- · Obtain the SIZE of the memory, as determined by the self  test.  Display the size. Then  display the following message:  " TEST STARTED"

- · Write the following patterns, in the memory.  Once all the locations are written verify the contents.  For every pattern ,display "W" after the entire memory is written and display "V"  after it is successfully verified.

The patterns to be written are:

0x00000000

0xFFFFFFFF

0xAAAAAAAA

0x55555555

0x84218421 "Walking 1’S

0x18421842

0x21842184

0x42184218

0x7BDE7BDE "Walking O's

0xE7BDE7BD

0xDE7BDE7B

0xBDE7BDE7

Address as Data

- If the read data is not same as the written data, display the memory address, data read, data written.

"MEMORY ADDRESS         :      XXXXXXXX"

"DATA WRITTEN                :      XXXXXXXX"

"DATA READ                       :      XXXXXXXX"

Then displays:

"TEST FAILED " at LOWER RIGHT corner.

- If the MEMORY TEST passes then displays

''TEST PASSED " at LOWER LEFT corner.

- It then displays the message

"PRESS ANY KEY TO CONTINUE..."

Note: The Memory Test is done for the following Memory Ranges. (When Memory Size is less than 16 MB)

0 -640 KB

1MB - 15MB

16 MB - Maximum Memory

Once a key is pressed, the memory test is carried out for the next range.  It returns to MAIN BOARD LOGIC TESTS MENU only after completing the entire Memory Range.

### Cache Tests

It obtains the size of the cache as determined by the self test.  Displays the size of the cache.  It then displays the menu for the CACHE TESTS.

LINE FILL TEST

FLUSH TEST

WRITE BACK TEST

#### Line Fill Test:

- Displays the message

"LINE FILL TEST"

"TEST STARTED"

- Disables CPU and secondary cache.

It writes an incremental pattern from 00h  into the address location starting from  0000:0000h up to the size of the cache ( 256KB or 512KB).  Since the cache is disabled, all patterns are written into main memory.  It displays a "W" after the patterns have been written.

- Enables CPU and secondary cache.

- Reading the same addresses now will bring the lines into the cache.

- Compares with the written data to test if the line fills have properly occurred or not. Displays a "V" after the entire data has been verified.

If the read data is not the same as the written data, displays the address, data read, data written as follows:

"MEMORY ADDRESS : XXXXXXXX"

"DATA WRITTEN        : XX''

"DATA READ               : XX"

Then it displays:

"TEST FAILED " at LOWER RIGHT corner.

If the LINE FILL TEST passes it displays:

"TEST PASSED  " at LOWER LEFT corner.

It then Displays  the message:

"PRESS ANY KEY TO CONTINUE..."

After key is pressed, it returns  to the CACHE TEST menu.

#### Flush Test

- Displays the message:

"FLUSH TEST"

"TEST STARTED"

- Enables CPU and secondary cache

- Writes an incremental  pattern from 00h into the address location starting from 0000:0000h  up to the size of the cache ( 256KB or 512KB).  Displays a "W" after all the patterns have been  written.

- Disables CPU and secondary  cache.  (disabling  the cache  flushes the cache  and the contents of the cache are written back to the main memory)

- Reading the same addresses and comparing with the data written will show whether the cache has been properly flushed, because all reads are from main memory.  Displays a "V" after the entire data has been verified.

- If the read data is not same as the written data, it displays the address, data read, data written as follows:

"MEMORY ADDRESS : XXXXXXXX"

"DATA WRITTEN         : XX"

"DATA READ                : XX"

Then it displays:

"TEST FAILED " at LOWER RIGHT corner.

- If the FLUSH TEST passes it displays:

"TEST PASSED " at LOWER LEFT corner.

- It then displays the message

"PRESS  ANY KEY TO CONTINUE...

- Once a key pressed, it returns to the CACHE TEST menu.

#### Write Back Test

Displays the message:

"WRITE  BACK TEST"

"TEST  STARTED"

- Enables CPU and secondary cache

- Writes an incremental  pattern from 00h into the address location starting from 0000:0000h up to the size of the cache (256KB or 512KB).  It Displays a "W" after all the  patterns have been written.

- Reads data from another  set of address  locations  (for a size of 256KB  or 512KB). This will write back the lines present in the cache and bring in the new lines.

- Disables the CPU and secondary cache.

- Reads from memory location starting from 0000:0000 up to the size of the cache(256KB or 512KB).  Compares with the written data to test, if the write back has properly occurred or not.  It Displays a "V" after the entire data has been verified.

- If the read data is not the same as the written data, displays the address, data read, data written as follows:

"MEMORY ADDRESS : XXXXXXXX"

''DATA WRITTEN        : XX"

"DATA READ               : XX"

It then displays:

"TEST  FAILED " at LOWER RIGHT corner

- If the WRITE BACK TEST passes it displays:

"TEST PASSED " at LOWER LEFT corner.

It then Displays the message:

"PRESS  ANY KEY TO CONTINUE...

- Once a key is pressed, it returns to the CACHE TEST menu.

### RTC Test

- The RTC test first checks for Valid RAM and the Time bit in the status register, which reflects the status of the internal lithium energy source.  When internal lithium battery is exhausted, the contents of both the RTC data and the RAM data are questionable. So it displays the message:

"INTERNAL LITHIUM BATTERY EXHAUSTED"

- Returns to the MAIN BOARD LOGIC TESTS menu.

- If the above check has passed, the RTC test continues by displaying the current date and time.

"REAL TIME CLOCK TEST"

"CURRENT DATE (DD-MM-YY) : dd-mm-yy"

"CURRENT TIME (HH:MM:SS) : hh:mm:ss"

- An option is then provided to enter new time and date or continue with the RTC test.  It then it Displays the message:

"PRESS Ctrl Q TO ENTER NEW DATE AND TIME OR CR TO

CONTINUE WITH RTC TEST"

- If Ctrl Q is pressed then it displays the message, prompting  user to enter the new date and time.

"ENTER NEW DATE (DD-MM-YY) : dd-mm-yy"

"ENTER NEW TIME (HM:MM:SS) : hh:rnm:ss"

- It updates the RTC with the new date and time.

- If the Carriage Return is pressed then it continues with the RTC NVRAM test.  It Displays the warning message that RTC data will be lost if this test continues.

"WARNING: RTC NVRAM DATA WILL BE LOST! PROCEED (Y/N):"

If "N" is pressed then it  displays the message:

"WARNING: RTC NVRAM 1S NOT TESTED"

It then Returns to the MAIN BOARD LOGIC TESTS menu.

If` "Y" is pressed then it displays the message

"RTC NVRAM TEST STARTED"

It writes the patterns shown below, in the memory.  Once all the locations are written it verifies the contents.  For every pattern, it displays "W" after the entire memory is written and it displays "V" after it is successfully verified.

The patterns to be written are:

- 0x00

- 0xFF

- 0xAA

- 0x55

- walking 1's

- Walking O's

- Address as Data

- If the read data is not same as the written data, it displays the memory address, data read, data written and displays it as follows:

"MEMORY ADDRESS : XX"

''DATA WRITTEN        : 'XX"

"DATA READ               : XX"

Then displays,

"TEST FAILED " at LOWER RIGHT corner.

- If the RTC NVRAM TEST passes it displays:

"TEST PASSED " at LOWER LEFT corner.

- Displays the message:

"PRESS ANY KEY TO CONTINUE..."

- After a key pressed, it returns to the MAIN BOARD LOGIC TESTS menu.

### Flash Test

The FLASH TEST ensures the programmability of the Flash memory.

- The Flash ID is checked.  If the ID is not correct then displays the message

"CHECK THE FLASH ..."

- It then displays the message:

"PRESS ANY KEY TO CONTINUE..."

- After a key pressed, it rectums to the  MAIN BOARD LOGIC TESTS menu.  If the ID is correct then it displays the message:

"FLASH - INTEL 28F008SA"

- Before doing the Flash Write/Read Test the flash memory is erased.  It displays the warning message that flash will be erased if this test continues.

"WARNING: FLASH DATA WILL BE LOST! PROCEED (Y/N):"

If 'N' is pressed, it displays the message:

"WARNING: FLASH NOT TESTED"

followed by:

"PRESS ANY KEY TO CONTINUE..."

After a key is pressed, it returns to the MAIN BOARD LOGIC TESTS menu.

- If “Y” is pressed, it continues with the Flash Erase and  displays the message:

"FLASH BEING ERASED"

- Checks if the flash has  been properly erased.  If the flash has not been properly erased it displays the message:

"FLASH HAS NOT BEEN PROPERLY ERASED"

- It displays the Block Address where the Erase failed as follows:

"BLOCK ADDRESS WHERE ERASE FAILED: XXXXXXXX"

and displays the message:

"TEST FAILED " at LOWER RIGHT corner.

Then it displays the message:

"PRESS ANY KEY TO CONTINUE..."

- After a key is pressed, it returns to the MAIN BOARD LOGIC TESTS menu.

- If the Flash memory has been properly erased, it continues by doing the Flash Write test.

- Displays the message:

"CONTINUE WITH FLASH READ/WRITE TEST (y/n)? "

- If 'N' is pressed, it displays the message:

"WARNING: FLASH NOT TESTED"

followed by:

"PRESS ANY KEY TO CONTINUE..."

- After a key is pressed, it returns to the MAIN BOARD LOGIC TESTS menu.

- If "Y" is pressed, it displays  the message:

"FLASH READ/WRITE TEST STARTED"

"Address = Data Pattern ....."

- Tests the Flash by writing Address as Data in every location.  Checks whether the Flash Byte Write is proper.  If the Flash write fails then it displays the message:

"FLASH WRITE NOT SUCCESSFUL"

"FLASH ADDRESS WHERE WRITE FAILED: XXXXXXXX"

- It then Displays the message:

"PRESS ANY KEY TO CONTINUE..,"

- After a  key pressed, it returns to the MAIN BOARD LOGIC TESTS menu.

- If Flash Write succeeds then it displays a "W" after the entire range has been written to.  Once all the locations are written, then it read/verify  the contents.

- If the read data is not the same as the written data, it displays the memory address, data read, data written as:

"MEMORY ADDRESS:  XXXXXXXX"

"DATA WRITTEN:        XX"

"DATA READ:               XX"

- Then displays:

"TEST FAILED " at lower right corner.

- If the FLASH TEST passes then it displays "V" after the entire range has been verified.  It then Displays:

"TEST PASSED " at lower left corner.

- Displays the message:

"PRESS ANY KEY TO CONTINUE...

- After a key is pressed, it returns to the MAIN BOARD LOGIC TESTS menu.

### NV RAM Test

Displays the NVRAM MENU

" NVRAM TEST MENU "

"1. I NVRAM- 128KB "

"2. II NVRAM- 128KB "

"3. III NVRAM- 128KB "

"4. IV NVRAM-128KB"

"5. V NVRAM- 128KB "

"6. VI NVRAM- 128KB "

"7. VII NVRAM- 128KB "

"8. VIII NVRAM- 128KB "

"9. CHECK MAXIMUM NVRAM SIZE "

- The NVRAM TEST provides the user with the option of testing each of the 8 NVRAM modules independently.  For this, the user has to select options 1-8 to test the desired NVRAM.  Option 9 checks the Maximum amount of NVRAM  available  and then proceeds  to test the entire range.

- The NVRAM TEST for the first 8 options is as follows:

- Display the NVRAM number being tested.

"NVRAM TEST"

"NVRAM TO BE TESTED: X"

- The NVRAM test warns that the contents will be lost if the test continues.  It displays the message:

"WARNING: NVRAM DATA WILL BE LOST ! PROCEED (Y/N):"

- If "N" is pressed then displays the message

"WARNING: NVRAM NOT TESTED"

"PRESS ANY KEY TO CONTINUE...

- After a key is pressed, it returns to the MAIN BOARD LOGIC  TESTS menu.

- If "Y" is pressed then it starts the NVRAM test.  Displays the message:

"NVRAM TEST STARTED"

- Writes the following patterns, in the NVRAM.  Once all the locations are written it verifies  the contents.  For every pattern, it displays "W".  After the entire NVRAM is written it  displays "V" after it is successfully verified.

- The patterns to be written are:

- 0x00

- 0xFF

- 0xAA

- 0x55

- 0x84 "Walking 1’S

- 0x18

- 0x21

- 0x42

- 0x7B "Walking 0's

- 0xE7

- 0xDE

- 0xBD

- If the read data is not same as the written data, it displays the memory address, data read, and data written.

"MEMORY ADDRESS :  XXXXXXXX"

"DATA WRITTEN:          XX"

''DATA READ:                XX"

- Then displays:

"TEST FAILED " at lower right corner.

- If the NVRAM TEST passes, then displays

"TEST PASSED " at lower left corner.

- Displays the message:

"PRESS ANY KEY TO CONTINUE..."

- After a key is pressed, it returns to the  MAIN BOARD LOGIC  TESTS menu.

The NVRAM TEST for option 0 checks the Maximum amount of NVRAM present in the system and then tests the NVRAM.  The NVRAM test warns the user that the contents will be lost if the test continues. Displays the message:

"NVRAM TEST"

"WARNING: NVRAM DATA WILL BE LOST! PROCEED (Y/N):"

- If' 'N' is pressed it displays the message:

"WARNING: NVRAM NOT TESTED""

"PRESS ANY KEY TO CONTINUE...

- After a key is pressed, it returns to the MAIN BOARD LOGIC TESTS menu.

- If' 'Y' is pressed it checks the Maximum amount of NVRAM present and displays the size.

"NVRAM SIZE : XXXXXXXX".

- If NVRAM is not present, it displays the message:

"NVRAM NOT PRESENT"

"PRESS ANY KEY TO CONTINUE...

- After a key is pressed, it returns to the MAIN BOARD LOGIC TESTS menu.

- Displays the message:

"NVRAM TEST STARTED"

- It then writes the following patterns, in the NVRAM:

0x00

0xFF

0xAA

0x55

0x12 "'Walking 1's

0x42

0x84

0x18

0x7B "Walking 0's

0xE7

0xDE

0xBD

Address as Data

Once all the locations are written it verifies the contents.  For every pattern, it displays "W" after the entire NVRAM is written and displays "V" after it is successfully verified.

- If the read data is not same as the written data, then it displays the memory address, data read, data written.

"MEMORY ADDRESS   :  XXXXXXXX"

"DATA WRITTEN          :  XX"

"DATA READ               :  XX"

- Then displays,

"TEST FAILED " at LOWER RIGHT corner.

- If the NVRAM TEST passes displays:

"TEST PASSED " at LOWER LEFT corner.

- Displays the message

"PRESS ANY KEY TO CONTINUE..."

### Continuous READ/WRITE Memory

The CONTINUOUS READ/WRITE MEMORY  helps  the user by continuously accessing a particular memory location specified by the user.

- This test needs to receive the address where the continuous access is to be made and also expects the data pattern to be written at that location.

- It displays the message

"CONTINUOUS READ/WRITE - MEMORY"

"ENTER HIGHER WORD OF ADDRESS : 0000"

- The user enters the higher word of the address. Then, the test prompts for the lower word of the address.

"ENTER LOWER WORD OF ADDRESS : 0000"

The user also enters the data pattern to be written in that location.

"ENTER DATA TO BE WRITTEN : 0000"

- This test continues until the user interrupts with a "Ctrl I."  It displays the message:

"PRESS ^I TO INTERRUPT"

When the test is interrupted, it displays the last data read from the given address location.

"LAST DATA REAL   :    dddd"

- Displays the message:

"PRESS ANY KEY TO CONTINUE..."

- Once a key is pressed, it returns to the SYSTEM DEBUG AID-menu.

### Continuous READ/WRITE - I/O

The CONTINUOUS READ/WRITE I/O helps the user by continuously accessing a particular I/O location specified by the user.

- This test expects the address where the continuous access is to be made and also expects the data pattern to be written at that location

- It displays the message:

"CONTINUOUS READ/WRITE - I/0"

"ENTER THE I/O ADDRESS  :  0000"

The user enters the I/0 address from where a continuous read/write is to be done.

The user also enters the data pattern to be written in that location.

"ENTER DATA TO BE WRITTEN  :  0000"

- This test continues until the user interrupts with a "Ctrl I."  It displays the message

"PRESS ^I TO INTERRUPT"

- When the test is interrupted, it displays the last data read from the given address location.

"LAST DATA READ : dddd''

- Displays the message

"PRESS ANY KEY TO CONTINUE..."

Once a key is pressed, it returns to the SYSTEM DEBUG AID menu.

### Dump Memory

The DUMP MEMORY option DUMPs the contents of memory location starting from the location specified by the user.

- The test prompts the user to enter the memory address which needs to be dumped.

- Displays the message:

"DUMP MEMORY"

''ENTER THE HIGHER WORD OF: THE ADDRESS    : 0000"

"ENTER THE LOWER WORD OF THE ADDRESS      : 0000"

The user enters the higher and lower address.

- The test then displays the contents of the memory location to a maximum of 256 bytes.

The first two columns represents the starting address of the corresponding row.  The next 16 bytes indicate the contents of the memory location.  The last 16 characters in each row gives the ASCII values of the memory contents.

- Displays the message:

"USE SPACE BAR FOR THE NEXT PAGE, 'B' FOR THE PREVIOUS PAGE , AND ^Q TO QUIT "

- If the user presses the space, the test will display the next 256 bytes.  If it is 'B', then    the previous 256 bytes are displayed.  CTRL-l Q returns back to SYSTEM DEBUG AID menu.

### DUMP I/O

The DUMP I/O option DUMPs the contents of an I/O location starting from the location specified by the user.

- The test prompts the user to enter the memory address which needs to be dumped.

- Displays the message:

"DUMP I/O"

"ENTER THE I/O ADDRESS :0000"

The user enters the I/O address.

- The test then displays the contents of the I/O location to a maximum of 256 bytes.

The first column represents the starting address of the corresponding row.  The next 16 bytes indicate the contents of the I/O location.  The last 16 characters in each row gives the ASCII values of the I/O contents.

- Displays the message:

"USE SPACE BAR FOR THE NEXT PAGE “B” FOR THE PREVIOUS PAGE , AND ^Q TO QUIT "

If the user presses the space bar, the test will display the next 256 bytes.  If it is `B', then the previous 256 bytes are displayed.  Ctrl Q returns back to SYSTEM DEBUG AID menu.

### Examine Memory

The EXAMINE MEMORY option helps the user to examine a particular location in memory, and change its contents. The contents of the location are 16 bit long.

- Display the message to obtain the memory address:

"EXAMINE MEMORY"

"ENTER HIGHER WORD OF ADDRESS    : 0000"

"ENTER LOWER WORD OF ADDRESS     : 0000"

- The test now displays the memory location and the contents

aaaa aaaa dddd (aaaa aaaa - memory location)  (dddd- data)

"USE + TO INCREMENT. - TO DECREMENT AND ^Q TO QUIT"

- If the user wants to change the contents, then the system will accept the new value and write it into the corresponding location.  The system then reads the location again and displays the read value:

aaaa aaaa nnnn*rrrr (nnnn - new value desired by the user) (rrrr - value read by test after writing nnnn)

- Pressing '+' will increment the address by 2 and show its contents, and '-' will decrement the address by 2 and show the contents.  Ctrl Q returns back to SYSTEM DEBUG AID menu.

### Examine I/O

The EXAMINE I/O option helps the user to examine a particular I/O location and change it's contents. The contents of the I/O location shall be 8 bit long.

- It displays the message to obtain the I/O address

"EXAMINE I/O"

"ENTER I/O ADDRESS     :  0000"

- The test now displays the I/O location and the contents

aaaa dd (aaaa -I/O location)      (dd -data)

"USE + TO INCREMENT, - TO DECREMENT AND ^Q TO QUIT"

If the user wants to change the contents, then the system will accept the new value and    write into corresponding location.  The system then reads the location again and displays the read value.

aaaa nn *rr (nn - new value desired by the user)

(rr - value read by test after writing "nn")

Pressing '+' will increment the address by 1 and show its contents and "-'" will increment the address by 1 and show the contents. Ctrl Q will return back to SYSTEM DEBUG AID menu .

### DPRAM Test

The following steps depicts be the sequence followed by the DPRAM test ,

- Display the slot number where the ASIO card is present.         "DPRAM TEST"

"TESTING ASIO CARD NUMBER : X"

Display the following message

"TEST STARTED"

- It writes the following patterns, in the memory.  Once all the locations are written verify the contents.  For every pattern, display "W" after the entire memory is written and display "V" after it is successfully verified.

The patterns to be written are:

0x0000000

0xFFFFFFFF

0xAAAAAAAA

0x55555555

0x84218421 "Walking 1’S

0x18421842

0x21842184

0x42184218

0x7BDE7BDE "Walking: 0's

0xE7BOE7E3D

0xDE7BDE7B

0xBDE7BDE7

Address as Data

- If the read data is not same as the written data, display the memory address, data read, data written as follows:

"MEMORY ADDRESS   :    XXXXXXXX"

"DATA WRITTEN          :    XXXXXXXX"

"DATA READ                 :    XXXXXXXX"

Then display,

"TEST FAILED " at LOWER RIGHT corner.

- If the DPRAM TEST Passes Display

"TEST PASSED " at LOWER LEFT corner.

It display the Message

"PRESS ANY KEY TO CONTINUE..,

After a key is pressed, returns to ASIO ADAPTER TESTS menu.

### Mail Box Test

The following sequence is followed by the MAIL BOX test.

- Display the slot number where the ASIO card is present.

"MAIL BOX TEST"

"TESTING ASIO CARD NUMBER : X"

Display the message

"TEST STARTED"

- Writes the following patterns, in the memory.  Once all the locations are written verify the contents.  For every pattern, display "W" after the entire memory is written and display "V" after it is successfully verified.

- The patterns to be written are

0x0000

0xFFFF

0xAAAA

0x5555

0x8421 "Walking 1’S

0x1842

0x2184

0x4218

0x7BDE "Walking 0's

0xE7BD

0xDE7B

0xBDE7

Address as Data

- If the data read is not same as the written data, display the memory address, data read, data written as follows:

"MEMORY ADDRESS XXXXXXXX"

"DATA WRITTEN XXXX"

"DATA READ XXXX"

Then displays,

"TEST FAILED " at LOWER Right corner.

- If the MAIL BOX TEST passes it displays:

"TEST PASSED " at LOWER Left corner

- It then displays the message:

"PRESS ANY KEY TO CONTINUE...

- After a key is pressed, it returns to the  SYSTEM DEBUG Main menu.

### HOST-ASIO Communications Test

- Displays the slot number in which the ASIO card is present.

"HOST-ASIO COMMUNICATION TEST"

"TESTING ASIO CARD NUMBER : X"

Displays the message:

"TEST STARTED"

- Initializes the MBOX and DPRAM with zeroes.  Writes the Host-to-386 Reset signature in the DPRAM.  Sends a Reset signal to the ASIO Board 386 Processor.

Displays the message:

"RESETTING THE ASIO PROCESSOR"

Time-out to wait for the ASIO Board to complete its POST and generate an Interrupt to the Main Board Processor.

- If (Interrupt is received)

then

Display the message

" 386 INTERRUPT RECEIVED"

Check for valid signature

If(Valid Signature)

then

Display the message " VALID SIGNATURE "

else

Display the message " INVALID SIGNATURE "

Display the signature expected and the signature expected and the signature found

"MEMORY ADDRESS  :  XXXXXXXX"

"DATA WRITTEN         :  XXXXXXXX"

"DATA READ                :  XXXXXXXX"

Display the message :

"TEST FAILED " at LOWER RIGHT corner.

"PRESS ANY KEY TO CONTINUE..."

After a key pressed, returns to ASIO ADAPTER TESTS menu.

else

Display the message

" 386 INTERRUPT NOT RECEIVED"

Check the signature

If(Valid Signature)

then

Display the message " VALID SIGNATURE "

else

Display the message "INVALID SIGNATURE "

Display the signature expected and the signature found

"MEMORY ADDRESS   : XXXXXXXX"

"DATA WRITTEN      : XXXXXXXX"

"DATA READ             : XXXXXXXX"

Display the message

"TEST FAILED " at LOWER RIGHT corner.

"PRESS ANY KEY TO CONTINUE...

After a key is pressed, return to ASIO ADAPTER TESTS menu.

Write the signature for the host-to 386 interrupt in the DPRAM.  Asserts an interrupt to the 386 processor.  Displays the message:

" INTERRUPT TO 386 PROCESSOR RAISED "

Time-out to wait for the Acknowledge from the 386 Processor.

- If (Acknowledge is Received)

then

Display the message

"386 ACKNOWLEDGE RECEIVED"

Check for valid signature

If(Valid Signature)

then

Display the message " VALID SIGNATURE "

else

Display the message:" INVALID SIGNATURE "

Display the signature expected and the signature found

"MEMORY ADDRESS : XXXXXXXX"

"DATA WRITTEN        : XXXXXXXX"

"DATA READ               : XXXXXXXX"

Display the message

"TEST FAILED " at LOWER RIGHT corner.

"PRESS ANY KEY TO CONTINUE	"

After a key press, returns to ASIO ADAPTER TESTS menu.

else

Display the message

" 386 ACKNOWLEDGE NOT RECEIVED"

Check the signature

If (Valid Signature)

then

Display the message " VALID SIGNATURE"

else

Display the message "INVALID SIGNATURE "

Display the signature expected and the signature found

"MEMORY ADDRESS   :  XXXXXXXX"

"DATA WRITTEN          :  XXXXXXXX"

"DATA READ                 :  XXXXXXXX"

Display the message

"TEST FAILED " at LOWER RIGHT corner.

"PRESS ANY KEY TO CONTINUE"

After a key is pressed, return to ASIO ADAPTER TESTS menu.

- Display the message

"TEST PASSED " at LOWER RIGHT corner.

"PRESS ANY KEY TO CONTINUE"

After a key is pressed, it returns to the ASIO ADAPTER TESTS menu.

### Reset ASIO

- Display the slot number where the ASIO card is present

"RESET ASIO"

"RESETTING ASIO CARD NUMBER  :  X"

- Resets the 386 Processor on the ASIO Board.  Display the Message:

"ASIO CARD HAS BEEN RESET"

- Display the message:

"PRESS ANY KEY TO CONTINUE..."

- After a key is pressed, return to the ASIO ADAPTER TESTS menu.

### Continues TEST

- Display the Message

"CONTINUOUS TEST"

"Enable Continuous Test Mode (y/n) ?"

- If “Y” is pressed then Continuous Test mode is enabled.

- Display the message

"Continuous Test Mode Enabled"

- If "N" is pressed then Continuous Test mode is disabled.

- Display the message

"Continuous Test Mode Disabled"

- Display the message

"PRESS ANY KEY TO CONTINUE..."

After a key is pressed, return to the Main Menu (ASIO ADAPTER TESTS MENU or MAIN BOARD LOGIC TEST MENU).

- Whenever any of the tests run in Continuous Mode, the display is as follows:

"CONTINUOUS TEST STARTED"

"Press ESC to end Continuous Test"

"PASS COUNT  : XXXXXXXX"                   "FAIL COUNT  : XXXXXXXX"

- The Pass Count indicates the number of time the test has Passed.  The Fail Count indicates the number of times the test has Failed.

### 79C970Internal Loop Back Test

The Internal Loop back Test follows the sequence below.

- Display the Message

" 79C970 INTERNAL LOOP BACK TEST "

- Before the loop back test, the controller is initialized for the proper loop back operation.  If the initialization is not completed by the specified duration of time, it displays the message:

"INITIALIZATION FAIL"

and returns back to MAIN MENU.

- During the loop back process, the content of the interrupt flag address is read.  If the loop back is not completed by the specified duration of time it displays the message:

"INTERRUPT TIME OUT"

and returns back to MAIN MENU

- After the successful loop back operation, any error occurred during the process is checked.

1. Transmitter Time Out Error.

2. Signal Quality Error.

3. Missed Frame Error.

4. Bus Timed Out Error.

5. Memory Error.

6. Receive Interrupt Error.

7. Transmit Interrupt Error.

8. Under Flow Error.

9. Late Collision Error.

10. Carrier Loss Error.

11. Retry Error.

12. Framing Error.

13. Overflow Error.

14. CRC error.

15. Buffer error.

Any failure during these tests results in the display of the appropriate error message and then control is transferred back to MAIN MENU.

- The successful completion of error checking results in display of the message

79C970 INTERNAL LOOP BACK TEST

TEST SUCCESSFUL

PRESS ANY KEY TO CONTINUE

After a key is pressed, control is transferred back to MAIN MENU

### 79C970 MENDEC Loop Back Test

Following is the sequence performed by the MENDEC Loop Back Test.

- Display the Message

"79C970 MENDEC LOOP BACK TEST "

- Before the loop back test, the controller is initialized for the proper loop back operation. If the initialization is not completed by the specified duration of time, display the message:

"INITIALIZATION FAIL"

and return back to MAIN MENU.

- During the loop back process, the content of the interrupt flag address is read.  If the loop back is not completed by the specified duration of time, display the message

"INTERRUPT TIME OUT"

and return back to MAIN MENU

After the successful loop back operation, any error occurred during the process is checked.

1. Transmitter Time Out Error.

2. Signal Quality Error

3. Missed Frame Error

4. Bus Timed Out Error

5. Memory Error

6. Receive Interrupt Error

7. Transmit Interrupt Error

8. Under Flow Error

9. Late Collision Error

10. Carrier Loss Error

11. Retry Error

12. Framing Error.

13. Overflow Error

14. CRC error

15. Buffer error.

- The successful completion of error checking results in the display of

79C970 MENDEC LOOP BACK TEST

TEST SUCCESSFUL

PRESS ANY KEY TO CONTINUE ....

After a key is pressed, returns back to MAIN MENU

### External Loop Back Test

Following is the sequence performed by the External Loop Back Test. · Display the Message:

" 79C970 EXTERNAL LOOP BACK TEST "

- Before the loop back test, the controller is initialized for the proper loop back operation.  If the initialization is not completed by the specified duration of time, display the message

"INITIALIZATION FAIL"

and return back to the MAIN MENU.

The choice of external connector is requested and the following message is displayed.

EXTERNAL LOOP BACK TEST

SELECT PORT 10 BASE T/AUI (0/1).

- During loop back process, the contents of the interrupt flag address is read. If the loop back    is not completed by the specified duration of time, display the message

"INTERRUPT TIME OUT'"

and return back to MAIN MENU

- After the successful loop back operation, any error occurred during the process is checked.

Transmitter Time Out Error Signal Quality Error

Missed Frame Error

- Bus Timed Out Error Memory Error Receive Interrupt Error Transmit Interrupt Error Under Flow Error Late Collision Error Carrier Loss Error Retry Error

- Framing Error. Overflow Error

- CRC error

Buffer error.

- The successful completion of error checking results in display of

- 79C970 EXTERNAL LOOP BACK TEST

Test SUCCESSFUL

PRESS ANY KEY TO CONTINUE ....

After a key is pressed, it returns back to MAIN MENU.

### Single/Continuous Mode

The loop back tests for the network controller can either be conducted only once or in a continuous loop.

- By selecting either SINGLE or CONTINUOUS mode, a flag is set in a location depending upon the choice. This flag is monitored every  time whenever any one of the NETWORK Tests are Executed

### Display Station Address

This choice checks the Ethernet address in EEPROM for accessibility through network controller and validity by checksum.

If the checksum is correct, the following message is displayed.

"ETHERNET STATION ADDRESS OF YOUR CARD :XX XX XX XX XX XX"

BOARD ID :II DD

BOARD REVISION :RR.VV

PRESS ANY KEY TO CONTINUE ......

where "xx xx xx xx xx xx xx" is a Six Byte Ethernet Address,

"II DD" is the Board Id and

"RR.VV" is the Board Revision Version Number.

After a key is pressed, controls returns back to NETWORK TESTS MENU.